3GPP TSG RAN WG2 Meeting #123	R2-2308196

Toulouse, France, August 21 –25, 2023

Agenda item:	7.7.3

Source:	Quectel

Title:	Discussion on multiple-RTT based positioning in NTN

Document for:	Discussion and Decision

- Introduction

In the RAN2#121 meeting, the following agreements on UE location verification have been achieved [1].

- RAN2 will work on a solution that ensures that location verification can be completed within a period of approximately 1 minute maximum and 30 seconds preferably. - For network verified UE location, the verification procedure can only be triggered by the CN.

In the RAN2#121bis-e meeting, the scope of this bullet is clarified [2]:

- NTN UE doesn’t support positioning measurement and report in RRC INACTIVE.

In the RAN2#122 meeting, the cell changing in the network verified UE location procedure is discussed, but no further agreement is attained [3].

In the contribution, some details about the multiple-RTT based positioning procedure for RRC connect UE are discussed.

- Discussion

Based on the agreement above, the verification procedure can only be triggered by CN, the next issue is what type of message shall be applied to trigger this procedure. There are two alternatives as illuminated in figure 1:

- Alternative 1: NAS message

For this alternative, this verification procedure is triggered by CN via NAS message, so the reasonable action is providing multiple RTT values via NAS message from UE point of view. As a result, it is due to AMF to deduce the UE location based on the multiple RTT values. If CN do not acquire the related emphasize information, it is difficult for AMF to deduce the UE location.

- Alternative 2: NG and Uu message

Figure 1: two alternatives for triggering

For alternative2, AMF triggers the procedure via NG message. Upon receiving this request, gNB triggers the location request via Uu message, receives the multiple RTT values from UE, deduces the UE location and report this location to AMF. Since gNB always owns the valid emphasize information, it is easy to deduce the UE location.

Based on the comparison above, alternative 2 is preferred.

Proposal 1: CN triggers UE location verification procedure via NG message.

If the UE location verification procedure is triggered via NG message, it is due to RRC to ensure the location verification within 1 minutes maximum or 30 seconds preferably. There are 2 choices which are descripted in figure 2.

Figure 2: 3 types of Uu procedure

The first choice is via multiple Uu procedures. As the type A in figure 2, gNB requests the RTT, and receives ONE RTT result from UE. So, it is due to gNB implementation to guarantee the whole UE location verification procedure within 30 seconds or 1 minute, without any extra modification to the procedure.

The second choice is via one Uu procedure, as the type B and type C in figure 2. For type B, gNB triggers the procedure via one RRC request, and receives multiple responses of RTT values via multiple RRC messages. Considering the limitation of 30 seconds or 1 minute, some duration information about this RTT report procedure shall be indicated to UE. For example, gNB begins the procedure in T1, and expects to finish the procedure in T2, so it indicates the T2 information to UE, and requests UE to report the RTT value before T2. For type C, gNB triggers the procedure, and receives multiple RTT values via ONE RRC message before T2. Obviously, type C is simpler than type B because of less uplink RRC message. Similar to type B, the T2 information shall be also indicated to UE.

Proposal 2: gNB requests multiple RTT values, and receives results before the indicated deadline.

Proposal 2a: UE reports RTT values via one message.

If proposal 2 and proposal 2a are applied, some further restrictions about the RTT response(s) may be needed to guarantee the accuracy of UE location. For example, the number of RTT values, the interval of 2 RTT values, the details of these further information shall be determined by RAN4.

Proposal 3: The further limitation about RTT results shall be determined by RAN4.

If multiple RTT values are reported via ONE RRC message, and these RTT values are measured by UE is in different cell, the emphasize information related to every RTT results shall be indicated to gNB, otherwise, the UE location could not be deduced by the network.

From gNB point of view, there are two alternatives to acquire the emphasize information: Xn or Uu.

Figure 3: cell changing case in RTT report

In the example of figure 3, UE acquires 5 RTT values in two cells. If emphasize information related to these RTTs are indicated via Uu interface, gNB could deduce the UE location directly, but this alternative shall introduce heavy burden for Uu interface, especially for the NTN case. The other alternative is via Xn interface. UE indicates the RTT value is measured before handover or not, so gNB could acquire related emphasize information via Xn interface. This alternative is friendly for wireless resource.

Proposal 4: If multiple RTT values are related to different cells, the maping cell shall be indicated to gNB.

- Conclusion

Based on the discussion above, the following proposals are suggested:

Proposal 1: CN triggers UE location verification procedure via NG message.

Proposal 2: gNB requests multiple RTT values, and receives results before the indicated deadline.

Proposal 2a: UE reports RTT values via one message.

Proposal 3: The further limitation about RTT results shall be determined by RAN4.

Proposal 4: If multiple RTT values are related to different cells, the maping cell shall be indicated to gNB.

- References

- Report of 3GPP TSG RAN WG2 meeting #121

- Report of 3GPP TSG RAN WG2 meeting #121bis

- Report of 3GPP TSG RAN WG2 meeting #122