3GPP TSG-RAN WG2 Meeting #123	R2-2308208

Toulouse, France, August 21-25, 2023

Agenda item:	7.25.1

Source:	Huawei, HiSilicon

Title:	RAN2 signalling design for FR2 unknown SCell activation enhancement

Document for:	Discussion

- Introduction

RAN4 agreed to support fast FR2 SCell activation by enabling UE to report available beam measurements of the to-be-activated SCells. The related RAN4 agreements were sent to RAN2 in LS R4-2306321 and R4-2310083 with the request for RAN2 signaling design. RAN2 replied the LS in R2-2306863, and confirmed the RAN4 solution is feasible by reusing the current measurement configuration and reporting framework, and the detailed signaling will be further discussed in later meetings.

In this contribution, to enable the RAN4 agreed solution, we discuss the potential options of RAN2 signaling design.

- Discussion

Regarding the serving cell measurement reporting, the key questions are how to trigger the measurement reporting, and where is the report configuration coming from.

According to the current specification, a UE derives serving cell measurements and performs measurement triggering/reporting as follows:

1. as described in 5.5.3.1 for performing measurement, the UE derives L3 filtered RSRP, RSRQ and SINR (if there is reportConfig containing SINR as trigger quantity and/or reporting quantity) of beam measurement for each serving cell based on SSB/CSI-RS if the reportConfig associated with at least one measId contains an rsType set to ssb/csi-rs and the corresponding RS is also configured in the serving cell.

2. as described in 5.5.4 for measurement report triggering, the UE considers whether the trigger conditions are satisfied (e.g. for reportType set to eventTriggered, the event is fulfilled for one applicable cell) and measurement reporting procedure should be initiated.

3. as described in 5.5.5 for measurement reporting, the UE includes beam measurement information according to the reportConfig for each serving cell, if the reportConfig associated with the measId that triggered the measurement reporting includes reportQuantityRS-Indexes and maxNrofRS-IndexesToReport, and if includeBeamMeasurements is set to true. This means the reportConfig associated to the triggered measId determines which beams/how many beams/whether beam measurement results to be reported. The details about beam measurement reporting can be found in 5.5.5.2.

From above, we can see the derivation of serving cell’s beam measurement information specified in 5.5.3.1 can be reused without any spec change. But how to trigger the measurement reporting to ensure the UE can include the serving cell measurements upon reception of the SCell activation MAC CE following the current specification with less spec change needs to be discussed. There are two options:

|  | Option description | Spec impact on 5.5.4 | Spec impact on 5.5.5 | Spec impact on asn.1 |
| --- | --- | --- | --- | --- |
| Option1 | introduce a new event which is associated to the SCell activation | Spec change is needed as below: In 5.5.4.x, add a new event: - Event xx: upon reception of the SCell activation MAC CE, consider the new event is fulfilled. | No spec change is needed. The other fields in reportConfig, like rsType, reportQuantityRS-Indexes, maxNrofRS-IndexesToReport, includeBeamMeasurements can be reused to derive the serving cell beam measurement results as described in 5.5.5. | Add asn.1 for this new event. |
| Option2 | introduce a new report type | Spec change is needed as below: In 5.5.4.1, add a new condition branch as below: 2> if the corresponding reportConfig is set to scellActivation and if a SCell activation MAC CE is received, initiate the measurement reporting procedure, as specified in 5.5. | No spec change is needed, i.e. UE include measurement results according to reportConfig in the existing way. | Add new reportType which needs to include beam measurement reporting configurations. |
|  |  |  | Spec change is needed to describe the default value of rsType, reportQuantityRS-Indexes, maxNrofRS-IndexesToReport, includeBeamMeasurements, which saves dedicated configuration via RRC. | Add new reportType e.g. scellActivation. , no need to include beam measurement reporting configurations |

The above two options are both feasible, and have spec impact on procedural text (in 5.5.4, 5.5.5) and asn.1.

Observation: By reusing the most part of existing L3 measurement framework to enable SCell (as a serving cell) beam measurement reporting upon SCell activation as requested by RAN4, a new measurement event or new reportType can be introduced in RRC spec, which are both feasible and need changes to procedural text as well as asn.1.

In our understanding, option 1 seems to have less spec impact, thus we slightly prefer option 1.

Proposal: RAN2 to introduce a new measurement event which is associated to the SCell activation.

- Conclusion

Based on the above discussion, we have the following observation and proposal:

Observation: By reusing the most part of existing L3 measurement framework to enable SCell (as a serving cell) beam measurement reporting upon SCell activation as requested by RAN4, a new measurement event or new reportType can be introduced in RRC spec, which are both feasible and need changes to procedural text as well as asn.1.

Proposal: RAN2 to introduce a new measurement event which is associated to the SCell activation.