3GPP TSG-RAN WG2 Meeting #123	R2-2308211

Toulouse, France, August 21-25, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.351 | CR | 0025 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on the BEARER ID in SRAP data PDU |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_relay-Core |  |  |  |  |  | Date: |  |  | 2023-08-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In clause 6.3.3, the description of the BEARER ID field is to carry Uu radio bearer identity. It can be understood that the Uu radio bearer idenfity equals to DRB ID. For instance, in RRC spec, the field sl-RemoteUE-RB-Identity is to identify Uu bearer, and for DRB the value is DRB-Identity. SL-RemoteUE-RB-Identity-r17 ::= CHOICE { srb-Identity-r17 INTEGER (0..3), drb-Identity-r17 DRB-Identity, ... } sl-RemoteUE-RB-Identity Identity of the end-to-end Uu bearer identity of the L2 U2N Remote UE. The value 3 for the field srb-identity-r17 (i.e., for configuring SRB3) is not supported in this version of the specification. However, in SRAP, for DRB the the BEARER ID field should be DRB identity minus 1, as described in 5.3.1.1. Thus, to make the spec consistent and avoid potential misunderstanding, the clarication can be added to clause 6.3.3. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In clause 6.3.3, - calrify that the value of BEARER ID in SRAP data PDU is equal to SRB identity or DRB identity minus 1. Impact analysis Impacted 5G architecture options: NR SA Impacted functionality L2 U2N relay, SRAP PDU Inter-operability: The change is a clarification and does not change any UE behaviour, thus there is no inter-operability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Without the change, it will not be clear that the value of BEARER ID in SRAP data PDU is equal to DRB identity or DRB identity minus 1. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

6.3.3	BEARER ID

Length: 5 bits.

This field carries info to idenfity Uu radio bearer for U2N Remote UE. For SRBs, the value is se to SRB Identity (which is configured by RRC paramether srb-Identity). For DRBs, this field is set to DRB Identity (which configured by RRC paramether srb-Identity) minus 1.

END OF CHANGES