3GPP TSG-RAN WG2 #123	R2-2308242

France Toulouse,  21st Aug – 25th Aug 2023

Source:		NEC

Agenda item:		7.18.3	User plane aspects

Title:		User plane aspects of MT-SDT

Document for:		Discussion and decision

# 1. Introduction

At RAN2 #122 meeting, regarding to user plan aspects of MT-SDT, the following agreements were made [1]:

Agreements:

- RRC explicitly indicates to MAC whether resume is trigged due to MT-SDT

- LCH restrictions are checked for DRBs as in MO-SDT (if UL data is available during SDT procedure).  Ensure CCCH can be transmitted in CG-SDT when MT-SDT is triggered in stage 3 discussion.

- Assumption is that if the UE has UL data the UE can still check and trigger MO-SDT (it is up to UE implementation)

- RA-SDT resources are not used for MT-SDT initiation RACH

- In case CG-SDT resources cannot be used or are not available for MT-SDT, UE uses non-SDT RACH for RA-based MT-SDT.  FFS whether new triggers are defined

- There is no need to define new Rel-18 CG configurations specific to MT-SDT.

- When the UE is configured with both MO and MT SDT the radio bearer configuration is common for both.

- For both MO and MT-SDT, if the next CG-SDT resource is too far, then RACH resource can be selected first.   This is checked at the point of initial resource selection (e.g. CG SDT selection).   FFS what is too far and how this is configured.   Assumption is that we will continue this discussion in SDT session.  CONFIRM with main session [CB]

In this paper, we provide our further consideration on the remaining open issue of MT-SDT.

# 2. Discussion

At last meeting, to avoid too long latency caused by using CG resource, it was agreed that for both MO and MT-SDT, if the next CG-SDT resource is too far, then RACH resource can be selected first. However, the detail of the mechanism is not crystal clear. Below, we provide some of our consideration on this.

To determine whether the next CG occasion is too far, a threshold value can be configured. If the offset between the initiation of SDT and next CG occasion is smaller than the threshold, CG-SDT resource can be selected.

In case that the time interval to the next CG occasion is larger than the threshold, RA-SDT resource can be selected. However, one possible case is that the offset to next RA occasion to be used is larger to the offset to the next CG occasion, then the UE behaviour should be discussed. There can be three possible ways, i.e., perform CG-SDT, perform RA-SDT or initiate legacy RRC resume procedure. Usually, to fulfil the paging response delay requirement, the periodicity of non-SDT RA occasion is usually small enough, and should not be larger than the threshold, so the scenario should not happen on the basis of a proper network configuration. For MO-SDT, although SDT-RA resource is used, but still the periodicity should be smaller than the offset threshold. And if the offset to the CG occasion is smaller than the offset to SDT-RA resource, then there is no need to configure the threshold at all.

So based on above, in case that the next CG resource is too far, the UE just performs RA-SDT.

Proposal 1: For MO-SDT and MT-SDT, if the next CG resource is too far and RA-SDT resource is available, the UE performs RA-SDT.

In case of MO-SDT, it is possible that the RA-SDT resource is not configured in the serving cell. There are two possible ways, i.e., initiate legacy RRC resume procedure or perform CG based MO-SDT.  To avoid issue caused by too long latency due to the use of CG resources, e.g., QoS requirement unfulfillment, the UE can just initiate legacy RRC resume procedure.

Proposal 2: For MO-SDT, if the next CG resource is too far, and RA-SDT resource is not configured, the UE initiates legacy RRC resume procedure.

The threshold is used to determine whether CG resource can be used or not, so it is only needed if CG-SDT resource is configured to the UE. Since CG-SDT is configured by RRCRelease message, the threshold value should also be configured together along with CG-SDT resource in RRCRelease message.

Proposal 3: The offset threshold to determine whether CG-SDT resource can be selected should be configured in RRCRelease message.

# 3. Conclusion

In this paper, we provide our further consideration on selection of CG-SDT resource of both MO-SDT and MT-SDT, and have the following proposals:

Proposal 1: For MO-SDT and MT-SDT, if the next CG resource is too far and RA resource is available, the UE performs RA-SDT.

Proposal 2: For MO-SDT, if the next CG resource is too far, and RA-SDT resource is not configured, the UE initiates legacy RRC resume procedure.

Proposal 3: The offset threshold to determine whether CG-SDT resource can be selected should be configured in RRCRelease message.

# Reference

[1] Meeting report of RAN2 #122.