3GPP TSG-RAN WG2 Meeting #123	            R2-2308246

Toulouse, France, August 21-25, 2023

Agenda Item	:	7.5.4.3

Title	:	On Configured Grant enhancements for XR

Source	:	Google Inc.

Document for	:	Discussion and Decision

# Introduction

Rel-18 Work Item for XR enhancements in NR was approved in RAN#98-e [1]. Multiple enhancements related to the system capacity are being specified in RAN1 and RAN2 including [1]:

-	 Multiple CG PUSCH transmission occasions in a period of a single CG PUSCH configuration

-	 Dynamic indication of unused CG PUSCH occasion(s) based on UCI by the UE

In this Tdoc, we propose some techniques to specify these enhancements and we share our views on the retransmission-less CG.

# Multiple CG PUSCH transmission occasions

The support of multiple PUSCH occasions in a CG period is beneficial to reduce latency for XR and improve performance.

Rel-16 NR-U framework was considered in RAN1 as the baseline for this XR enhancement in Rel-18. The following agreement has been made in RAN1#113:

Agreement

For time domain resource allocation for multi-PUSCH CGs, support

- For TDRA determination (based on NR-U framework)

  - For Type-1, follow the rules for DCI format 0_0 on UE specific search space, as defined in Clause 6.1.2.1.1 of TS 38.214.

    - Note: To determine the configuration of TDRA, PUSCH repetition type A is assumed according to description in 6.1.2.3 in 38.214 for Type-1.

      - It is still an open issue whether repetition is supported. If it is decided repetition is not supported, it implies the corresponding repetition factor for is one.

  - For Type-2, the TDRA table is determined by the TDRA table associated with activation DCI, as defined in Clause 6.1.2.1 of TS 38.214.

    - Note: The DCI format for activation DCI with pusch-RepTypeA is applicable.

      - It is still an open issue whether repetition is supported. If it is decided repetition is not supported, it implies the corresponding repetition factor for is one.

- N is configured by higher layers

- A single SLIV is determined from TDRA.

  - The SLIV used for 1st PUSCH per CG period.

- PUSCH is used in each of N consecutive slots per CG period

- Note: N is configured independently from cg-nrofSlots-r16 and cg-nrofPUSCH-InSlot-r16, respectively. N configuration is independent from cgRetransmissionTimer configuration.

- To determine corresponding slots for CG PUSCHs in a period of a multi-PUSCH CG configuration:

  - For the first PUSCH in the period, follow the legacy procedures.

  - For remaining PUSCHs in the period

    - ForType-1 and Type-2, reuse the corresponding procedures for NR-U by applying the RRC parameters N and M, instead of cg-nrofSlots-r16 and cg-nrofPUSCH-InSlot-r16, respectively.

In the legacy Configured Grant, the UE does not receive dynamic scheduling for PUSCH transmission. Therefore, the UE is required to derive the HARQ Process Number associated with each PUSCH transmission using the following formula specified in 38.321:

HPI = [floor(CURRENT_symbol / periodicity)] modulo nrofHARQ-Processes

Where:

CURRENT_symbol = (SFN x SlotsPerFrame x SymbolsPerSlot) + (SlotNumber x SymbolsPerSlot) + SymbolNumber

Where SymbolsPerSlot is 14 for normal CP and 12 for ECP. The SFN is the system Frame Number. SlotNumber is the slot number within which the PUSCH starts. The SymbolNumber is the symbol number at which the PUSCH starts.

The gNB configures the UE with the number of HARQ processes and the periodicity. This is done using the ConfiguredGrantConfig information. The gNB will also dynamically schedule the UE with the HARQ retransmissions for configured grants. In that case, the gNB signals the HARQ process number associated with the requested retransmission in the scheduling DCI.

The existing formula has a limitation and cannot be applied as is for the case of multiple PUSCH occasions in the CG period as all PUSCH occasions in the same CG period will get the same HARQ Process Number.

RAN1 made the following agreement in RAN1#113 to modify the formula to derive the HPI:

Agreement

From RAN1 perspective, for determination of HARQ process IDs associated to PUSCHs in multi-PUSCHs CG assuming one TB per PUSCH:

- The HARQ process ID for the first configured/valid PUSCH in a period is determined based on the legacy CG procedure when cg-RetransmissionTimer is not configured, and applying the following formula, whichever is applicable

  - HARQ Process ID = [X*floor( (CURRENT_symbol ) / periodicity)] modulo nrofHARQ-Processes

  - HARQ Process ID = [X*floor((CURRENT_symbol ) / periodicity)] modulo nrofHARQ-Processes + harq-ProcID-Offset2

    - X= the number of configured PUSCHs in the CG period

- The HARQ process ID of the remaining configured/ and valid CG PUSCHs in the period is determined by incrementing the HARQ process ID of the preceding PUSCH in the period by one with module operation with nrofHARQ-Processes or module operation with (nrofHARQ-Processes + harq-ProcID-Offset2), whichever applicable.

- Note: A configured CG PUSCH is invalid if the CG PUSCH is dropped due to collision with DL symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated or SSB.

Send an LS to RAN2 to convey the above RAN1 agreement. Final LS is in R1-2306233.

We propose below to adjust the RAN1 formula to take the index of the PUSCH occasion in the CG period into account in the formula rather than leaving it outside in the specification text (Current RAN1 formulation is : The HARQ process ID of the remaining configured/ and valid CG PUSCHs in the period is determined by incrementing the HARQ process ID of the preceding PUSCH in the period by one with module operation with nrofHARQ-Processes or module operation with (nrofHARQ-Processes + harq-ProcID-Offset2), whichever applicable).

We propose the following formulation:

HPI = [X *floor( CURRENT_symbol / periodicity)   + I ] modulo nrofHARQ-Processes

where the X defines the number of PUSCH occasions in the CG period as in the RAN1 agreement and the proposed index I is the index of the PUSCH occasion in the CG period. The index I can be in the range from 0 to X-1 and is incremented for each PUSCH occasion.

- Adjust the RAN1 HARQ Process ID determination formula to include the index of the CG PUSCH occasion in the CG period:

- HPI = [X *floor( CURRENT_symbol / periodicity)   + I ] modulo nrofHARQ-Processes

- where the X defines the number of PUSCH occasions in the CG period and I is the index of the PUSCH occasion in the CG period.

# Conclusion

In this contribution, we have presented several proposals for the configured enhancements to support the XR traffic:

- Adjust the RAN1 HARQ Process ID determination formula to include the index of the CG PUSCH occasion in the CG period:

- HPI = [X *floor( CURRENT_symbol / periodicity)   + I ] modulo nrofHARQ-Processes

- where the X defines the number of PUSCH occasions in the CG period and I is the index of the PUSCH occasion in the CG period.

# References

- RP-223502, “New WID on XR Enhancements for NR” Nokia, Qualcomm, 3GPP TSG RAN Meeting #98-e, Dec. 2022

- 3GPP TR 38.838 V17.0.0 (2021-12) Study on XR (Extended Reality) Evaluations for NR (Release 17)