3GPP TSG-RAN WG2 Meeting #123											R2-2308282

Toulouse, France, August 21-25, 2023

Agenda item:		7.3.5

Title: 	The remaining issues on connected mode mobility

Source: 			ZTE Corporation, Sanechips

Document for: 	Discussion and Decision

- Introduction

In RAN#98, a new WID [1] of Network energy savings for NR has been agreed. And the objective about CHO enhancement is the following:

- Specify CHO procedure enhancement(s) in case source/target cell is in NES mode [RAN2]

In this paper, we will further discuss the CHO enhancement for NES, and discuss the impact on RAN2 specification.

- Discussion

In RAN2#122 meeting, the agreements about mobility enhancements for NES are as below:

Agreements

- We will have a CHO solution that considers NES mode of at least source cell.

- We can have a specific NES CHO execution condition based on source cell NES mode.   FFS how the UE determines is in NES mode.   FFS on how this is achieved in RRC

- We will not introduce new L1 signalling for the purpose of CHO

- Event A3, A4, A5 can be configured as a CHO execution condition in the NES scenario.   We will study the time based mechanism

First of all, if the source cell enables cell DTX/ DRX, it means the source cell enters into NES mode. According to the above agreements, if the source cell enables cell DTX/ DRX, and the discontinuous transmission or reception would cause the UE performance deterioration, the CHO is preferred. In general, the handover is initiated before or when the source cell enables cell DTX/ DRX. Considering the enabled cell DTX/ DRX in source cell couldn’t deteriorate the RSRP of source cell, Event A3, A4, A5 would be configured with a lower threshold to trigger a handover, but this will trigger a handover immediately. Hence, in order to trigger a handover before or when the source cell enables cell DTX/ DRX, the triggering time could be supplied in CHO configuration to UE before or when the source cell enters NES mode.

As above, the triggering condition is a time condition. According to the agreement ‘The Cell DTX/DRX configuration contains at least: periodicity, start slot/offset, on duration’, gNB could determine the start time at advance, and enable or disable the cell DTX/DRX configuration based on the varied load of source cell. From our understanding, the effective time of the cell DTX/DRX configuration would at least last multiple periodicity. If the effective time is too short, the handover is unnecessary. Although the transmission of enabling or disabling signaling is dynamic, gNB would determine to enable or disable based on the long term estimation. Hence, gNB would evaluate the load of source cell, predict the effective start time of cell DTX/DRX configuration at advance.

In NTN, a time based CHO triggering event is introduced as below:

CondEvent T1 (Time measured at UE is within a duration from threshold) Inequality T1-1 (Entering condition) Mt>Thresh1 Inequality T1-2 (Leaving condition) Mt>Thresh1+Duration

CondEvent T1 could be reused. Specifically, gNB could predict the load of source cell, estimate the time (e.g.Thresh 1) that source cell is going to enter into NES mode and then transmit a CHO command carrying Thresh 1 and duration parameters to UE at advance. UE could search for a target cell whose RSRP satisfies Event A3, A4 or A5, then, trigger a handover towards it after Thresh 1, and finish the handover before Thresh 1+duration. And the value range of duration is (1..6000), it could be configured as a suitable value to distribute UE within duration in order to avoid the RACH congest in the target cell. As a result, a time based CHO triggering event (CondEvent T1) could be used in CHO for NES.

Proposal 1: CondEvent T1 in combination with CondEvent A3, A4 or A5 could be configured as execution event in CHO for NES.

For the selection of the target cell, considering UE doesn’t know the load and scheduling of the cell, it is unable to distinguish a cell that could allocate the most resource for it. But gNB could obtain the information of the neighbor cell, the information could include the load, NES mode and so on. From network perspective, if the neighbor cell enables NES mode, but it still could supply better service quality for UE, it could be the candidate cell. Hence, it is up to NW implementation to select some candidate cells and configure them to UE via CHO. If there are more than one triggered cells, it is up to UE to select one of the triggered cells as the selected cell for reconfiguration execution.

Proposal 2: It is up to NW implementation to configure the candidate cells for CHO and it is up to UE implementation to select one of the triggered cells if there are more than one, i.e. the existing conditional reconfiguration execution applies.

- Conclusion and proposals

Based on the analysis in previous sections, the following observations and proposals are given:

Proposal 1: CondEvent T1 in combination with CondEvent A3, A4 or A5 could be configured as execution event in CHO for NES.

Proposal 2: It is up to NW implementation to configure the candidate cells for CHO and it is up to UE implementation to select one of the triggered cells if there are more than one, i.e. the existing conditional reconfiguration execution applies.

- References

[1] RP-223540 New WID proposal: Network energy savings for NR