3GPP TSG-RAN WG2 #123	R2-2308374

Toulouse, France, August 21- 25, 2023

Agenda Item:	7.7.4.2 Handover enhancements

Source:	NEC

Title:	Support RACH-less CHO

Document for:	Discussion, Decision

# Introduction

RAN2 confirmed to support RACH-less handover for all scenarios including, intra and inter satellite, with and without gateway/gNB switch. Moreover, many details regarding RACH-less handover have been confirmed with some minor FFSs left. Regarding RACH-less CHO, during RAN2#121bits meeting, we had initial agreement on considering RACH-less with time-based CHO:

Consider to support combining RACH-less HO with time-based CHO for NTN, taking into account the 1) validity of pre-allocated grant and potential waste of reserved resource; 2) when/how to provide dynamic grant in PDCCH.

In this contribution we further discuss and propose to support RACH-less CHO for all conditional handover scenarios.

# Discussion

## Support RACH-less CHO

In NTN, considering the very frequent handovers experienced by a given UE (in earth moving cell scenario) and a large amount UEs handovers at the same time (in satellite switch scenario), CHO is very important and fundamental feature. With CHO, measurement report and subsequent handover command signalling exchange between UE and source gNB could be skipped, and UE execute the HO directly once the condition fulfilled. This reduces the handover interruption largely, the gain is particular high considering the longer gNB-UE RTT in NTN scenario. The execution condition and RRC configuration in target gNB are pre-configured far before the handover time point. this avoids RRC signalling storm issue e.g., at satellite switch scenarios. In addition, It will increase HO reliability since the signalling between UE and gNB are exchanged before UE going to lose the coverage of the source gNB.

On the other hand, RACH-less will further reduce the interruption and avoid RACH signalling storm by omitting RACH procedure at target cell. But without CHO, RACH-less itself neither can avoid RRC signalling storm, nor increase handover reliability. Even both features will reduces the handover interruption, but the gain come from different aspect, the maximum gain would be achieved by enabling both.

Observation1: Combining RACH-less and CHO would provide maximum gain including shortest handover interruption, no RRC and RACH signalling storm, high handover reliability, low signalling overhead.

If we only support RACH-less handover, NW will be forced to choose either avoid RACH via RACH-less handover or avoid RRC signalling around handover time via RACH-based CHO, then handover issues in NTN scenario would be solved in half way. And the gain would not be maximized.

Observation2: only supporting RACH-less handover and RACH-based CHO in NTN scenario would solve issue in half way and make it difficult for gNB to choose.

Following the observation1 and observation2, we propose:

Proposal 1: Support RACH-less CHO for NR-NTN.

Technically speaking, RACH-less conditional handover will be supported once RACH-skip is introduced in RRCReconfiguration for handover case, because the pre-prepared RRC configuration at candidate cell is defined as a container contain the whole RRCReconfiguration.

Moreover, in the case of service link change, where handover happens around T-service, which is known by both gNB and UE via configured time-based condition. It is very easy that gNB know when the handover will happen and when to dynamically schedule the UE at target cell without any enhancement. If pre-allocated CG is configured, it will also only be valid within the time window defined by CondEvent T1. If proposal 1 is not agreeable yet, at least, we can propose following:

Proposal 1a: confirm RAN2 can support RACH-less CHO with CondEvent T1 without further specification change once RACH-less HO is supported

## Issue and Enhancements of RACH-less CHO

Two issues have been mentioned regarding support RACH-less CHO

- validity of pre-allocated grant and potential waste of reserved resource;

- when/how to provide dynamic grant in PDCCH.

In fact, this two issues are both coming from uncertainty of the CHO execution timing. once we make the predicated CHO timing clear at gNB/UE side, there will be neither waste of reserved resource issue nor dynamic grant scheduling timing issue.

Firstly, we look the CHO timing uncertainty issue: there is uncertainty on whether/when UE will hand over to the pre-prepared cells, this is main difference from RACH-less handover. See following figure comparing on RACH-less HO and RACH-less CHO procedure and the main difference as highlighted:

Figure 1: handover timing in RACH-less HO

Figure 2: handover timing in RACH-less CHO

Hence, in RACH-less CHO case, it is difficult to decide when to send dynamic grants from target cell to UE, or in case per-allocated grant is configured, it is not sure when it will be used. In worst case, the pre-allocated grant might be not used and wasted.

To overcome this timing issue, we can make NW aware of the CHO execution timing. When CHO execution condition is fulfilled or is going to fulfilled, UE can report to the source cell ( current serving cell) that UE will execute CHO to the selected target cell. Source gNB then inform target gNB/cell the upcoming CHO of the UE (if it is inter-gNB CHO) to the target cell. this would be helpful for target cell to start dynamic scheduling for the UE at right time, or know when the pre-configured CG will be used in real.

Figure 3: CHO notification

Proposal 2: Introduce CHO notification from UE to source gNB to support RACH-less CHO

On the other hand, as discussed during last meeting, the handover timing would be largely predictable even for location-based CHO in earth moving cell scenario, because UE will move out of the cell due the movement of the cell coverage. So a simple enhancement is to introduce a valid time window for RACH-skip config, if the execution condition is fulfilled within the time window, RACH-skip config is valid, UE transmit on pre-allocated CG or UE will expect DG from target gNB during this time window, i.e., RACH-less CHO is performed. Otherwise if the execution condition is fulfilled out of the configured time window. RACH-skip should be invalid, and RACH-based CHO can be performed.

Proposal 3: Introduce a valid time window for RACH-skip configuration. UE executes RACH-less CHO if the time is in the configured time window, otherwise executes RACH-based CHO

# Summary

In this contribution, we discuss motivations of supporting RACH-less CHO and possible enhancements to make RACH-less CHO feasible in all NTN scenario.

Observation1: Combining RACH-less and CHO would provide maximum gain including shortest handover interruption, no RRC and RACH signalling storm, high handover reliability, low signalling overhead.

Observation2: only supporting RACH-less handover and RACH-based CHO in NTN scenario would solve issues in half way and make it difficult for gNB to choose.

Proposal 1: Support RACH-less CHO for NR-NTN.

Proposal 1a: confirm RAN2 can support RACH-less CHO with CondEvent T1 without further specification change comparing once RACH-less HO is supported

Proposal 2: Introduce CHO notification from UE to source gNB to support RACH-less CHO

Proposal 3: Introduce a valid time window for RACH-skip configuration. UE executes RACH-less CHO if the time is in the configured time window, otherwise executes RACH-based CHO