3GPP TSG-RAN WG2 Meeting #123	R2-2308386

Toulouse, France, August 21 – 25, 2023

Agenda item:	7.2.1

Source: 	Qualcomm Incorporated

Title: 	Stage 2 TP for SL-MO-LR/SL-MT-LR

Document for:		Discussion and Decision

# 1.	Introduction

At SA2#157, CRs #0321 and #0322 to TS 23.273 (tdocs S2-2307514 and S2-2307552) adding new SL-MT-LR  and SL-MO-LR procedures were agreed. In addition, the applicability of the 5GC MT-LR and 5GC MO-LR procedures using sidelink positioning with one or more anchor UEs was clarified.

This contribution proposes corresponding description for TS 38.305.

# 2.	Proposal

Proposal 1:	Endorse the TP in the Annex of this contribution for the 38.305 baseline CR.

## 7.3	Service Layer Support using combined LPP, NRPPa, and SLPP Procedures

### 7.3.1	General

As described in TS 23.502 [26] and TS 23.273 [35], UE-positioning-related services can be instigated from the 5GC for an NI-LR or MT-LR location service, or from the UE in case of an MO-LR location service. The complete sequence of operations in the 5GC is defined in TS 23.502 [26] and TS 23.273 [35]. This clause defines the overall sequences of operations that occur in the LMF, NG-RAN and UE as a result of the 5GC operations.

### 7.3.2	NI-LR and MT-LR Service Support

Figure 7.3.2-1 shows the sequence of operations for an NI-LR or MT-LR location service, starting at the point where the AMF initiates the service in the LMF.

Figure 7.3.2-1: UE Positioning Operations to support an MT-LR or NI-LR

1.	The AMF sends a location request to the LMF for a target UE and may include associated QoS, the scheduled location time and the UE LPP positioning capabilities when available, as described in TS 23.273 [35].

2.	The LMF may obtain location related information from the UE and/or from the serving NG-RAN Node. In the former case, the LMF instigates one or more LPP procedures to transfer UE positioning capabilities, provide assistance data to the UE and/or obtain location information from the UE. The UE may also instigate one or more LPP procedures after the first LPP message is received from the LMF (e.g., to request assistance data from the LMF). If a scheduled location time is provided in step 1, the LMF may schedule location measurements by the UE to occur at or near to the scheduled location time. The LPP procedures to transfer UE LPP positioning capabilities may be skipped if the LMF already obtained the UE positioning capabilities from the AMF in step 1.

2a.	If the LMF needs location related information for the UE from anchor UEs 2 to n, the LMF instigates one or more SLPP procedures with the UE to transfer sidelink positioning capabilities for the UE and anchor UEs 2 to n, to transfer SLPP assistance data to the UE, and to transfer sidelink location information for the UE obtained at step 2b. The UE may also instigate one or more SLPP procedures with the LMF after the first SLPP message has been received from the LMF (e.g., to request sidelink assistance data from the LMF).

2b.	The UE may instigate one or more SLPP procedures among UE and anchor UEs 2 to n to transfer sidelink positioning capabilities, provide sidelink assistance data, and/or obtain sidelink location information for the UE. If the UE received a SLPP request for sidelink location information at Step 2a, the UE sends a SLPP response to the LMF and includes the location results obtained at Step 2b.
Steps 2a/2b are not necessarily serialised with step 1 or step 3; steps 2a/2b could precede or overlap with step 1 or step 3.

NOTE 1: 	The UE may perform the discovery procedures defined in sub-clause 6.4 TS 23.586 [46] to discover anchor UEs 2 to n, if not already discovered.

NOTE  2:	The LMF may also trigger the discovery procedure in the UE using the SL-MT-LR procedure in sub-clause 7.3A.2.

3.	If the LMF needs location related information for the UE from the NG-RAN, the LMF instigates one or more NRPPa procedures. Step 3 is not necessarily serialised with step 2 or steps 2a/2b; if the LMF and NG-RAN Node have the information to determine what procedures need to take place for the location service, step 3 could precede or overlap with step 2 or steps 2a/2b. If a scheduled location time is provided in step 1, the LMF may schedule location measurements by the NG-RAN to occur at or near to the scheduled location time.

4.	The LMF returns a location response to the AMF with any location estimate obtained as a result of steps 2, 2a/2b and 3. The LMF may also return the LPP UE capabilities as described in TS 23.273 [35].

NOTE 3:	The LMF may send a location request to the UE at step 2 containing the scheduled location time sometime before the scheduled location time to allow the UE to enter CM Connected state shortly before the scheduled location time.

### 7.3.3	MO-LR Service Support

Figure 7.3.3-1 shows the sequence of operations for an MO-LR service, starting at the point where an LCS Client in the UE or the user has requested some location service (e.g., retrieval of the UE's location or transfer of the UE's location to a third party).

Figure 7.3.3-1: UE Positioning Operations to support an MO-LR

1.	The UE sends an MO-LR location service request message included in a UL NAS TRANSPORT message as specified in TS 24.501 [29] to the AMF. The MO-LR location service request message may carry an LPP PDU to instigate one or more LPP procedures to transfer capabilities, request assistance data, and/or transfer location information and the scheduled location time, as described in TS 23.273 [35].

2.	The AMF invokes the Nlmf Determine Location Request service operation towards the LMF as specified in TS 29.572 [33] and includes any LPP PDU, the scheduled location time received in step 1 and the UE LPP positioning capabilities when available.

3.	The LMF may obtain location related information from the UE and/or from the serving NG-RAN node. In the former case or if an immediate response is needed to any LPP procedure instigated by the UE in step 1 (e.g., a request for assistance data), the LMF instigates one or more LPP procedures to transfer UE positioning capabilities, provide assistance data to the UE and/or obtain location information from the UE. The UE may also instigate further LPP procedures after the first LPP message is received from the LMF (e.g., to request assistance data or to request further assistance data). If a scheduled location time is provided in step 2, the LMF may schedule location measurements by the UE to occur at or near to the scheduled location time. The LPP procedures to transfer UE positioning capabilities may be skipped if the LMF already obtained the UE positioning capabilities from the AMF in step 2.

3a.	If the LMF needs location related information for the UE from anchor UEs 2 to n, the LMF instigates one or more SLPP procedures with the UE to transfer sidelink positioning capabilities for the UE and anchor UEs 2 to n, to transfer SLPP assistance data to the UE, and to transfer sidelink location information for the UE obtained at step 3b. The UE may also instigate one or more SLPP procedures with the LMF after the first SLPP message has been received from the LMF (e.g., to request sidelink assistance data from the LMF).

3b.	The UE may instigate one or more SLPP procedures among UE and anchor UEs 2 to n to transfer sidelink positioning capabilities, provide sidelink assistance data, and/or obtain sidelink location information for the UE. If the UE received a SLPP request for sidelink location information at Step 3a, the UE sends a SLPP response to the LMF and includes the location results obtained at Step 3b.
Steps 3a/3b are not necessarily serialised with step 3 or step 4; steps 3a/3b could precede or overlap with step 3 or step 4.

NOTE 1: 	The UE may perform the discovery procedures defined in sub-clause 6.4 TS 23.586 [46] to discover anchor UEs 2 to n, if not already discovered.

NOTE  2:	The LMF may also trigger the discovery procedure in the UE using the SL-MT-LR procedure in sub-clause 7.3A.2.

4.	If the LMF needs location related information for the UE from the NG-RAN, the LMF instigates one or more NRPPa procedures. Step 4 may also precede step 3 or steps 3a/3b or occur in parallel with it. If a scheduled location time is provided in step 1, the LMF may schedule location measurements by the NG-RAN to occur at or near to the scheduled location time.

5.	The LMF invokes the Nlmf Determine Location Response service operation towards the AMF as specified in TS 29.572 [33] which includes any location estimate obtained as a result of steps 3, 3a/3b and 4. The LMF may also return the LPP UE capabilities as described in TS 23.273 [35].

6.	If the UE requested location transfer to a third party the AMF transfers the location received from the LMF in step 5 to the third party as defined in TS 23.273 [35].

7.	The AMF sends an MO-LR location service response message included in a DL NAS TRANSPORT message as specified in TS 24.501 [29].

### 7.3.4	Deferred MT-LR Event Reporting Support

Figure 7.3.4-1 shows the sequence of operations for a Deferred MT-LR Event Reporting starting at the point where the UE reports an event to the LMF.

Figure 7.3.4-1: UE Positioning Operations to support a Deferred MT-LR

1.	The UE sends a supplementary services event report message to the LMF as described in TS 24.571 [41] which is transferred via the serving AMF and is delivered to the LMF using an Namf_Communication_N1MessageNotify service operation. The event report may indicate the type of event being reported and may include an embedded positioning message which includes any location measurements or location estimate, protection level and achievable target integrity risk.

2.	If LMF determines no positioning procedure is needed, steps 3, 3a/3b and 4 are skipped.

3.	The LMF may utilize any location information received in step 1. The LMF may also retrieve location related information from the UE and/or from the serving NG-RAN Node. In the former case, the LMF instigates one or more LPP procedures to provide assistance data to the UE and/or obtain location information from the UE. The UE may also instigate one or more LPP procedures after the first LPP message is received from the LMF (e.g., to request assistance data from the LMF).

3a.	If the LMF needs location related information for the UE from anchor UEs 2 to n, the LMF instigates one or more SLPP procedures with the UE to transfer sidelink positioning capabilities for the UE and anchor UEs 2 to n, to transfer SLPP assistance data to the UE, and to transfer sidelink location information for the UE obtained at step 3b. The UE may also instigate one or more SLPP procedures with the LMF after the first SLPP message has been received from the LMF (e.g., to request sidelink assistance data from the LMF).

3b.	The UE may instigate one or more SLPP procedures among UE and anchor UEs 2 to n to transfer sidelink positioning capabilities, provide sidelink assistance data, and/or obtain sidelink location information for the UE. If the UE received a SLPP request for sidelink location information at Step 3a, the UE sends a SLPP response to the LMF and includes the location results obtained at Step 3b.
Steps 3a/3b are not necessarily serialised with step 3 or step 4; steps 3a/3b could precede or overlap with step 3 or step 4.

NOTE 1: 	The UE may perform the discovery procedures defined in sub-clause 6.4 TS 23.586 [46] to discover anchor UEs 2 to n, if not already discovered.

NOTE  2:	The LMF may also trigger the discovery procedure in the UE using the SL-MT-LR procedure in sub-clause 7.3A.2.

4.	If the LMF needs location related information for the UE from the NG-RAN, the LMF instigates one or more NRPPa procedures. Step 4 is not necessarily serialised with step 3 or steps 3a/3b; if the LMF and NG-RAN Node have the information to determine what procedures need to take place for the location service, step 4 could precede or overlap with step 3 or 3a/3b.

5.	The LMF invokes an Nlmf_Location_EventNotify service operation towards the GMLC with an indication of the type of event being reported and any location estimate obtained as a result of steps 3 and 4.

## 7.3A	Service Layer Support for Sidelink Positioning

### 7.3A.1	General

As described in TS 23.273 [35], UE-positioning-related services can be instigated from the 5GC for an SL-MT-LR location service to obtain ranging/sidelink positioning location results for a group of UEs, or from the UE in case of an SL-MO-LR location service to obtain sidelink positioning/ranging location results using one or more other UEs with the assistance of an LMF. When at least one participating UE is in network coverage and enabled via a subscription to access a PLMN, the UE may request LMF assistance for a sidelink ranging/positioning session or may be requested by an LMF to participate in a sidelink ranging/positioning session. The complete sequence of operations in the 5GC is defined in TS 23.273 [35]. This clause defines the overall sequences of operations that occur in the LMF and UE.

### 7.3A.2	SL-MT-LR Service Support

Figure 7.3A.2-1 shows the sequence of operations for an SL-MT-LR location service, starting at the point where the AMF initiates the service in the LMF.

Figure 7.3A.2-1: UE Positioning Operations to support an SL-MT-LR.

1.	The AMF serving UE1 sends a location request to the LMF including information on the required location results of the n UEs (e.g., relative location or ranges and directions between pairs of UE(s)), the SUPI of UE1, the required LCS QoS and the Application layer IDs of the UEs when available, as described in TS 23.273 [35].

2.	The LMF sends an SL-MT-LR request to UE1 (via serving AMF) including the types of required location results (e.g., relative locations or distances and/or directions) and the Application layer IDs of the other UEs 2 to n.

3.	UE1 attempts to discover the other UEs 2 to n using their Application layer IDs if not already discovered, as described in TS 23.273 [35].

4.	UE1 obtains the sidelink positioning capabilities of the discovered UEs using the SLPP capability transfer procedures specified in sub-clause X.Y.Z.

5.	UE1 returns a SL-MT-LR Response to the LMF (via serving AMF) indicating which of the UEs 2 to n have been discovered and the sidelink positioning capabilities of the discovered UEs, if obtained.

6a.	The LMF may request the sidelink positioning capabilities of the UEs 1 to n from UE1 if not already received at step 5. UE1 may instigate the procedure for step 6b and returns the obtained capabilities of the UEs to the LMF.

6b.	UE1 may instigate the SLPP Capability Transfer procedure specified in sub-clause X.Y.Z with UEs 2 to n to obtain the sidelink positioning capabilities for UEs 2 to n, if not already obtained (e.g., at step 4).

7a.	UE1 may send a request for specific sidelink assistance data to the LMF. The LMF determines the sidelink assistance data, possibly taking the received UE capabilities from step 4 or 6a into account and provides the assistance data to UE1.

7b.	UE1 may instigate the SLPP Assistance Data Transfer procedure specified in sub-clause X.Y.Z with UEs 2 to n to provide the received assistance data from step 7a to the UEs 2 to n. The assistance data may assist UEs 1 to n to obtain sidelink location measurements and/or may assist UE1 to calculate sidelink positioning/ranging location results.

8a.	The LMF sends a request for sidelink location information to UE1. The request may include an indication whether location calculation should be performed by the UEs (UE-based), or whether location measurements should be provided for location calculation at the LMF (UE-assisted). UE1 may instigate the procedure for step 8b and provides the sidelink location measurements or location estimates to the LMF.

8b.	UE1 instigates the SLPP Location Information Transfer procedure specified in sub-clause X.Y.Z among UEs 1 to n in which UE1 to n obtain sidelink location measurements and UEs 2 to n transfer their sidelink location measurements to UE1.

NOTE 1:	Steps 6a, 7a, 8a and 6b, 7b, 8b could be performed in any order and may also overlap.

NOTE 2:	Steps 6b, 7b, and 8b are not necessarily serialised with steps 6a, 7a, and 8a, respectively.

9.	The LMF returns a location response to the AMF with any sidelink positioning/ranging location results obtained as a result of steps 6 to 8.

### 7.3A.3	SL-MT-LR Service Support for periodic, triggered Location Events

Figure 7.3A.3-1 shows the sequence of operations for an SL-MT-LR location service for periodic and triggered location events, starting at the point where the AMF initiates the service in the LMF.

Figure 7.3A.3-1: UE Positioning Operations to support an SL-MT-LR for periodic, triggered Location Events.

1.	The AMF serving UE1 sends a location request to the LMF to request periodic or triggered sidelink positioning/ranging location results of the n UEs (e.g., relative location or ranges and directions between pairs of UE(s)). The request includes information on the time interval between successive location reports, the total number of reports, and/or the details of the trigger event (e.g., change of location or ranges), the SUPI of UE1, the required LCS QoS and the Application layer IDs of the UEs when available, as described in TS 23.273 [35].

2.	The LMF sends a Periodic-Triggered SL-MT-LR request to UE1 (via serving AMF) including the information of the periodic or triggered event, the types of required location results (e.g., relative locations or distances and/or directions) and the Application layer IDs of the other UEs 2 to n.

3.	UE1 attempts to discover the other UEs 2 to n using their Application layer IDs if not already discovered, as described in TS 23.273 [35].

4.	UE1 obtains the sidelink positioning capabilities of the discovered UEs using the SLPP capability transfer procedures specified in sub-clause X.Y.Z.

5.	UE1 returns a Periodic-Triggered SL-MT-LR Response to the LMF (via serving AMF) indicating which of the UEs 2 to n have been discovered, whether the UEs 1 to n accept the periodic or triggered location request, and the sidelink positioning capabilities of the discovered UEs, if obtained.

6a.	The LMF may instigate one or more SLPP procedures with UE1 to transfer sidelink positioning capabilities for UEs 1 to n, to transfer SLPP assistance data to UE1, and to transfer sidelink location information for UEs 1 to n (e.g., sidelink ranging and/or location measurements or location estimates obtained at step 6b), as described in steps 6 – 8 in Figure 7.3A.2-1. 
The UE1 may also instigate one or more SLPP procedures with the LMF after the first SLPP message has been received from the LMF (e.g., to request sidelink assistance data from the LMF).

6b.	UE1 may instigate one or more SLPP procedures among UEs 1 to n to transfer sidelink positioning capabilities, provide sidelink assistance data to the UEs 2 to n, and/or obtain sidelink location information for UEs 1 to n (e.g., sidelink ranging and/or location measurements or location estimates) as described in steps 6 – 8 in Figure 7.3A.2-1. If UE1 received a SLPP request for sidelink location information at Step 6a, UE1 sends a SLPP response to the LMF and includes the location results obtained at Step 6b.

7.	The LMF returns a location response to the AMF indicating that the SL-MT-LR for periodic, triggered Location Events has been successfully initiated and with any initial sidelink positioning/ranging location results obtained as a result of steps 6.

8.	The UEs 1 to n may periodically perform sidelink positioning/ranging in order to support steps 9 and 10. The UE may perform steps 6b, 7b, and 8b in Figure 7.3A.2-1.

9.	The UE monitors for occurrence of the trigger or periodic event requested at step 2.

10.	UE1 sends an event report message to the LMF (via serving AMF) indicating the type of event being reported and may include sidelink location results obtained at step 8.

11.	The LMF (via serving AMF) returns an event report acknowledgement to UE1.

12.	If sidelink location results are needed for event reporting and not received at step 10, the LMF may instigate one or more SLPP procedures with UE1 as described in step 6a/6b.

13.	The LMF returns the event report and any sidelink location results to the LCS Client or AF.

14.	The UEs 1 to n continue to periodically perform sidelink positioning/ranging as in step 8.

15.	The UE1 continues to monitor for further periodic or trigger events as in step 9 and instigates steps 10-12 each time a periodic or trigger event is detected.

### 7.3A.4	SL-MO-LR Service Support

Figure 7.3A.4-1 shows the sequence of operations for an SL-MO-LR service instigated by UE1 to obtain sidelink positioning/ranging location results using one or more other UEs with the assistance of an LMF.

Figure 7.3A.4-1: UE Positioning Operations to support an SL-MO-LR

1.	Based on a trigger of service request (e.g., received from the application layer in UE1) UE1 discovers the other UEs 2 to n, as described in TS 23.273 [35].

2.	UE1 may obtain the sidelink positioning capabilities of the discovered UEs using the SLPP capability transfer procedures specified in sub-clause X.Y.Z.

3.	UE1 sends an SL-MO-LR Request to the LMF via the serving AMF, including the Application layer IDs and/or GPSI of the other UEs 2 to n together with an indication on any sidelink assistance data needed, position calculation assistance from the LMF needed, and/or whether location results should be transferred to an LCS client or AF. For position calculation assistance from the LMF, the SL-MO-LR Request may include information on the type of sidelink positioning/ranging location results (e.g., absolute location, relative location or distances and directions between pairs of UEs) and desired QoS.

4a.	The LMF may request the sidelink positioning capabilities of the UEs 1 to n from UE1 if not already received at step 3. UE1 may instigate the procedure for step 4b and returns the obtained capabilities of the UEs to the LMF.

4b.	UE1 may instigate the SLPP Capability Transfer procedure specified in sub-clause X.Y.Z with UEs 2 to n to obtain the sidelink positioning capabilities for UEs 2 to n, if not already obtained (e.g., at step 2).

5a.	UE1 may send a request for specific sidelink assistance data to the LMF, if the request was not already included at step 3. The LMF determines the sidelink assistance data, possibly taking the received UE capabilities from step 2 or 4a into account and provides the assistance data to UE1.

5b.	UE1 may instigate the SLPP Assistance Data Transfer procedure specified in sub-clause X.Y.Z with UEs 2 to n to provide the received assistance data from step 5a to the UEs 2 to n. The assistance data may assist UEs 1 to n to obtain sidelink location measurements and/or may assist UE1 to calculate sidelink positioning/ranging location results.

6a.	If UE1 requested position calculation assistance from the LMF or location transfer to an LCS client or AF at step 3, the LMF sends a request for sidelink location information to UE1. The request may also include an indication whether location calculation should be performed by the UEs (UE-based), or whether location measurements should be provided for location calculation at the LMF (UE-assisted). UE1 may instigate the procedure for step 6b and provides the sidelink location measurements or location estimates to the LMF.

6b.	UE1 instigates the SLPP Location Information Transfer procedure specified in sub-clause X.Y.Z among UEs 1 to n in which UE1 to n obtain sidelink location measurements and UEs 2 to n transfer their sidelink location measurements to UE1.

NOTE 1:	Steps 4a, 5a, 6a and 4b, 5b, 6b could be performed in any order and may also overlap.

NOTE 2:	Steps 4b, 5b, and 6b are not necessarily serialised with steps 4a, 5a, and 6a, respectively.

7.	The LMF sends a SL-MO-LR Response to the AMF which forwards the SL-MO-LR Response to UE1. If UE1 requested position calculation assistance at step 3, the SL-MO-LR Response includes the calculated UE locations obtained from step 6.

8.	If requested at step 3, the AMF provides the location results to the AF/LCS Client.