3GPP TSG-RAN WG2#123	R2-2308424

Toulouse, August 21st – 25th, 2023

Agenda Item:	7.13.8

Source:	Ericsson

Title:	Discussion on fast MCG recovery and SCG failure optimization

Document for:	Discussion, Decision

# 1	Introduction

In this contribution, we discuss the MRO enhancements related to fast MCG link recovery and near MCG recovery failure cases based on the recent discussion in the 3GPP RAN2 #122 meeting and an email discussion [1].

# 2	MRO enhancements for fast MCG link recovery

In case of successive failures of the MCG and SCG, two scenarios may be possible:

(1) the SCG failure happens before the MCG failure, or

(2) SCG failure happens after the MCG failure.

In the first case, that the SCG failure has happened before the MCG failure happened, the MCGFailureInformation message will not even be initiated as it requires the availability of the SCG leg to be able to send the MCGFailureInformation message to the network.

In the second case, the MCGFailureInformation message may be initiated but its delivery to the network will fail as SCG is no longer available or the SCG failure may be detected even after successful transmission of the MCGFailureInformation.

To facilitate these cases, the network needs to know which of the link failures (MCG or SCG failures) happened first, that is, whether it was SCG or MCG failure which happened first. This may be indicated in the already existing RLF report as a flag, as the time between those failures might be rather short and would not allow for generation of a new report.

Additionally, it may be useful for the network to know the time between those two failures. When it comes to a case of SCG deactivation before UE sends the MCGFailureInformation, an important distinction to make is whether the MCGFailureInformation message can at all be generated if the SCG deactivation occurred before it.

UE logs a flag indicating which of the link failures, SCG or MCG failure, happened first in the RLF report.

UE logs the time between the SCG and MCG failures in the RLF report.

Near MCG recovery failure report’s triggering threshold

The following proposal was discussed in the Tdoc R2-2305779 and in the email discussion [1]:

For near failure fast MCG link recovery, one T316 related triggering threshold is configured, and UE only generates the report when the threshold is met.

While the proponents of this proposal state that the reporting overhead would be reduced, the RLF reporting is not a frequent event and configuring threshold for this reporting case may just increase the implementation complexity rather than giving a significant overhead reduction. RLF by itself is not a very frequent event and hence we assume successful MCG recovery is not very frequent as well, so there is no need to limit it to specific cases.

In addition, the RLF report is already logged by the UE for the RLF at the MCG leg and stored at the UE, so there is no need to have a triggering threshold to log the elapsed time information of T316 timer in that existing report. Needless to say, adding the elapsed time of the T316 timer in the existing RLF report enables the network to figure out the retrieved RLF report is associated to a failure that was recovered by a fast MCG recovery procedure.

Moreover, this scheme would require to specify a new configuration signalling. Whereas for RLF-Report we do not need specific configuration. So it would be more implementation effort both for UE and gNB

Based on the reasons above, we propose the following.

No need to introduce a triggering threshold condition for successful (near failure) MCG recovery scenario.

UE logs the elapsed time of T316 timer (elapsed time from transmission of MCGFailureInformation to the reception of the response from the network) in the already existing RLF report.

SCG failure types in RLF report

In the RAN3#119 meeting, additional information were agreed to be reported by a UE:

- PSCell where SCG failure happened, and

- the cause of the fast MCG recovery failure containing at least:

  - T316 expiry,

  - SCG failure, and

  - SCG was deactivated or other cases where SCG is not available

- SCG failure type (at least t310-Expiry, randomAccessProblem, rlc-MaxNumRetx) if the cause of the fast MCG recovery is SCG failure.

In addition to the highlighted SCG Failure types agreed by RAN3, there are other SCG Failure types that are required to be logged by the UE upon SCG Failure as part of RLF report. A list of the SCG failure types based on the RRC procedural text is shown in the following excerpt.

All these failure types, highlighted here, are included in SCGFailureInformation message and could not be sent by a UE to the network in the case of MCG failure simultaneous with the SCG failure. However, this information is of use to the network for failure analysis and DC operation optimization. Therefore, we propose the following:

Upon MCG recovery failure due to SCG failure UE logs any possible SCG failure type (that in legacy may be included in the SCGFailureInformation) as MCG recovery failure cause in the RLF report.

# 3	Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	UE logs a flag indicating which of the link failures, SCG or MCG failure, happened first in the RLF report.

Proposal 2	UE logs the time between the SCG and MCG failures in the RLF report.

Proposal 3	No need to introduce a triggering threshold condition for successful (near failure) MCG recovery scenario.

Proposal 4	UE logs the elapsed time of T316 timer (elapsed time from transmission of MCGFailureInformation to the reception of the response from the network) in the already existing RLF report.

Proposal 5	Upon MCG recovery failure due to SCG failure UE logs any possible SCG failure type (that in legacy may be included in the SCGFailureInformation) as MCG recovery failure cause in the RLF report.

# References

- [1] Summary of [Post122][584][R18 SON/MDT] Open issues on fast MCG recovery.