3GPP TSG-RAN WG2 Meeting #123	R2-2308551

Toulouse, France, August 21st – August 25th, 2023

Agenda item:	7.19.3 Further reduced UE complexity in FR1

Source: 	Semtech

Title: 	Access restrictions for eRedCap UE

Document for:	Discussion/Decision

# Introduction

The Rel-18 eRedCap WID “New WID on enhanced support of reduced capability NR devices” [1] was updated in RAN#98e, that includes the objectives for the new Rel-18 eRedCap. This tdoc discusses the considerations for access restrictions.

# Discussion

## Access restrictions and barring

From RAN#99 RP-230778[2]:

Rel-18 eRedCap UE capable of 20MHz + PR1 and Rel-18 eRedCap UE capable of BW3/PR3 + PR1 are designed/targeted to same peak data rate, i.e., 10Mbps

Note 1: Peak data rate of "Rel-18 eRedCap: UE capable of 20MHz + PR1" and "Rel-18 eRedCap: UE capable of BW3/PR3 + PR1" is same including unicast and broadcast respectively.

Note 2: PRB processing capability of "Rel-18 eRedCap: UE capable of 20MHz + PR1" is not limited to "25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS" and it corresponds to PRB size corresponding to 20 MHz.

Note 3: The only difference between "Rel-18 eRedCap: UE capable of 20MHz + PR1" and "Rel-18 eRedCap: UE capable of BW3/PR3 + PR1" is Note 2 and vLayers·Qm·f   in order to have the same peak rate.

Note 4: The initial access procedure of Rel-18 eRedCap UE capable of 20MHz + PR1 is realized by following:

- Same as Rel-18 eRedCap UE capable of BW3/PR3 + PR1

In the endorsed proposal above from RAN#99, it was agreed that both of these configurations are supported:

- Rel-18 eRedCap UE (PR1) with 20MHz capability and peak data rate reduction

- Rel-18 eRedCap UE (BW3/PR3 + PR1) with limited PRB (5MHz) capability and peak data rate reduction

From the following agreement in RAN2#121bis [3], the FFS on the granularity of the access enablement need to be resolved. It was discussed in RAN2#122 [4] but not resolved.

SIB1 should be able to indicate whether the cell enables access for eRedCap UEs or not (assuming that eRedCap UE is not allowed to access to the legacy cell nor the cell not supporting eRedCap). FFS on the relationship and granularity with the access control/cell barring purpose indication.

Having separate access enablement indications for Rel-18 eRedCap BW3/PR3+PR1 devices and Rel-18 eRedCap PR1-only would be beneficial for several reasons.

Reason #1: Without separate access enablement indications, networks would need to fully implement network support for both BW3/PR3+PR1 devices and PR1-only devices before deployment.

Reason #2: Both BW3/PR3+PR1 devices and PR1-only devices need to be available for interoperability testing. With separate access enablement indications, the network implementation can instead be prioritized based on device availability.

Reason #3: To allow reuse of the RAN scheduler, operators could limit the BW3+PR1 devices to only use carriers with 5 MHz BW.

Reason #4: To improve spectral efficiency, operators could limit the BW3+PR1 devices to only use carriers with 5 MHz BW.  Since PR1-only is 20MHz for PUSCH/PDSCH, a PR1 device has better performance due to increased frequency diversity compared to BW3+PR1 devices, thus, operators may want to limit the BW3+PR1 devices to only use carriers with 5 MHz BW while allowing PR1-only devices on wider carriers where the frequency diversity can be utilized.

There are no significant drawbacks to having separate access enablement indications as there would be very small overhead or no overhead in the SIB, and it would not be a significant specification effort.

Separate access enablement indications for BW3+PR1 devices and PR1 only devices should be supported

# Summary

- Separate access enablement indications for BW3+PR1 devices and PR1 only devices should be supported

# References

RP-223544 New WID on enhanced support of reduced capability NR devices

RP-230778 Proposal for PR1 in eRedCap, March 2023

3GPP RAN2#121bis Draft_R2_121bis_meeting_report_v1, April 2023

3GPP RAN2#122 Draft_R2_122_meeting_report_v2, May 2023