3GPP TSG-RAN WG2 Meeting #123                                    	R2-2308555

# Toulouse, France: 21st August – 25th August 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.331 | CR | xxxx | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on UE location information in NB-IoT RLF report |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT-Core |  |  |  |  |  | Date: |  |  | 2023-08-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For the NB-IoT UEs, the immediate MDT (i.e., ReportConfigEUTRA) is not configured. Furthermore, the otherConfig is also not configured to the NB-IoT UEs. According to the current eNB behavior, the obtainLocation is configured by the eNB only if includeLocationInfo is configured for one or more measurements. Moreover, according to the current UE behavior, UE reports the location information in any subsequent measurement report including RLF report and SCGFailureInformationNR if obtainLocation is configured at the UE. #### 5.3.10.9 Other configuration 1> if the received otherConfig includes the obtainLocation: 2> attempt to have detailed location information available for any subsequent measurement report; NOTE 1: The UE is requested to attempt to have valid detailed location information available whenever sending a measurement report for which it is configured to include available detailed location information. The UE may not succeed e.g. because the user manually disabled the GPS hardware, due to no/poor satellite coverage. Further details, e.g. regarding when to activate GNSS, are up to UE implementation. NOTE 1a: Any subsequent measurement report includes RLF report and SCGFailureInformationNR.. Note that the RLF report is an optional feature for NB-IoT UE. If the UE supports it, then after RLF, the UE prepares the RLF report. However, without the network indication, the NB-IoT UEs may not be aware of whether they should also include location information if the UE has GNSS location information. Without the network indication, similar to the normal UE behavior, UE should not report the location information in any report (i.e., RLF report). Furthermore, according to the current procedure, the NB-IoT UEs do not report the availability of the RLF report to the network, and the network may request NB-IoT UE to report the RLF report without explicit indication in the UEInformationRequest-NB by configuring rlf-ReportReq. Therefore, if the network is interested in obtaining the location information in the RLF report from NB-IoT UEs, it should configure a flag in the UEInformationRequest-NB to report location information in the RLF report. |
| Summary of change: |  | It is clarified that if the network is interested in obtaining the location information in the RLF report from NB-IoT UEs, it should configure a flag in the UEInformationRequest-NB to report location information in the RLF report. - If the flag (to indicate NB-IOT UEs to report location information in the RLF report) is configured and UE has available location information, UE should report location information in the RLF report. - If the flag (to indicate NB-IOT UEs to report location information in the RLF report) is not configured, then NB-IoT UE does not report location information in the RLF report. Impact analysis: Impacted functionality: RLF report Inter-operability: No inter-operability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | For NB-IoT UEs, currently, there is no indication from the network if UE should report location information in SON reports (i.e., RLF report). |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.11.3, 6.7.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

#### 5.3.11.3	Detection of radio link failure

The UE shall:

1>	in case any DAPS bearer is configured, only the target PCell is considered in the following;

1>	upon T310 expiry; or

1>	upon T312 expiry; or

1>	upon T318 expiry; or

1>	upon random access problem indication from MCG MAC while neither T300, T301, T304 nor T311 is running; or

1>	upon indication from MCG RLC, which is allowed to be send on PCell, that the maximum number of retransmissions has been reached for an SRB or DRB:

2>	consider radio link failure to be detected for the MCG i.e. RLF;

2>	discard any segments of segmented RRC messages received;

2>	store the following radio link failure information in the VarRLF-Report (VarRLF-Report-NB in NB-IoT) by setting its fields as follows:

3>	clear the information included in VarRLF-Report (VarRLF-Report-NB in NB-IoT), if any;

3>	set the plmn-IdentityList to include the list of EPLMNs stored by the UE (i.e. includes the RPLMN);

3>	set the measResultLastServCell to include the RSRP and RSRQ, if available, of the PCell based on measurements collected up to the moment the UE detected radio link failure;

3>	except for NB-IoT, set the measResultNeighCells to include the best measured cells, other than the PCell, ordered such that the best cell is listed first, and based on measurements collected up to the moment the UE detected radio link failure, and set its fields as follows;

4>	if the UE was configured to perform measurements for one or more EUTRA frequencies, include the measResultListEUTRA;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring UTRA frequencies, include the measResultListUTRA;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring GERAN frequencies, include the measResultListGERAN;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring CDMA2000 frequencies, include the measResultsCDMA2000;

4>	if the UE was configured to perform measurement reporting, not related to NR sidelink communication, for one or more neighbouring NR frequencies, include the measResultListNR;

4>	for each neighbour cell included, include the optional fields that are available;

NOTE 1:	The measured quantities are filtered by the L3 filter as configured in the mobility measurement configuration. The measurements are based on the time domain measurement resource restriction, if configured. Exclude-listed cells are not required to be reported.

3>	except for NB-IoT, if available, set the logMeasResultListWLAN to include the WLAN measurement results, in order of decreasing RSSI for WLAN APs;

3>	except for NB-IoT, if available, set the logMeasResultListBT to include the Bluetooth measurement results, in order of decreasing RSSI for Bluetooth beacons;

3>	Except for NB-IoT, if detailed location information is available, set the content of the locationInfo as follows:

4>	include the locationCoordinates;

4>	include the horizontalVelocity, if available;

3>	For NB-IoT, if location-RLF-ReportReq is configured in the UEInformationRequest-NB and if detailed location information is available, set the content of the locationInfo as follows:

4>	include the locationCoordinates;

4>	include the horizontalVelocity, if available;

3>	set the failedPCellId to the global cell identity, if available, and otherwise , except for NB-IoT, to the physical cell identity and carrier frequency of the PCell where radio link failure is detected;

3>	except for NB-IoT, set the tac-FailedPCell to the tracking area code, if available, of the PCell where radio link failure is detected;

3>	except for NB-IoT, if an RRCConnectionReconfiguration message including the mobilityControlInfo was received before the connection failure:

4>	if the last RRCConnectionReconfiguration message including the mobilityControlInfo concerned an intra E-UTRA handover:

5>	include the previousPCellId and set it to the global cell identity of the PCell where the last RRCConnectionReconfiguration message including mobilityControlInfo was received;

5>	set the timeConnFailure to the elapsed time since reception of the last RRCConnectionReconfiguration message including the mobilityControlInfo;

4>	if the last RRCConnectionReconfiguration message including the mobilityControlInfo concerned a handover to E-UTRA from UTRA and if the UE supports Radio Link Failure Report for Inter-RAT MRO:

5>	include the previousUTRA-CellId and set it to the physical cell identity, the carrier frequency and the global cell identity, if available, of the UTRA Cell in which the last RRCConnectionReconfiguration message including mobilityControlInfo was received;

5>	set the timeConnFailure to the elapsed time since reception of the last RRCConnectionReconfiguration message including the mobilityControlInfo;

4>	if the last RRCConnectionReconfiguration message including the mobilityControlInfo concerned a handover to E-UTRA from NR and if the UE supports Radio Link Failure Report for Inter-RAT MRO NR:

5>	include the previousNR-PCellId and set it to the global cell identity of the PCell where the last RRCConnectionReconfiguration message including mobilityControlInfo was received embedded in NR RRC message MobilityFromNRCommand message as specified in TS 38.331 [82] clause 5.4.3.3;

5>	set the timeConnFailure to the elapsed time since reception of the last RRCConnectionReconfiguration message including the mobilityControlInfo embedded in NR RRC message MobilityFromNRCommand message as specified in TS 38.331 [82] clause 5.4.3.3.

3>	except for NB-IoT, if the UE supports QCI1 indication in Radio Link Failure Report and has a DRB for which QCI is 1:

4>	include the drb-EstablishedWithQCI-1;

3>	except for NB-IoT, set the connectionFailureType to rlf;

3>	except for NB-IoT, set the c-RNTI to the C-RNTI used in the PCell;

3>	except for NB-IoT, set the rlf-Cause to the trigger for detecting radio link failure;

2>	if the UE is configured with (NG)EN-DC; and

2>	if T316 is configured; and

2>	if SCG transmission is not suspended; and

2>	if the SCG is not deactivated; and

2>	if neither NR PSCell change nor NR PSCell addition is ongoing (i.e. T304 for the NR PSCell is not running as specified in TS 38.331 [82], clause 5.3.5.5.2, in (NG)EN-DC):

3>	initiate the MCG failure information procedure as specified in 5.6.26 to report MCG radio link failure;

2>	else:

3>	if AS security has not been activated:

4>	if the UE is a NB-IoT UE:

5>	if the UE is connected to EPC and the UE supports RRC connection re-establishment for the Control Plane CIoT EPS optimisation; or

5>	if the UE is connected to 5GC, the UE supports RRC connection re-establishment for the Control Plane CIoT 5GS optimisation and the UE is configured with a truncated 5G-S-TMSI:

6>	initiate the RRC connection re-establishment procedure as specified in 5.3.7;

5>	else:

6>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'RRC connection failure';

4>	else:

5>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'other';

3>	else:

4>	initiate the connection re-establishment procedure as specified in 5.3.7;

In case of DC or NE-DC, the UE shall:

1>	upon T313 expiry; or

1>	upon random access problem indication from SCG MAC; or

1>	upon indication from SCG RLC, which is allowed to be sent on PSCell, that the maximum number of retransmissions has been reached for an SCG, for a split DRB or for a split SRB:

2>	consider radio link failure to be detected for the SCG i.e. SCG-RLF;

2>	if the UE is configured with DC; or

2>	if the UE is configured with NE-DC and MCG transmission is not suspended:

3>	initiate the SCG failure information procedure as specified in 5.6.13 to report SCG radio link failure;

2>	else:

3>	initiate the connection re-establishment procedure as specified in TS 38.331 [82], clause 5.3.7.

In case of CA PDCP duplication, the UE shall:

1>	upon indication from an RLC entity, which is restricted to be sent on SCell only, that the maximum number of retransmissions has been reached:

2>	initiate the failure information procedure as specified in 5.6.21 to report RLC failure of type duplication;

If any DAPS bearer is configured and T304 is running, the UE shall:

1>	upon T310 expiry for the source PCell; or

1>	upon random access problem indication from source MCG MAC; or

1>	upon indication from source MCG RLC, which is allowed to be sent on source PCell, that the maximum number of retransmissions has been reached for an DRB:

2>	consider radio link failure to be detected for the source MCG;

2>	suspend the transmission of all DRBs in the source MCG;

2>	reset MAC for the source MCG;

2>	release the source connection;

The UE may discard the radio link failure information, i.e. release the UE variable VarRLF-Report (VarRLF-Report-NB in NB-IoT), 48 hours after the radio link failure is detected, upon power off or upon detach, and for NB-IoT, upon entering another RAT.

### <<skipped>>

### 6.7.2	NB-IoT Message definitions

<<skipped>>

#### –	UEInformationRequest-NB

The UEInformationRequest-NB is the command used by E-UTRAN to retrieve information from the UE.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: E-UTRAN to UE

UEInformationRequest-NB message

-- ASN1START

UEInformationRequest-NB-r16	::=			SEQUENCE {

rrc-TransactionIdentifier			RRC-TransactionIdentifier,

criticalExtensions					CHOICE {

ueInformationRequest-r16			UEInformationRequest-NB-r16-IEs,

criticalExtensionsFuture			SEQUENCE {}

}

}

UEInformationRequest-NB-r16-IEs ::=		SEQUENCE {

rach-ReportReq-r16					BOOLEAN,

rlf-ReportReq-r16					BOOLEAN,

anr-ReportReq-r16					BOOLEAN,

lateNonCriticalExtension			OCTET STRING						OPTIONAL,

nonCriticalExtension				UEInformationRequest-NB-r17-IEs 	OPTIONAL

}

UEInformationRequest-NB-r17-IEs ::=		SEQUENCE {

location-RLF-ReportReq-r17			BOOLEAN,

nonCriticalExtension				SEQUENCE {}							OPTIONAL

}

-- ASN1STOP

UEInformationRequest-NB field descriptions

anr-ReportReq Indicates whether the UE shall report, if available, ANR measurement information.

rach-ReportReq Indicates whether the UE shall report, if available, information about the random access procedure.

rlf-ReportReq Indicates whether the UE shall report, if available, information about radio link failure.

location-RLF-ReportReq Indicate whether the UE shall include, if available, location information in the radio link failure report. E-UTRAN configures location-RLF-ReportReq only when rlf-ReportReq is configured.

#### –	UEInformationResponse-NB

The UEInformationResponse-NB message is used by the UE to transfer the information requested by the E-UTRAN.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to E-UTRAN

UEInformationResponse-NB message

-- ASN1START

UEInformationResponse-NB-r16		::=		SEQUENCE {

rrc-TransactionIdentifier			RRC-TransactionIdentifier,

criticalExtensions					CHOICE {

ueInformationResponse-r16			UEInformationResponse-NB-r16-IEs,

criticalExtensionsFuture			SEQUENCE {}

}

}

UEInformationResponse-NB-r16-IEs ::=	SEQUENCE {

rach-Report-r16							RACH-Report-NB-r16					OPTIONAL,

rlf-Report-r16							RLF-Report-NB-r16					OPTIONAL,

anr-MeasReport-r16						ANR-MeasReport-NB-r16				OPTIONAL,

lateNonCriticalExtension				OCTET STRING						OPTIONAL,

nonCriticalExtension					SEQUENCE {}							OPTIONAL

}

RACH-Report-NB-r16 ::=					SEQUENCE {

numberOfPreamblesSent-r16				INTEGER (1..64),

contentionDetected-r16					BOOLEAN,

initialNRSRP-Level-r16					INTEGER (0..2),

edt-Fallback-r16						BOOLEAN

}

RLF-Report-NB-r16 ::=					SEQUENCE {

failedPCellId-r16						CellGlobalIdEUTRA,

reestablishmentCellId-r16				CellGlobalIdEUTRA					OPTIONAL,

locationInfo-r16						LocationInfo-r10					OPTIONAL,

measResultLastServCell-r16				SEQUENCE {

nrsrpResult-r16							NRSRP-Range-NB-r14,

nrsrqResult-r16							NRSRQ-Range-NB-r14				OPTIONAL

},

timeSinceFailure-r16					TimeSinceFailure-r11				OPTIONAL

}

-- ASN1STOP

UEInformationResponse-NB field descriptions

anr-MeasReport Indicates the ANR measurement information.

contentionDetected Value TRUE indicates that contention was detected for at least one of the transmitted preambles, see TS 36.321 [6].

edt-Fallback Value TRUE indicates that EDT fallback indication was received from the lower layers, see TS 36.321 [6].

failedPCellId Indicates the PCell in which RLF is detected.

initialNRSRP-Level Indicates the NRSRP level of the NPRACH resource selected for the first preamble transmission.

measResultLastServCell Refers to the last measurement results taken in the PCell, where radio link failure happened.

numberOfPreamblesSent Indicates the number of RACH preambles that were transmitted. Corresponds to parameter PREAMBLE_TRANSMISSION_COUNTER in TS 36.321 [6].

reestablishmentCellId Indicates the cell in which the re-establishment attempt was made after connection failure.

timeSinceFailure Indicates the time that elapsed since the connection failure. Value in seconds. The maximum value 172800 means 172800s or longer.

END OF CHANGE