# 3GPP TSG RAN WG2 Meeting #122                          	    R2-2308592
Toulouse, France, August 21-25, 2023

Agenda item:	7.15.6

Source:	Qualcomm Incorporated

Title:	Discussion on SL CA

WID/SID:	NR_SL_enh2 – Release 18

Document for:	Discussion and Decision

# Introduction

In this contribution, we discuss design consideration for sidelink carrier aggregation (SL CA).

# Discussion

## Sidelink carrier mapping in NR V2X

Sidelink carrier mapping was discussed at RAN2 #121-bis e-meeting, and the following agreement was made [1].

Agreement:

Proposal 3:	For groupcast/broadcast, as in LTE SL CA, the carrier(s) that can be used for transmitting data are configured by V2X layer for the L2 destination. FFS on backwards compatibility issue.

As specified in TS 23.287 [2] (shown below), NR V2X service layer maps a service type to V2X frequencies with Geographical Area(s), where the frequencies are frequency bands (not carriers or carrier components) in a Geographical Area. The frequency resources (i.e., carriers or carrier components) are based on network’s configuration or pre-configuration. Therefore, NR V2X service layer does not map a service type to carriers or carrier components (e.g., the NR V2X layer does not know the channelization which defines frequency carriers or carrier components).

#### 5.1.2.1	Policy/Parameter provisioning

The following sets of information for V2X communications over PC5 reference point is provisioned to the UE:

….

2) Radio parameters when the UE is "not served by E-UTRA" and "not served by NR":

- Includes the radio parameters per PC5 RAT (i.e., LTE PC5, NR PC5) with Geographical Area(s) and an indication of whether they are "operator managed" or "non-operator managed". These radio parameters (e.g., frequency bands) are defined in TS 36.331 [14] and TS 38.331 [15]. The UE uses the radio parameters to perform V2X communications over PC5 reference point when "not served by E-UTRA" and "not served by NR" only if the UE can reliably locate itself in the corresponding Geographical Area. Otherwise, the UE is not authorized to transmit.

NOTE 2: Whether a frequency band is "operator managed" or "non-operator managed" in a given Geographical Area is defined by local regulations.

…

6)	Policy/parameters when NR PC5 is selected:

- The mapping of V2X service types to V2X frequencies with Geographical Area(s).

- The mapping of V2X service types to the default mode of communication (i.e., broadcast mode, groupcast mode or unicast mode).

- The mapping of V2X service types to Destination Layer-2 ID(s) for broadcast.

- The mapping of V2X service types to Destination Layer-2 ID(s) for groupcast mode communication.

- The mapping of V2X service types to default Destination Layer-2 ID(s) for initial signalling to establish unicast connection.

…

#### 5.1.2.2	Principles for applying parameters for V2X communications over PC5 reference point

…

-	The UE shall use radio resources for V2X communications over PC5 reference point as follows:

- While a UE has a serving cell and is camped on a cell and the UE intends to use for V2X service the radio resources (i.e., carrier frequency) operated by this cell, then the UE shall use the radio resource description indicated by this cell the UE is camped on and ignore any radio resource description of the same radio resource provisioned in the ME or the UICC. If the cell does not provide radio resources for V2X service, the UE shall not perform V2X message transmission and reception on radio resources operated by this cell.

- If the UE intends to use "operator-managed" radio resources (i.e. carrier frequency) for V2X service that are not operated by the UE's serving cell, as specified in clause 5.1.2.1, or if the UE is out of coverage, the UE shall search for a cell in any PLMN that is operating the provisioned radio resources (i.e. carrier frequency) as defined in TS 36.300 [9] and TS 36.304 [10] (if LTE based PC5 is selected for the V2X communication) or as defined in TS 38.300 [11] and TS 38.304 [12] (if NR based PC5 is selected for the V2X communication), and:

…

-  If the UE intends to use "non-operator-managed" radio resources (i.e., carrier frequency) for V2X service, according to TS 36.331 [14] or TS 38.331 [15] and as specified in clause 5.1.2.1, then the UE shall perform V2X communication over PC5 using resource provisioned in the ME or the UICC. If no such provision exists in the ME or the UICC or the provision does not authorize V2X communications over PC5 reference point, then the UE is not authorized to transmit.

…

##### 5.4.1.1.3 Handling of PC5 QoS Flows based on PC5 QoS Rules

…

In addition, the V2X layer also provides the communication mode (e.g., broadcast, groupcast, unicast) and radio frequencies to the AS layer for the PC5 operation. The radio frequencies are determined based on the V2X service type. The V2X layer ensures that V2X service types associated with different radio frequencies are classified into distinct PC5 QoS Flows.

…

Observation 1. NR V2X service layer maps a service type to frequency bands, not frequency carriers or carrier components.

Observation 2. For NR V2X, frequency carriers or carrier components are preconfigured or configured for a sidelink communication.

Proposal 1. RAN2 needs to further discuss how to allocate carriers or carrier components for sidelink carrier aggregation.

## Backward compatibility issue

Since a Rel-16/Rel-17 UE do not support sidelink carrier aggregation and can operate with only one carrier, the Rel-16/Rel-17 UE may not be able to receive all TB transmissions which are broadcasted or groupcasted by a Rel-18 UE using sidelink CA with more than one sidelink carrier. How to ensure that the Rel-16/Rel-17 UE will not miss any transmission broadcasted or groupcasted from Rel-18 UE needs to be addressed at RAN2.

How to address backward compatibility was discussed at RAN2 #122 with the follows [3].

FFS on backward compatibility issue in SL CA (for GC/BC)

- No issue with service-to-carrier mapping (P3:4668)

- Need TX profile to handle the compatibility issue (P2:5093)

As described previously, NR V2X service layer does not support service-to-carrier mapping. To support service-to-carrier mapping at service layer requires extra work for upper layer working group which has no time allocation for studying how to map a service type to frequency carriers or carrier components without understanding the channelization in a geographical area.

As described in TS 23.287, frequency carriers or carrier components (i.e., frequency resources) are allocated via pre-configuration or configuration (e.g., based on the channelization). Therefore, it’s more suitable for AS layer to select multiple carriers or carrier components for sidelink carrier aggregation or a carrier or carrier component supported by legacy UEs for backward compatibility.

Proposal 2. Carrier selection for backward compatibility should be conducted at AS layer.

# Conclusion

In this contribution, we discussed RAN2 scope on sidelink carrier aggregation (CA) and concluded with the following observations and proposals.

Observation 1. NR V2X service layer maps a service type to frequency band, not frequency carriers or carrier components.

Observation 2. For NR V2X, frequency carriers or carrier components are preconfigured or configured for a sidelink communication.

Proposal 1. RAN2 needs to further discuss how to allocate carriers or carrier components for sidelink carrier aggregation.

Proposal 2. Carrier selection for backward compatibility should be conducted at AS layer.

# References

[1] Session chair’s meeting note, “RAN2-121bis-e_R16_NR SL_R17&R18_NR SL enh_20230425-EOM_r1”, 3GPP TSG RAN2 #121-bis e-meeting, April 17 – 26, 2023.

[2] 3GPP TS 23.287 “Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services (Release 17)”, Version 17.5.0 (2022-12).

[3] R2-2306545 “Report from session on LTE V2X and NR SL” Session chair (Samsung), RAN2 #122 Incheon, South Korea, May 22 – 26 2023.