3GPP TSG-RAN2 #123	R2- 2308594

# Toulouse, France, August 21st – 25th, 2023

Agenda item:		7.11.2.2 (NR_MBS_enh-Core)

Source:	LG Electronics Inc.

Title: 	Discussion on UP issues for Multicast in RRC Inactive

Document for:	Discussion and Decision

1.	Introduction

This document discusses user plane related issues for multicast in RRC_INACTIVE state. The agreements related to user plane in R2-121bis-e are as follows.

Agreements in R2-121bis-e meeting HARQ feedback related information in the DCI is not needed or can be ignored for multicast transmission to RRC_INACTIVE UE. The HARQ operation for multicast reception in RRC_INACTIVE is same as the operation without HARQ feedback in RRC_CONNECTED state. The multicast transmission in RRC_INACTIVE is performed via beam sweeping based on SSB index like broadcast MBS (i.e. beam information is not needed in DCI). For MTCH, RAN2 assumes to reuse the same DCI format of R17 multicast (i.e. DCI format 4-1/4-2) for dynamic scheduling of multicast in RRC INACTIVE. RAN2 assumes for MCCH scheduling, DCI format 4-0 is used. We will ask RAN1 to confirm whether it is feasible and whether both 4-1 and 4-2 are needed. We will also indicate other relevant agreements to RAN1 (e.g. on beam sweeping etc.) On support of multicast SPS in RRC_INACTIVE, postpone RAN2 discussion to next meeting. On DRX operation for multicast in RRC_INACTIVE, take the multicast DRX as baseline. FFS handling on PTM related HARQ RTT Timer and DRX Retransmission Timer. The common LCID space is used for multicast MRB and unicast DRB regardless of UE RRC state (i.e. no change on the LCID table for MTCH). Postpone the UP discussion on L2 operation during RRC state transition until the signaling design of PTM configuration in RRCRelease message is concluded. Postpone the discussion on L2 operation during mobility to next RAN2 meeting. Including the following two issues in LS to RAN1: - Issue 1: RAN1 to confirm RAN2 understanding that PDSCH aggregation is supported for multicast MTCH in RRC_INACTIVE (as that is supported in Rel-17 multicast MTCH in RRC_CONNECTED as well as broadcast MTCH). - Issue 2: RAN1 to check the feasibility of following Rel-17 CSS design for multicast MTCH and MCCH: 1) reusing the same CSS for multicast MTCH in RRC_INACTIVE (same as multicast MTCH in RRC_CONNECTED); 2) separate CSS for MCCH and MTCH. Change the working agreement to the agreement below: - Agreement: The same CFR is used for multicast MCCH and MTCH. It can be revisited if there is any issue found, e.g. for RedCap UEs. UE in RRC CONNECTED state is not required to read multicast MCCH to be able to receive multicast MBS service i.e. the UE receives the PTM configuration via dedicated signalling. This can be revisited if issues with service continuity are identified.

The agreements related to multicast reception in R2-122 are as follows.

The multicast MCCH configuration takes the broadcast MCCH configuration structure (i.e., mcch-Config-r17) as baseline. To notify the multicast MCCH change, change notification mechanism for Rel-17 broadcast MCCH is the baseline. Working assumption (to be confirmed by RAN1 via pending reply LS): One bit in the MCCH DCI is used to notify the change of the multicast MCCH. We reuse the bit used for MCCH change indication from Rel-17 MBS broadcast. This does not cover session deactivation which is FFS. It is not supported to provide the PTM configuration of intra-gNB neighbour cells in the dedicated signalling. For PTM configuration structure on the multicast MCCH, Rel-17 broadcast PTM configuration structure is taken as baseline. As a baseline, The PTM configuration in the RRCRelease message with suspendconfig has the same structure as the PTM configuration in multicast MCCH. FFS how existing MRBs are handled. Introduce a new indication per tmgi in the group paging which informs Rel-18 UEs having a valid PTM configuration to receive the multicast in RRC_INACTIVE. MCCH is used for notifying MC session deactivation for multicast reception in RRC_INACTIVE to enable Rel-18 UE to stay in RRC_INACTIVE and stop monitoring corresponding G-RNTI. This is assumed to have no/minor impact on RAN1/PHY

2.	Discussion

Whether to support SPS for multicast reception in RRC_INACTIVE state has been discussed but not concluded.

It is generally assumed that when a multicast service is provided in RRC_INACTIVE, the same PDCCH/PDSCH resource is used for RRC_CONNECTED UEs. We think it can be also applied to SPS PDSCH resource. That is, if a multicast session is served by SPS in RRC_CONNECTED, the SPS PDSCH resources can be shared with RRC_INACTIVE UEs. Regarding HARQ feedback for multicast SPS, since HARQ feedback is not supported for multicast reception in RRC_INACTIVE, the same rule can be applied to multicast SPS.

Therefore, we think MBS SPS can be supported without HARQ feedback in RRC_INACTIVE state.

Proposal 1. Support MBS SPS without HARQ feedback in RRC_INACTIVE state.

When multicast SPS is assumed to be supported in RRC_INACTIVE, the next issue to be discussed is whether to support SPS activation and SPS deactivation without HARQ feedback.

For comparison, we first go through HARQ feedback and SPS activation for MBS SPS in RRC_CONNECTED.

HARQ feedback for multicast can be enabled or disabled for dynamic scheduled PDSCH or SPS PDSCH in RRC_CONNECTED. For SPS activation for MBS SPS, SPS activation is indicated by contents of PDCCH addressed to G-CS-RNTI. HARQ feedback for SPS PDSCH after SPS activation can be disabled in RRC_CONNECTED by following conditions which is followed by not instructing the physical layer to generate acknowledgement in the MAC specification.

1> if the HARQ process is associated with a transmission indicated with a G-RNTI or a G-CS-RNTI or a configured downlink assignment for MBS multicast and HARQ feedback is disabled for this G-RNTI or G-CS-RNTI, as specified in clause 18 of TS 38.213 [6]; or

When HARQ feedback is disabled for multicast, HARQ feedback for SPS PDSCH after SPS activation is not provided. It means that L1 SPS activation command without HARQ feedback is already supported for MBS SPS in RRC_CONNECTED.

Observation 1. SPS activation without HARQ feedback is already supported for MBS SPS in RRC_CONNECTED.

Similarly, we think L1 SPS activation without HARQ feedback can be supported for MBS SPS in RRC_INACTIVE. However, there may be a reliability issue because there is no HARQ feedback for L1 SPS activation. Thus, the gNB may need to transmit SPS activation multiple times to increase the reliability.

Proposal 2. Support L1 SPS activation without HARQ feedback for MBS SPS in RRC_INACTIVE.

For SPS deactivation, SPS deactivation without HARQ feedback can be supported in RRC_INACTIVE considering that SPS activation without HARQ feedback is supported. Possibility of loss is same for SPS activation and SPS deactivation because they are indicated by contents of PDCCH addressed to the same G-CS-RNTI. Thus, the gNB may also need to transmit SPS deactivation multiple times, similar to SPS activation case.

Proposal 3. Support L1 SPS deactivation without HARQ feedback for MBS SPS in RRC_INACTIVE.

For SPS release, the MBS SPS is released by dedicated RRC signalling in RRC_CONNECTED. If the same method for SPS release is used for RRC_INACTIVE UEs, multiple UEs need to move to RRC_CONNECTED to receive SPS release message. It may cause problems such as collision of RA procedures if multiple UEs perform SPS release at the same time. Also, it seems redundant action to move to RRC_CONNECTED only for receiving SPS release message. Therefore, it is worth discussing how to release SPS in RRC_INACTIVE without state transition to RRC_CONNECTED.

In Rel-17 MBS, group paging is introduced to indicate a multicast session start to a group of UEs. We think that a new group indication can be introduced to indicate SPS release to a group of UEs without state transition.

Proposal 4. Introduce group paging for SPS release in RRC_INACTIVE.

Regarding multicast DRX operation in RRC_INACTIVE, it is agreed in R2-121bis meeting that the multicast DRX is taken as baseline. However, handling of PTM related HARQ RTT timer and DRX Retransmission Timer is left to FFS.

Since the same PDCCH/PDSCH resource for RRC_CONNECTED UEs and RRC_INACTIVE UEs can be used, RRC_INACTIVE UE may receive the PTM retransmission caused by other UEs which may be in RRC_CONNECTED. It is same for RRC CONNECTED UEs whose HARQ feedback is disabled.

However, for the RRC CONNECTED UEs whose HARQ feedback is disabled, handling of PTM related HARQ RTT timer and DRX Retransmission Timer is not specified. In other words, PTM related HARQ RTT timer and DRX Retransmission Timer are not started for RRC CONNECTED UEs whose HARQ feedback is disabled. It is just assumed that those UEs may receive possible retransmission caused by other UEs without spec. impact.

Observation 2. PTM related HARQ RTT timer and DRX Retransmission Timer are not started for RRC CONNECTED UEs whose HARQ feedback is disabled.

We think that the same rule can be applied to the RRC_INACTIVE UEs. That is, even in RRC_INACTIVE, PTM related HARQ RTT timer and DRX Retransmission Timer are not running and disabled.

Proposal 5. PTM related HARQ RTT timer and DRX Retransmission Timer are not running in RRC_INACTIVE.

During RRC state transition, UE can continue receiving on-going multicast sessions at least if the same PDCCH/PDSCH resource are used for the multicast sessions in RRC_CONNECTED and RRC_INACTIVE. Also, in the same cell, we think that the gNB can schedule the multicast transmissions continuously regardless of UE’s RRC state.

For comparison, we first go through MBS broadcast. UE receives MBS broadcast services without suspension of MRB regardless of RRC state changes, and UE does not flush soft buffer for HARQ process being used for MBS broadcast during RRC state transition.

We think the same behavior can be applied to multicast reception. It can be considered that during RRC state transition between RRC_CONNECTED and RRC_INACTIVE, multicast MRB is not suspended and the soft buffer for HARQ process used for multicast reception in RRC_INACTIVE is not flushed.

Proposal 6. During RRC state transition between RRC_CONNECTED and RRC_INACTIVE, multicast MRB is not suspended and the soft buffer for HARQ process used for multicast reception in RRC_INACTIVE is not flushed.

During mobility, PDCP COUNT needs to be synchronized between the source cell and the target cell for multicast reception in RRC_INACTIVE. If PDCP COUNT is synchronized, PDCP re-establishment may not be needed during mobility. However, there is an issue that gNB and UE don’t know if PDCP COUNT is synchronized, and gNB and UE cannot determine if PDCP re-establishment is needed.

If it is assumed that PDCP COUNT is synchronized within RAN-based Notification Area (RNA) when multicast reception in RRC_INACTIVE is supported, a UE can know if PDCP COUNT is synchronized between the source cell and the target cell by checking whether the target cell belongs to the same RNA with the serving cell or not. If the target cell belongs the same RNA, PDCP COUNT is synchronized and the UE can determine not to re-establish PDCP. Otherwise, the UE can send an indication to the target cell to trigger PDCP re-establishment.

Therefore, we propose that PDCP COUNT is synchronized within a configured area, e.g. RAN-based Notification Area (RNA).

Proposal 7. PDCP COUNT is synchronized within RAN-based Notification Area (RNA) when multicast reception in RRC_INACTIVE is supported.

3.	Conclusion

This document discusses user plane related issues for multicast in RRC_INACTIVE state.

Proposal 1. Support MBS SPS without HARQ feedback in RRC_INACTIVE state.

Observation 1. SPS activation without HARQ feedback is already supported for MBS SPS in RRC_CONNECTED.

Proposal 2. Support L1 SPS activation without HARQ feedback for MBS SPS in RRC_INACTIVE.

Proposal 3. Support L1 SPS deactivation without HARQ feedback for MBS SPS in RRC_INACTIVE.

Proposal 4. Introduce group paging for SPS release in RRC_INACTIVE.

Observation 2. PTM related HARQ RTT timer and DRX Retransmission Timer are not started for RRC CONNECTED UEs whose HARQ feedback is disabled.

Proposal 5. PTM related HARQ RTT timer and DRX Retransmission Timer are not running in RRC_INACTIVE.

Proposal 6. During RRC state transition between RRC_CONNECTED and RRC_INACTIVE, multicast MRB is not suspended and the soft buffer for HARQ process used for multicast reception in RRC_INACTIVE is not flushed.

Proposal 7. PDCP COUNT is synchronized within RAN-based Notification Area (RNA) when multicast reception in RRC_INACTIVE is supported.