3GPP TSG-RAN WG2 Meeting #123	 R2-2308699

Toulouse, France, 21th – 25th Aug., 2023

Agenda Item:	7.15.2

Source: 	ITL

Title: 	Remaining issue on SL Consistent LBT failure

Document for:	Discussion and decision

Introduction

In RAN2#122 [1], the agreements related to this contribution are as follows:

Agreements on SL C-LBT failure recovery (mode 2, RRC ide/inactive UE)

1: 	Exclusion of RB set(s) that SL C-LBT failure was detected in candidate resource selection + resource pool (re)selection

2:	The UE performs resource pool (re)selection

-  When SL C-LBT failure was detected for all RB-sets within a selected resource pool or;

-  Up to UE implementation although the above condition is not met

3a:	MAC informs L1 of the RB set information where SL C-LBT failure was detected.

3b:	L1 performs the resource exclusion for the RB set that SL C-LBT failure was detected.

3c:	RAN2 will send a LS to RAN1 to ask to take it into consideration in their job.

4:	It is up to UE implementation to select a resource pool out of resource pools that has at least one RB-set that SL C-LBT failure was not detected.

Agreements on SL C-LBT failure recovery (mode 1)

1: 	Leave it to gNB implementation after UE reporting SL C-LBT failure indication. No spec change.

Agreements on SL C-LBT failure recovery (mode 2, RRC connected UE)

1: 	RAN2 confirms that SL C-LBT failure indication is reported to the gNB also for mode 2, RRC connected UE.

Agreements on SL C-LBT failure and S-SSB

1: 	Counting LBT failure indication regardless of whether LBT failure was provided because of S-SSB transmission or data transmission when RB set for S-SSB transmission belongs to the selected TX resource pool.

Agreements on SL C-LBT failure and PSFCH

1: 	Counting LBT failure indication regardless of whether LBT failure was provided because of PSFCH transmission or not when RB set for PSFCH transmission belongs to the selected TX resource pool. FFS when multiple PSFCH occasions are configured.

Agreements on SL C-LBT cancellation

1: 	For mode 1, SL C-LBT is cancelled upon SL C-LBT failure MAC CE transmission

In this contribution, we discuss sidelink un-licensed (SL-U) operation related to SL C-LBT.

Discussion

SL C-LBT failure and multiple PSFCH occasions

In RAN2#122, RAN2 has agreed to count LBT failure indication regardless of whether LBT failure was provided because of PSFCH transmission or not when RB set for PSFCH transmission belongs to the selected Tx resource pool. FFS was left for multiple PSFCH occasions. In this section, we show our opinion on SL C-LBT failure counting when multiple PSFCH occasions are configured.

In RAN1#113, there were agreements related to multiple PSFCH occasions for PSCCH/PSSCH as follows:

RAN1#113 Agreement Regarding one PSCCH/PSSCH transmission has N associated candidate PSFCH occasion(s) via (pre-)configuration: - Regarding locations of candidate PSFCH occasion(s): - Down-select at RAN1#114: - Alt 1 (15): Associated PSFCH occasion(s) are within the RB set(s) occupied by PSSCH transmissions - FFS details - Alt 2 (2): PSSCH transmission and its related PSFCH occasion(s) are in the same or different RB set(s) of the same resource pool - For one PSCCH/PSSCH transmission, at least support that its associated candidate PSFCH occasion(s) are in different slots of the same RB set(s) - FFS: whether to support its associated candidate PSFCH occasion(s) are in different RB sets of the same slot

Until now, multiple PSFCH occasions for a PSCCH/PSSCH can be (pre-)configured only on the time domain, i.e., candidate PSFCH occasions are in different slots of same RB set(s). Multiple PSFCH occasions on different RB sets of same slot still remains on FFS.

As discussed in RAN2#122 regarding SL C-LBT counting for S-SSB and PSFCH, it's important to note that SL C-LBT counting does not differentiate between channel accesses on specific channels. Similarly, even when multiple PSFCH occasions are configured, we do not perceive the need for any enhancement. Additionally, taking into account the ongoing discussions in RAN1 regarding the multiple PSFCH occasions on different RB sets of the same slot, we have the view that counting of LBT failure indication may not be affected because the granularity of SL C-LBT is per RB set.

Proposal 1: RAN2 to not pursue further enhancement related to counting LBT failure indication when multiple PSFCH occasions are configured

SL C-LBT cancellation for Mode 2

In RAN2#122, SL C-LBT cancellation method was discussed for each Mode 1/2 and RRC state. RAN2 has agreed “For mode 1, SL C-LBT is canceled upon SL C-LBT failure MAC CE transmission” which is quite straightforward since Mode 1 can rely on the network control. For the Mode 2, several options were discussed as follows:

- Option 1: Timer based cancellation

- Option 2: Measured channel condition based cancellation (FFS on what to be measured)

- Option 3: Rely on recovery action and no need for both option 1 and option 2

Initially, we would like to remind the recovery action for mode 2. In our understanding, if SL C-LBT failure is detected and there is at least one RB set where SL C-LBT was not detected in the selected resource pool, UE can perform either resource pool re-selection or candidate resource re-selection in the selected resource pool.  In both of cases, MAC may inform L1 of RB set information where SL C-LBT failure was detected to (re-)select resource(s) within the RB set(s) where SL C-LBT failure was not detected.

Agreements on SL C-LBT failure recovery (mode 2, RRC ide/inactive UE)

1: 	Exclusion of RB set(s) that SL C-LBT failure was detected in candidate resource selection + resource pool (re)selection

2:	The UE performs resource pool (re)selection

-  When SL C-LBT failure was detected for all RB-sets within a selected resource pool or;

-  Up to UE implementation although the above condition is not met

3a:	MAC informs L1 of the RB set information where SL C-LBT failure was detected.

3b:	L1 performs the resource exclusion for the RB set that SL C-LBT failure was detected.

3c:	RAN2 will send a LS to RAN1 to ask to take it into consideration in their job.

4:	It is up to UE implementation to select a resource pool out of resource pools that has at least one RB-set that SL C-LBT failure was not detected.

Figure 1. Example of SL C-LBT cancellation for mode 2

As mentioned above, it is possible to perform candidate resource re-selection in the selected resource pool without performing a resource pool re-selection. As an example, in Figure 1, if the selected resource pool is resource pool 0 and SL C-LBT is detected on RB set 1 at time T1, MAC informs L1 about the RB set information where SL C-LBT was detected. Subsequently, MAC could re-select the candidate resource(s) from the candidate resource set, which comprises resources from RB set 2, i.e., resources in RB set 1 are excluded from the candidate resource set by L1 at T1. RAN2 has not yet discussed and defined when resources in RB set 1 can be used again. Furthermore, at this time, to receive appropriate candidate resource set from the L1, MAC also needs to inform L1 of the RB set information where the SL C-LBT failure was canceled.

Observation 1: MAC needs specific timing to reuse resources within the RB set where SL C-LBT failure was detected

We think that RAN2 should consider Observation 1 when discussing the SL C-LBT cancellation for mode 2. Therefore, we present our view on each option taking into account Observation 1. Both Option 1 and Option 2 offer clear SL C-LBT failure cancellation timing, which can be considered as specific timing., i.e., right after timer expiration for Option 1 and right after condition is met for Option 2. However, in our understanding for the Option 3, the SL C-LBT failure cancellation timing can be right after the recovery action (resource pool re-selection or candidate resource re-selection), which is too short after detecting SL C-LBT. Therefore, in the case of Option 3, we believe there is a need to determine the specific timing based on UE implementation.

We prefer to introduce a clear approach (Option 1 and/or Option 2) to reuse resources within the RB set where SL C-LBT failure was detected. For example, in Figure 1, the timer for Option 1 starts at T1 when SL C-LBT is detected and ends at T2. For Option 2, a time at which the measured channel condition is met may be T2. At T2, MAC needs to inform L1 of the RB set information where SL C-LBT was canceled.

Proposal 2: Introduce Timer and/or measured condition based SL C-LBT cancellation for Mode 2

Proposal 3: RAN2 to discuss when to reuse resources within the RB set where SL C-LBT was detected

- Option 1/2: At the SL C-LBT failure cancellation

- Option 3: Up to UE implementation

Proposal 4: MAC informs L1 of the RB set information where SL C-LBT was canceled

Conclusion

In this contribution, we discussed sidelink operation in un-licensed. According to discussion in section 2, we have the following proposals:

Proposal 1: RAN2 to not pursue further enhancement related to counting LBT failure indication when multiple PSFCH occasions are configured

Proposal 2: Introduce Timer and/or measured condition based SL C-LBT cancellation for Mode 2

Proposal 3: RAN2 to discuss when to reuse resources within the RB set where SL C-LBT was detected

- Option 1/2: At the SL C-LBT failure cancellation

- Option 3: Up to UE implementation

Proposal 4: MAC informs L1 of the RB set information where SL C-LBT was canceled

Reference

RAN2#122 Meeting report