3GPP TSG-RAN WG2 Meeting #123	R2-2308702

Toulouse, France, 21 August – 25 August 2023              (revision of R2- 2306215)

Agenda item:	7.8.2

Source: 	Huawei, HiSilicon

Title: 	Measurement report enhancement for NR UAV

Document for:	Discussion and Decision

- Introduction

In RAN2 meeting #122, the following agreements have been reached regarding measurement report enhancement for NR UAVs:

Agreements

- Add height-based list of SSB-ToMeasure with corresponding height ranges and hysteresis in MeasObjectNR.  FFS on the number of height ranges

- As a basic principle, if no height-specific SSB-ToMeasure is configured for a specific height region, the legacy behaviour applies.

- For UE behavior on L1 and L3 measurement, it is left to UE implementation whether to keep/discard the old samples while UE moves to a new height region with a different SSB-ToMeasure value

- New event types will be introduced on the combination of event Ax and event Hx, at least for event A4 + event H1/H2. FFS for other event Ax + event H1/H2. FFS on details, e.g. whether to include one height threshold (H1 or H2 threshold) or a height range (both H1 and H2 threshold) in the new event, how to configure height-dependent numberOfTriggeringCells, etc.    This will be applied to all height dependent MR parameters.

- Whether UE height is included when UAV specific MR is triggered is configurable by the network.

- We will use LTE UEheight.

As we can see, there are some FFS that remain to be analyzed. This contribution is a revision of R2-2306215. Compared to the previous version we analyzed a few more technical aspects due to be resolved in RAN2 (i.e. FFSs).

- Discussion

As the UAV ascends, the radio environment will change dramatically when it crosses the roof level. This is because the NLoS path is dominant when the UAV is below the roof level, and the LoS path will be dominant gradually as the UAV is above the roof level. Besides, even with the UAV above roof level, the radio environment may also vary in different height ranges. This is because the signal received by the UAV is from the side lobes of the gNBs, and it varies in different height ranges (due to the radio signal being transmitted by different gNBs). Thus, we need to consider multiple height ranges. Furthermore, we can see that the altitude requirement for UAV applications in TS 22.125 [1] is up to 300m (see annex). When considering practical deployment situations, we think that 4 height ranges should be enough for the height-dependent configurations of NR UAVs.

Proposal 1: The maximum altitude for UAV applications in TS 22.125 is 300m, and four height ranges should be the maximum.

Besides, note that supporting 4 height ranges does not mean that the network should configure 4 height ranges for the UE every time. How many height ranges should be configured is up to network implementation.

Proposal 2: It is left to the network implementation the number of height ranges that can be configured for the UAV, i.e., the network does not need to always configure the maximum height ranges for the UE.

The combination of event A4 and event H1/H2 is proposed for handling vertical mobility [2]. To recap the vertical mobility issue, the signal strength of the serving cell will disappear suddenly without any symptom, e.g., the received signal strength of the UAV is not gradually decreasing, when the UAV is ascending or descending. At the same time, the neighbour cell can be detected by the UAV (the received signal of neighbour cell may be better or lower than that of the serving cell) but the UAV cannot handover to the neighbour cell (the serving cell is not lower than the threshold). Then, the handover failure will occur. Thus, we introduced the combination of event A4 and event H1/H2 to handle vertical mobility. Regarding the remaining Ax events, we will analyse them one by one. First, event A1 and A2 are not used for handover. Then, event A3 and event A6 require that the neighbour becomes offset better than SpCell and the neighbour becomes offset better than SCell, respectively. However, the received signal of the neighbouring cell may be lower than that of SpCell or Scell as above mentioned. Therefore, they cannot be combined with event H1/H2 to handle vertical mobility. Finally, event A5 requires that SpCell becomes worse than threshold1 and neighbour becomes better than threshold2, but the signal strength of SpCell may disappear suddenly, and the UAV cannot handover immediately. Thus, it also cannot be combined with event H1/H2 to handle vertical mobility.

Proposal 3: Beside for event A4, the combination of the other event Ax and event H1/H2 should not be introduced.

In the last meeting, it was agreed to introduce a new event type for event combinations. On FFS, the question is "whether to include one height threshold (H1 or H2 threshold) or a height range (both H1 and H2 threshold) in the new event".

At present, only events A4 and H1/H2 need to be considered. Regarding the new event, what should be included? First, the event A4 threshold should be included. Then, one height threshold (H1 or H2) should also be included. If the new event includes a height range (both H1 and H2 thresholds), the A4 threshold is equal for both the UAV UE ascending and UAV UE descending cases. However, the coverage of each cell is different.

For example, the cells traversed by the UAV are cells 1 and 2 when the UAV is descending, as shown in Fig. 1. The network will configure the height threshold (point A) of event H1 of cell 1 for the UAV descending case and the height threshold (point D) of event H2 of cell 2 for the UAV ascending case. In this case, if the height range (both H1 and H2 thresholds) is included in the new event, the event A4 threshold is the same for cell 1 and cell 2. Obviously, the event A4 threshold should be able to be configured differently at points D and A. Thus, the height range is not suitable for the new event. Besides, the hysteresis of event A4 and height should also be included in the new event.

Fig.1 illustration for UAV descending

Proposal 4: The Event A4 threshold with the corresponding hysteresis and the height threshold with the corresponding hysteresis should be included in the new event.

Regarding the height-dependent numberOfTriggeringCells, one FFS is how to configure the height-dependent numberOfTriggeringCells. We think the height-dependent numberOfTriggeringCells should be configured by the network, i.e., the network configures the correspondence between the numberOfTriggeringCells and different height ranges for the UAV. This is because numberOfTriggeringCells is introduced to control the number of measurement reports; it should be controlled by the network because the UE does not know the interference level of the whole system.

Specifically, we can define an ASN.1 sequence for numberOfTriggeringCells, which includes height range with the corresponding hysteresis and numberOfTriggeringCells. If the network wants to configure multiple numberOfTriggeringCells for different height ranges, the structure of ToAddModList can be used for multiple height-dependent numberOfTriggeringCells.

Proposal 5: Regarding the height-dependent numberOfTriggeringCells, height range with the corresponding hysteresis and numberOfTriggeringCells should be included in an ASN.1 sequence, and the structure of ToAddModList can be used for multiple height-dependent numberOfTriggeringCells.

One more issue needs to be clarified, i.e., if a height-dependent numberOfTriggeringCells is applied, how does the UE handle the old measurement results (e.g., the cell in cellTriggeredList) when the UE applies a new numberOfTriggeringCells when it reaches a new height range. This issue is also present in height-dependent SSB-ToMeasure, and it was agreed in the last meeting that how to handle the old samples is up to UE implementation.

However, the purpose of SSB-ToMeasure and numberOfTriggeringCells are different. The SSB-ToMeasure is used to configure a pattern of SSBs. Once the SSB-ToMeasure has changed, the UE will change the measured objective. The old samples may be useless if the measured objective is totally different. But the old samples may be partially useless if the measured objective changes partially. Thus, how to handle the old samples can be left to the UE implementation.

The numberOfTriggeringCells is used to indicate the number of cells detected that are required to fulfil an event for a measurement report to be triggered. Once the numberOfTriggeringCells is changed, the old measured cells are still useful for the network. If the UE discards the old measured cells, the late measurement report will be a more serious issue. Thus, RAN2 should consider that the UE should keep the old measurement results (e.g., the cell in cellTriggeredList) when the UE moves to a new height region with a different numberOfTriggeringCells value.

Proposal 6: RAN2 should consider that the UE keeps the old measurement results (e.g., the cell in cellTriggeredList) when the UE moves to a new height region with a different numberOfTriggeringCells value.

- Conclusion

Proposal 1: The maximum altitude for UAV applications in TS 22.125 is 300m, and four height ranges should be the maximum.

Proposal 2: It is left to the network implementation the number of height ranges that can be configured for the UAV, i.e., the network does not need to always configure the maximum height ranges for the UE.

Proposal 3: Beside for event A4, the combination of the other event Ax and event H1/H2 should not be introduced.

Proposal 4: The Event A4 threshold with the corresponding hysteresis and the height threshold with the corresponding hysteresis should be included in the new event.

Proposal 5: Regarding the height-dependent numberOfTriggeringCells, height range with the corresponding hysteresis and numberOfTriggeringCells should be included in an ASN.1 sequence, and the structure of ToAddModList can be used for multiple height-dependent numberOfTriggeringCells.

Proposal 6: RAN2 should consider that the UE keeps the old measurement results (e.g., the cell in cellTriggeredList) when the UE moves to a new height region with a different numberOfTriggeringCells value.

- Reference

[1] 22.125 “Uncrewed Aerial System (UAS) support in 3GPP”; Stage 1;

[2] R2- 2212638 “Further discussion on UAV measurement enhancements”, Huawei, HiSilicon

- Annex

Table 7.1-1 KPIs for services provided to the UAV applications

| Use case | Services | Data rate | End to end Latency | Altitude AGL | service area (note 4) |
| --- | --- | --- | --- | --- | --- |
| 1 | 8K video live broadcast | 100Mbps UAV originated | 200 ms | <100 m | Urban, scenic area |
|  |  | 600Kbps UAV terminated | 20 ms | <100 m |  |
| 2 | Laser mapping/ HD patrol Note 7 | 120Mbps UAV originated Note 1 | 200 ms | 30-300 m | Urban, rural area, scenic area |
|  |  | 300Kbps UAV terminated | 20 ms | 30-300 m |  |
| 3 | 4*4K AI surveillance | 120Mbps UAV originated | 20 ms | <200 m | Urban, rural area |
|  |  | 50Mbps UAV terminated | 20 ms | <200 m |  |
| 4 | Remote UAV controller through HD video | >=25Mbps UAV originated (Note 3) | 100 ms | <300 m | Urban, rural area |
|  |  | 300Kbps UAV terminated | 20 ms | <300 m |  |
| 5 | Real-Time Video | 0.06 Mbps w/o video UAV originated | 100 ms | - | Urban, rural, countryside |
| 6 | Video streaming | 4 Mbps for 720p video 9 Mbps for 1080p video UAV originated | 100 ms | - | Urban, rural, countryside |
| 7 | Periodic still photos | 1Mbps UAV originated; 1S; 0.1m; <120m; -; Urban, rural area. | 1s | <120 m | Urban, rural, countryside |
| NOTE 1: The flight average speed is 60km/h. The KPI is referring to [5]. NOTE 2: The latency is the time of the 5G system provide higher accuracy location information of a UAV to a third party. NOTE 3: Referring to clause 5.2.2, the absolute flying speed of UAV in this service can be up to 160km/h. NOTE 4: The density of active UAV is 10/200km2. The maximum altitude is 300m. The flight average speed is 60km/h. |  |  |  |  |  |