3GPP TSG-RAN WG2 Meeting #123		R2-2308789

Toulouse, Aug 21 – 25, 2023

Agenda Item:	7.17.2

Source: 	LG Electronics

Title:  	Timer based approach in MUSIM

Document for:	Discussion

Introduction

According to the last RAN2meeting, we have following agreement for the temporary capability restriction [1]:

1: For Rel-18 MUSIM dual active operation, the maximum MIMO layer may be changed and the change can be indicated to the NW. FFS if this is only for NW A or also NW B. 3: For Rel-18 MUSIM dual active operation, the measurement gap requirement may be changed and the change can be indicated to the NW. FFS if this is only for NW A or also NW B. 4: For Rel-18 MUSIM dual active operation, the measurement gap requirement change is reported for each serving cells, and for target bands or all supported NR bands depending on whether target bands are configured by the NW. FFS on whether the reporting can reuse the current needForGapInfoNR in RRC reconfiguration complete or extend the similar function in UAI. FFS if this is only for NW A or also NW B. 8: The maximum UL power may be changed due to Rel-18 MUSIM dual active operation, but there is no need to introduce any new UE behavior for reporting this change. 6: UE can explicitly request specific serving cells or serving cell group to be released for Rel-18 MUSIM purpose. FFS how/whether this works for the proactive case.

In this contribution, we would like to discuss a timer-based approach for both the reactive and the proactive cases in MUSIM.

Discussion

If capability conflict happens while Rel-18 Multi-SIM operation, it means that both NET A and NET B are in trouble with the conflict. Due to this, The UE may experience service interruptions caused in both networks that data communication cannot be successfully transmitted and received on both sides.

In particular, the serious point is that the UE has suffered from this problem of data communication until one of the networks provides an RRC reconfiguration message to restrict the conflict. For example, let us assume a case that a frequency conflict may happen between NET A and NET B. If PCell where the UE wants to request a temporary capability restriction is busy, the UE may not receive the restriction from the network quickly. In this case, the UE just waits only for a reconfiguration message for the restriction from the network and continues to suffer service interruption.

There was a similar issue in Rel-17 Multi-SIM WI. When the UE needs to perform a SIM leaving procedure from NET A to NET B, the service interruption of NET B can last long until NET A provides a response to the SIM leaving request received from the UE. In particular, if the UE in NET A misses the RRC Release message, this interruption time becomes very long.

To minimize this service interruption in Rel-17 Multi-SIM WI, a wait timer has been introduced. The wait timer starts when the UE requests a SIM leaving procedure, and upon the expiry of the wait timer, it is possible to perform UE-based SIM leaving without a response from the network.

We think that a similar timer concept can be applied for temporary capability restriction in Rel-18 Multi-SIM WI. The network optionally provides a wait timer to the UE in advance. When a capability conflict is detected, the UE starts the wait timer while notifying the network of the expected response for a temporary capability restriction. If RRC Reconfiguration message to resolve the capability conflict is not received from the network until the timer expires, the UE temporarily restricts the UE capability as the expected response notified before.

Proposal 1.	A timer is introduced to allow the UE to perform the temporary capability restriction w/o the network response when the timer, which should start upon request of the restriction, expires.

Furthermore, if the network allows, we think the timer-based approach can also be applied for the proactive cases. In other words, if the network configures the timer to the UE and the network supports the UE's restriction request in the proactive cases, the UE can perform the restrictions more quickly using the timer-based approach even in the proactive case.

Proposal 2.	The UE can utilize the timer to support temporary capability restrictions in the proactive cases if the network allows.

Conclusion

We propose to discuss and decide on the following proposals:

Proposal 1.	A timer is introduced to allow the UE to perform the temporary capability restriction w/o the network response when the timer, which should start upon request of the restriction, expires.

Proposal 2.	The UE can utilize the timer to support temporary capability restrictions in the proactive cases if the network allows.

Reference

- R2-2304202 Report on LTE legacy, XR, QoE and MUSIM, Nokia