3GPP TSG-RAN WG2 Meeting #123											   R2-2308822

Toulouse, France, 21-25 Aug, 2023

Source: 	Xiaomi

Title:  	Discussion on flight path reporting for NR UAV

Agenda Item:	7.8.3

Document for:	Discussion and Decision

Introduction

A revised WID on NR support for UAV (Uncrewed Aerial Vehicles) [1] has been approved. In the WID, one objective is to enhance measurement reports, including flight path reporting.

In previous meetings [2-7], the following agreements have been reached for flight path reporting for NR UAV:

RAN2#119-e: 3 As in LTE, flight path plan reporting will be introduced. Location list of waypoints (3D location information) and timestamp is adopted as the basic content of flight path report. FFS if timestamp is mandatory or optional for NR. FFS if further enhancements are needed RAN2#119bis-e: 1. The time information reported as part of flight path plan is optional. UE includes time info, if configured by the network and available at the UE. FFS on flight path details (waypoints and what is time information). 2. Allow the flight path to be updated. FFS on the details. 3. FFS on reporting format and initial flight path reporting (i.e. what information to report and how) – next meeting RAN2#120: 1. A waypoint is a planned location for the UE along the flight path and is described via the existing parameter type LocationCoordinates defined in TS 37.355. 2. A timestamp provides the UTC time associated with estimated time of arrival to a waypoint as baseline. FFS on granularity 3. No requirements are placed on spatial distribution of waypoints 4. A UE indicates whether flight plan information is available within the RRCReconfigurationComplete, RRCReestablishmentComplete, RRCResumeComplete, or RRCSetupComplete message. Flight path reporting uses at the UE Information request/response procedure as baseline. 5. UE indicates to the network a new flight path is available in the UE (whether it is initial or update). Then, reuse the normal request/response procedure of flight path report. 6. UAI message can also be used to indicate the UE has flight path availability. 7. FFS whether and what triggering conditions are specified for flight update. FFS The maximum number of waypoints within flight path plan is left FFS. RAN2#121: 1. The granularity of flightpath timestamp is 1s. 2. Timestamp in flightpath is encoded using AbsoluteTimeInfo-r16 IE RAN2#121bis-e: 1. Flightpath update indication in UAI is configurable by the network 2. Maximum number of waypoints is set to 20 same as in LTE and number of waypoints is configurable by network as in LTE 3. Flightpath information should be forwarded from source gNB to target gNB during handover. Send LS to RAN3 to check for feasibility [LS to RAN3 over email 307] 4. As a baseline, we can consider a simple network control mechanisms (e.g. a threshold(s)) that controls triggering the flightpath update indication in UAI. FFS if new threshold or the kind of threshold(s) 5. As a baseline, single indication is used for both initial and updated flightpath available (i.e. same flag is used for initial and updated flight path indication. FFS if further differentiation is needed if we decide to have delta signaling RAN2#122: 1. The network can configure the UE to trigger a flightpath update notification based on a configured delta time (when timestamp is configured to be reported) or distance configuration.

In this contribution, we will provide our understandings on the flight path reporting for NR UAV.

Discussion

  - The content of flight path information

1) Which waypoints can be included in flight path information

The meanings of waypoint and timestamp has reached consensus in previous meetings. However, which waypoints should be included in the flight path information is FFS. Some options have been provided in previous meetings, e.g. the flight path information follows the granularity of the waypoints (i.e. the distance or interval between consecutive waypoints) configured by network and/or provides the information on the destination of the flight. However, it is uncertain whether UE has full flight path information or not e.g. whether the flight path information stored in UE includes the destination of flight and can satisfy the granularity configured by network. And the size and speed of UAV UE may influence over the granularity of waypoint. Hence, whether the requirement of flight path information configured by network can be fulfilled or not depend on the flight path information in UE. It may be difficult to configured appropriate requirements to UE to limit the content of flight path information. And, in LTE UAV, which waypoints are included in flight path information is up to UE implementation. Therefore, no optimization for the content of flight path information is needed for NR UAV.

Proposal 1: It is up to UE implementation which waypoints are included in flight path information.

2) The definition of UAV flight path information

In LTE UAV, the flightPathInfoReport IE has been defined to include the flight path Information reported to the network.

FlightPathInfoReport-r15 ::=		SEQUENCE {

flightPath-r15	SEQUENCE (SIZE (1..maxWayPoint-r15)) OF WayPointLocation-r15	OPTIONAL,

dummy							SEQUENCE {}							OPTIONAL

}

WayPointLocation-r15 ::=			SEQUENCE {

wayPointLocation-r15						LocationInfo-r10,

timeStamp-r15							AbsoluteTimeInfo-r10		OPTIONAL

}

For NR UAV, RAN2 has agreed the following agreements for the ASN.1 structure of flight path information:

- A waypoint is a planned location for the UE along the flight path and is described via the existing parameter type LocationCoordinates defined in TS 37.355.

- Timestamp in flightpath is encoded using AbsoluteTimeInfo-r16 IE

Based on the legacy ASN.1 structure of LTE and agreements in R18 NR UAV, the example of ASN.1 for NR UAV shown as below can be considered:

FlightPathInfoReport-r18 ::=		SEQUENCE {

flightPath-r18	SEQUENCE (SIZE (1..maxWayPoint-r18)) OF WayPointLocation-r18	       OPTIONAL,

dummy							SEQUENCE {}							OPTIONAL

}

WayPointLocation-r18 ::=			SEQUENCE {

locationCoordinate-r18					OCTET STRING,

timeStamp-r18							AbsoluteTimeInfo-r16		            OPTIONAL

}

locationCoordinate: Parameter type LocationCoordinates defined in TS 37.355. The first/leftmost bit of the first octet contains the most significant bit.

Proposal 2: An IE FlightPathInfoReport-r18 similar to LTE FlightPathInfoReport-r15 can be introduced in UEInformationResponse message for the definition of UAV flight path information and the above ASN.1 example can be considered.

In RAN2#121bis-e, RAN2 agreed to forward flightpath information from source gNB to target gNB during handover. And Send LS to RAN3 to check for feasibility. And in the reply LS [8], RAN3 agreed adding the UAV flight path information into RAN3 specifications for NGAP (TS 38.413), and XNAP (TS 38.423). The TPs [9-10] provided by RAN3 intentionally uses the referred IE to RRC (FlightPathInfoReport IE as defined in TS 38.331).

Because RAN2 has not finalized the definition of UAV fight path information. The TP from RAN3 is FFS. When RAN2 defines the UAV fight path information or makes some progress on UAV flight path information, RAN2 can send a Reply LS to RAN3.

Proposal 3: RAN2 can send the reply LS to RAN3 including the definition of UAV flight path information.

  - Flight path reporting

For NR UAV, UE can indicate the UE has a new flight path availability (whether it is initial or update). And a single indication is used for both initial and updated flightpath available (i.e. same flag is used for initial and updated flight path indication). In LTE, flightPathInfoAvailable is used for the indication of flight path available. To align with legacy principle, LTE flightPathInfoAvailable can be reused for NR UAV to indicate initial and updated flight path.

Proposal 4: LTE flightPathInfoAvailable can be reused for initial and updated flight path indication in NR UAV.

1) The indication of initial flight path via UAI

In RAN2#120 meeting, it has been agreed that flight path availability can be reported within the RRCReconfigurationComplete, RRCReestablishmentComplete, RRCResumeComplete, or RRCSetupComplete message. Upon reception of a corresponding RRC message, if the UE has flight path information available, UE shall including flightPathInfoAvailable in the associated complete message. However, if UE doesn’t have flight path information available upon reception of the corresponding RRC message, UE will not report the flag of flight path availability and then if UE has flight path information available after the transmission of RRC complete message, UE will not report flightPathInfoAvailable unless UE receives a new RRC message e.g. RRCReconfiguration, RRCReestablishment, RRCResume, or RRCSetup, which will cause that network cannot obtain flight path information timely.

According to above analysis, if the initial indication can only be reported via complete message, the network cannot obtain flight path information in time. In RAN2#120 meeting, RAN2 has agreed that UAI message can be used to indicate the UE has flight path availability.

In addition to RRC Complete messages, UAI message can also be used to report the indication of initial flight path availability. If UE has flight path information available and the UE has not reported the indication in the latest complete message, the UE should report the initial flight path available indication (i.e. flightPathInfoAvailable) via UAI.

Proposal 5：UAI message can be used to report the indication of initial flight path availability. If UE has flight path information available and the UE has not reported the indication in the latest complete message, the UE should report the initial flight path available indication (i.e. flightPathInfoAvailable) via UAI.

2) The indication of update flight path via UAI

In NR UAV, the network configured flightpath update indication in UAV has been supported and the following cases for flight path update can be considered:

- Case 1: Compared with the old flight path, one or more new waypoints are added in the new flight path.

Table 1: an example for case 1

| Old flight path information | New flight path information |
| --- | --- |
| Location 1, t1 | Location 1, t1 |
| Location 2, t2 | Location 2, t2 |
| Location 3, t3 | Location 3, t3 |
|  | Location 4, t4 |

For Case 1, the update of flight path info can be triggered based on the network implementation. For the example shown in Table 1, if current time is larger than or close to t3, network can request UE to report the new flight path and UE triggered flightpath update notification is not needed for the case.

- Case 2: Compared with the old flight path, the timestamps for included waypoints are changed in new flight path.

Table 2: an example for case 2

| Old flight path information | New flight path information |
| --- | --- |
| Location 1, t1 | Location 1, t1 |
| Location 2, t2 | Location 2, t2’ |
| Location 3, t3 | Location 3, t3’ |
| Location 4, t4 | Location 4, t4’ |

For case 2, some waypoints in old flight path become invalid but network cannot know the change of flight path. Hence, for case 2, UE triggered flightpath update notification is needed.

- Case 3: Compared with the old flight path, the locations for included waypoints are changed in new flight path.

Table 3: an example for case 3

| Old flight path information | New flight path information |
| --- | --- |
| Location 1, t1 | Location 1, t1 |
| Location 2, t2 | Location 2’, t2 |
| Location 3, t3 | Location 3’, t3 |
| Location 4, t4 | Location 4, t4 |

For case 3, some waypoints in old flight path become invalid but network cannot know the change of flight path. Similar to case 2, UE triggered flightpath update notification is needed.

For UE triggered flightpath update notification, RAN2 has supported the network can configure the UE to trigger a flightpath update notification based on a configured delta time (when timestamp is configured to be reported) or distance configuration. For case 2 and case 3, UE can trigger flightpath update notification based on the delta time or distance configuration separately. How to trigger a flightpath update notification based on a configured delta time or distance configuration should be discussed and the following options can be considered.

- Option 1: For the delta time configuration based flightpath update notification, when the timestamp of one included waypoint has changed more than the configurable delta time, UE triggers a flightpath update notification.

  - For example, when |t2’-t2|> the configured delta time, UE triggers a flightpath update notification.

- Option 2: For the distance configuration based flightpath update notification, when the location of one included waypoint has changed more than the configurable distance, UE triggers a flightpath update notification.

  - For example, when the distance between location 2 and location 2’> the configured distance, UE triggers a flightpath update notification.

Proposal 6: The following options can be considered for flightpath update notification:

- Option 1: For the delta time configuration based flightpath update notification, when the timestamp of one included waypoint has changed more than the configurable delta time, UE triggers a flightpath update notification.

- Option 2: For the distance configuration based flightpath update notification, when the location of one included waypoint has changed more than the configurable distance, UE triggers a flightpath update notification.

Conclusions

According to the analysis given above, we have the following proposals:

The content of flight path information

Proposal 1: It is up to UE implementation which waypoints are included in flight path information.

Proposal 2: An IE FlightPathInfoReport-r18 similar to LTE FlightPathInfoReport-r15 can be introduced in UEInformationResponse message for the definition of UAV flight path information and the above ASN.1 example can be considered.

Proposal 3: RAN2 can send the reply LS to RAN3 including the definition of UAV flight path information.

Flight path reporting

Proposal 4: LTE flightPathInfoAvailable can be reused for initial and updated flight path indication in NR UAV.

Proposal 5：UAI message can be used to report the indication of initial flight path availability. If UE has flight path information available and the UE has not reported the indication in the latest complete message, the UE should report the initial flight path available indication (i.e. flightPathInfoAvailable) via UAI.

Proposal 6: The following options can be considered for flightpath update notification:

- Option 1: For the delta time configuration based flightpath update notification, when the timestamp of one included waypoint has changed more than the configurable delta time, UE triggers a flightpath update notification.

- Option 2: For the distance configuration based flightpath update notification, when the location of one included waypoint has changed more than the configurable distance, UE triggers a flightpath update notification.

Reference

[1] RP-230782, Nokia, “Revised WID: NR Support for UAV (Uncrewed Aerial Vehicles)”

[2] R2-2209301, “Report of 3GPP TSG RAN WG2 meeting #119-e”

[3] R2-2211101, “Report of 3GPP TSG RAN WG2 meeting #119bis-e”

[4] R2-2300002, “Report of 3GPP TSG RAN WG2 meeting #120”

[5] R2-2302401, “Report of 3GPP TSG RAN WG2 meeting #121”

[6] R2-2306553, “Report of 3GPP TSG RAN WG2 meeting #121bis-e”

[7] R2-xxxxxx, “Report of 3GPP TSG RAN WG2 meeting #122”

[8] R3-233493, “Reply LS to RAN2 on flightpath information forwarding for UAV”

[9] R3-233467, “Text Proposal on XnAP BL CR: Flight Path in UAV”

[10] R3-233492, “(TP to 38.413) Support flightpath information forwarding for UAV”