3GPP TSG-RAN WG2 Meeting #123                 	R2-2308832

Toulouse, FR, 21th – 25th Aug, 2023

Agenda Item:	7.8.2

Source: 	vivo

Title:         	Discussion on measurement reporting enhancement for NR UAV

Document for: 	Discussion and Decision

- Introduction

So far, RAN2 has made the following agreements for measurement reporting enhancement for NR UAV:

Agreements

- Use LTE principle as a baseline, introduce similar event H1 (aerial UE height become higher than threshold) and H2 (aerial UE height become lower than threshold.  FFS if further NR enhancements are needed.  FFS study scaling of RRM parameters (e.g. which parameters and what is the purpose/benefit of the scaling and how)

FFS how to limit excessive measurements and measurement reporting

FFS if user consent is needed for location reporting in CONNECTED

FFS study the vertical movement and associated mobility for UAV UEs

2	Rel-18 NR supports reporting of UAV UE’s height, location and velocity. It is for further study what accuracy and reporting mechanisms are required and if further enhancements are needed.

3	As in LTE, flight path plan reporting will be introduced.  Location list of waypoints (3D location information) and timestamp is adopted as the basic content of flight path report.  FFS if timestamp is mandatory or optional for NR.  FFS if further enhancements are needed

4	Introduce similar functionality to LTE (numberofTriggeringCells).  FFS whether numberoftriggerbeams for NR is required or other enhancements.  FFS study how to avoid sending the measurement reports mainly due to reportOnLeave.

- The time information reported as part of flight path plan is optional. UE includes time info, if configured by the network and available at the UE.  FFS on flight path details (waypoints and what is time information).

- Allow the flight path to be updated.  FFS on the details.

- FFS on reporting format and initial flight path reporting (i.e. what information to report and how) – next meeting

- Continue to study height-depending scaling, triggering and combinations

- As in LTE, as a baseline, events A3, A4 and A5 can be configured with the configured number of cells (numberofTriggeringCells)

- When event H1 or H2 triggers, the content of the measurement report is configurable by the network (i.e. it can contain UAV UEs height, location information and/or RSRP/RSRQ measurement results). FFS whether UAV UE’s height is mandatorily reported and which parameter/IE is used for height reporting.

- Joint use of height-dependent condition and RSRP/RSRQ/SINR-based condition for measurement report triggering is supported in NR Rel-18 UAV.  The combination of existing events will be used

- Height-dependent parameter scaling is not supported as a part of Rel-18 NR

- Do not extend the Number of triggering cells mechanism to apply to the inter-RAT scenario, i.e. event B1 and B2 triggering

- Do not restrict the applicability of Number of triggering cells mechanism to FR1 only. In other words, the Number of triggering cells mechanism is applicable to FR1 and FR2 (up to network configuration).

- The UE shall not ignore or bypass the Number of triggering cells mechanism, once configured.

- Do not introduce the use of a “numberOfTriggeringBeams” mechanism.

- Do not introduce an alternative mechanism to the Number of triggering cells mechanism.

- Do not introduce an additional mechanism based on Number of changed cells.

- For the purpose of interference control (i.e. for number of trigger cells), do not introduce a prohibit timer mechanism.

- Report on leave is not triggered by a cell that was not previously included in the measurement report for the number of triggering cell.

- Support configuring height-dependent more-than-one configurations targeting measurement and measurement reporting enhancement. UE applies corresponding configuration based on the UE height. The proposed solutions should aim at avoiding RAN4 impacts.  FFS how this would be configured (i.e. different MO configurations or different parameters  FFS Exact parameters and details.

- Height-dependent more-than-one configurations is supported on parameter/field level (i.e. different fields/values within the same MO) where different values (or value ranges) of the parameter/field applies to different height or height range.

- For MO configuration parameters: at least the following will have ability to be configured with height-dependent more-than-one configurations/values, each for a specific height region: SSB-ToMeasure. Details on how to specify is FFS.    FFS on UE behavior on L1 and L3 measurement.

- For MR configuration parameters: at least the following will have ability to be configured with height-dependent more-than-one configurations/values, each for a specific height region: Event A4 threshold and numberoftriggeringcells.  Details on how to specify is FFS (i.e. maybe it can be achieved by combination of events).

- When height-dependent more-than-one configurations are provided, UE applies the new value once it moves to new height (or height range) similar to the case of RRC reconfiguration. Need Codes, field descriptions, etc. as in legacy specifications apply

- If a height-specific value is not explicitly configured for certain height, whether to keep using the value that was used or consider the parameter as released (i.e. parameter/value not applicable at this height) should be looked into case by case, and can be clarified by need code, field description, or procedural text as needed.   FFS details

- Add height-based list of SSB-ToMeasure with corresponding height ranges and hysteresis in MeasObjectNR.  FFS on the number of height ranges

- As a basic principle, if no height-specific SSB-ToMeasure is configured for a specific height region, the legacy behaviour applies.

- For UE behavior on L1 and L3 measurement, it is left to UE implementation whether to keep/discard the old samples while UE moves to a new height region with a different SSB-ToMeasure value

- New event types will be introduced on the combination of event Ax and event Hx, at least for event A4 + event H1/H2. FFS for other event Ax + event H1/H2. FFS on details, e.g. whether to include one height threshold (H1 or H2 threshold) or a height range (both H1 and H2 threshold) in the new event, how to configure height-dependent numberOfTriggeringCells, etc.    This will be applied to all height dependent MR parameters.

- Whether UE height is included when UAV specific MR is triggered is configurable by the network.

- We will use LTE UEheight.

This contribution further discusses the remaining issues for measurement reporting enhancement for NR UAV.

- Discussion

Height reporting content:

CommonLocationInfo IE contains gnss-TOD-msec, locationCoordinate, velocityEstimate, locationTimestamp, locationError and locationSource.

- gnss-TOD-msec: This field specifies the GNSS TOD for which the measurements and/or location estimate are valid.

- locationCoordinate: This field provides a location estimate using one of the geographic shapes defined in TS 23.032 [15].

- velocityEstimate: This field provides a velocity estimate using one of the velocity shapes defined in TS 23.032 [15].

- locationTimestamp: This field provides the UTC time when the location estimate is valid.

- locationError: The field includes information concerning the reason for the lack of location information. For example, periodicLocationMeasurementsNotAvailable.

- locationSource: This field provides the source positioning technology for the location estimate, for example, a-gnss, wlan, TBS, etc.

In LTE UAV, gnss-TOD-msec, locationCoordinate, velocityEstimate are reported, and we think it can be the same in NR UAV. From our understanding, locationTimestamp, locationSource, locationError could be also useful for the network. These fields are all optional, so we see no reason not to include these in the MR if these are available at the UE. Currently, the RSRP results of neighboring cells and CommonLocationInfo are configurable in NR. So, the current specification already supports this. Thus, we have the following proposal:

Observation 1: When event H1 or H2 triggers, all the parameters in existing IE CommonLocationInfo can be included in the measurement report.

Observation 2: Both RSRP results of neighboring cells and CommonLocationInfo are configurable in current NR specification.

Proposal 1: When event H1 or H2 triggers, both RSRP results and CommonLocationInfo reporting are configurable. No spec change is needed.

Hight-dependent MO configuration:

In the RAN2#122 meeting, the below agreements were made:

Agreements

- Add height-based list of SSB-ToMeasure with corresponding height ranges and hysteresis in MeasObjectNR.  FFS on the number of height ranges

- As a basic principle, if no height-specific SSB-ToMeasure is configured for a specific height region, the legacy behaviour applies.

- For UE behavior on L1 and L3 measurement, it is left to UE implementation whether to keep/discard the old samples while UE moves to a new height region with a different SSB-ToMeasure value

From our understanding, the network will use partial beams for serving the ground UEs and use partial beams for UAV UEs. So, at least two height ranges are needed. For different height level, it is uncertain to us how the network can configure it in reality. And regarding how to configure height range, it can include the upper bound and/or lower bound of the height range.

Proposal 2: Two height ranges are introduced for height-dependent MO configurations.

In the current measurement configuration procedure, the network can modify the configuration for each MO and MR. And upon one MO or MR is reconfigured, the UE shall remove the measurement reporting entry for the related measId from the VarMeasReportList and stop the periodical reporting timer or timer T321 or timer T322, whichever one is running, and reset the associated information (e.g. timeToTrigger) for this measId. Now we are going to introduce something like conditional modification of MO. From our understanding, taking MO reconfiguration as an example, the steps at the UE can be the following:

- In Measurement object addition/modification procedure (section 5.5.2.5), if receiving the introduced ToAddModList (e.g., ConditionalParaToAddModList), the UE stores the list and starts to evaluate whether the height region is reached.

- In a separate section, how the UE evaluates the height region condition will be specified. Upon the UE’s height satisfies the height region, the UE goes to Measurement object addition/modification procedure (section 5.5.2.5), in which the UE will reconfigure the SSB-ToMeasure.

- After applying the new configuration, the UE shall remove the measurement reporting entry for the related measId from the VarMeasReportList and stop the periodical reporting timer or timer T321 or timer T322, whichever one is running, and reset the associated information (e.g. timeToTrigger) for this measId, as legacy. Then, the UE restarts to evaluate the height region condition.

Alternatively, if companies have concern on the spec change, RAN2 can discuss how to use height-dependent MO configurations is described in the field description. Thus, the below proposal is given:

Proposal 3: To support multiple MO configuration each for a specific height region, RAN2 to discuss the below options:

- Option 1: how to apply height-dependent MO configurations is based on conditional MO configuration modification.

- Option 2: how to apply height-dependent MO configurations is described in the field description.

- Conclusion

In this paper, the following proposal are given:

Proposal 1: When event H1 or H2 triggers, both RSRP results and CommonLocationInfo reporting are configurable. No spec change is needed.

Proposal 2: Two height ranges are introduced for height-dependent MO configurations.

Proposal 3: To support multiple MO configuration each for a specific height region, RAN2 to discuss the below options:

- Option 1: how to apply height-dependent MO configurations is based on conditional MO configuration modification.

- Option 2: how to apply height-dependent MO configurations is described in the field description.

- References

[1] RP-213600, New WID on NR support for UAV (Uncrewed Aerial Vehicles), Nokia.