3GPP TSG-RAN WG2 Meeting #123	R2-2308862

Toulouse, France 21st – 25th  Aug, 2023

Agenda Item:	7.25.1

Source:	Huawei, HiSilicon

Title:	Discussion on maximum aggregated bandwidth

Document for:   Discussion and decision

Introduction

In RAN2#122 meeting, the aggregated bandwidth capability was discussed with following agreement.

- Working Assumption that we go with P1-P6 in R2-2304856, under the condition that BW compatibility is supported. Next meeting describe how BW compatibility can work and confirmation of WA (or the opposite if serious issues are found)

In this contribution, we analyse the backward compatibility of the aggregated bandwidth capability, and provide our views on the signaling design.

Discussion

Background

In RNA4 LS[1], it was identified that according to current spec, when the UE supports different component carrier(CC) bandwidths for a band combination, multiple feature sets will be reported to indicate these CC bandwidth combinations which could not be considered as fall-back from each other (i.e. option1 in RAN4 LS). To avoid the signaling overhead, option2 was provided to introduce a new capability indicating the maximum aggregated bandwidth that UE supports for a band combination. The aggregated bandwidth is for FR1 inter-band CA band combinations for BCS4/5.

RAN2 was asked to consider the signalling overhead reduction, backwards compatibility, inter-operation and spec impacts for the option2. In this contribution, we discuss the above issues for the aggregated bandwidth for FR1 inter-band CA.

Signaling overhead

According to the example in RAN4 LS [1], for BCS4/5, the option2 is beneficial to signalling overhead when the supported total aggregated bandwidth across the CCs in a band combination is lower than the theoretical maximum aggregated bandwidth based on the maximum channel bandwidth on corresponding bands. For example, for CA_n25A-n41A-n66A, the maximum channel bandwidth for n25, n41 and n66 are 45M, 100M and 45M respectively, then the maximum aggregated bandwidth for this band combination is up to 190MHz in theory. If the UE is able to support 190M totally, the supported CC bandwidth can be set the same as the maximum channel bandwidth. There is no additional signalling overhead. Only when the supported aggregated bandwidth is lower than 190MHz, the signalling overhead exists.

This issue is not specific to BCS4/5, but common for all BCSs. Even for BCS#0, when the supported aggregated bandwidth is lower than the maximum aggregated bandwidth defined by BCS#0 (i.e. 160MHz), multiple FeatureSets are needed to report all possible bandwidth combinations.

Table-1  Bandwidth combination sets for CA_n25A-n41A-n66A

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n25A-n41A-n66A | - | n25 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
|  | CA_n25A-n41A CA_n25A-n66A CA_n41A-n66A | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  | CA_n25A-n41A CA_n25A-n66A CA_n41A-n66A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |

Observation 1: The signalling overhead exists when the supported aggregated bandwidth is lower than the maximum aggregated bandwidth defined by corresponding BCS.

Backward compatibility

In last RAN2 meeting, the backward compatibility of the aggregated bandwidth capability was discussed. In the option2 provided by RAN4, to avoid signalling overhead of multiple FeatureSets, the aggregated bandwidth is used together with the over-reported CC bandwidth. In this way, different fallback CC bandwidth combinations could be derived following the current fallback principle. In the example in [1], the over-reported CC bandwidth could be set as following, then only one FeatureSet is enough.

Table-2 Example of over-reported CC bandwidth for CA_n25A-n41A-n66A

| # | n25A CC BW (mHz) | n41A CC BW (mHz) | n66A CC BW (mHz) | Aggregated BW (mHz) |
| --- | --- | --- | --- | --- |
| 1 | 45 | 100 | 45 | 120 |

However, when the legacy NW cannot identify the aggregated bandwidth capability, the configured CC bandwidth may exceed actual UE capability. To avoid inter-operability issue, the aggregated bandwidth is proposed to be specified for BCS#5 only. Then two cases should be considered.

Case1: Legacy NW supporting BCS#5, but not support the aggregated bandwidth capability, e.g. Rel-17 NW.

In case1, one may argue there is no such Rel-17 NW in the market. If this is common understanding, there will be no inter-operability issue with caes-1 NW. In other words, it is required that the NW supporting BCS#5 will definitely support the aggregated bandwidth capability.

Case2: Legacy NW not supporting BCS#5, and not supporting the aggregated bandwidth capability, e.g. Rel-15/16 NW.

According to the agreement in RAN2#116, the UE indicating support of BCS#4/5 should also indicate the other BCS that it supports for this band combination, and introduction of BCS#4/5 will cause no backward compatibility problem. Since the CC bandwidth combinations for BCS#4/5 is larger than that for other BCS, to avoid the reported band combination missed by the legacy NW, at least one other BCS (e.g. BCS#0) should be reported, in this way, no NBC problem happens.

- A UE that indicates BCS#4/5 for a band combination should also indicates the other BCS that it supports for this band combination (no specification change expected). - RAN2 confirm that the introduction of BCS4 and BCS5 does not cause a backward compatibility problem, and the signalling can be introduced within the existing band combination list, i.e. no need to introduce a new band combination list.

However, in case2, the legacy NW may only support legacy BCS, e.g. BCS#0. Without the limitation of aggregated bandwidth, the CC bandwidth implicitly derived based on other BCS may be larger than the actual CC bandwidth supported by the UE. To avoid NBC, following options are discussed:

- Option1: The aggregated bandwidth is only reported for band combinations with BCS#5 specified in spec.

With option1, we see there is no NBC issue if there is the common understanding as in case1. However, according to current 38.101, we see most typical band combinations are specified from BCS#0. In other words, if option1 is adopted, the new signaling will be only applicable for corner case which may even not be deployed in most network.

- Option2: Make sure the signalled CC bandwidths are compatible with the other BCS (e.g.BCS#0) supported by the UE.

For example, for CA_n25A-n41A-n66A above, if the supported aggregated bandwidth is 160MHz, there will be no NBC issue to report BCS#0, as the signalled CC bandwidths are no more than the CC bandwidth combinations defined in BCS#0 as table-1. The issue of option2 is, it is hard to guarantee the signalled CC bandwidths are compatible with other supported BCS. It depends on the supported aggregated bandwidth of the UE, as well as the CC bandwidth combinations defined by other BCS.

First, the supported aggregated bandwidth is up to UE. In the example above, if the supported aggregated bandwidth is less than 160MHz, the BCS#0 cannot be compatible. Besides, for some band combinations, the CC bandwidths for BCS#0 is already large enough. For example, for CA_n41A-n79A, the CC bandwidth for BCS#0 has been specified up to 100MHz, which makes it hard to be compatible with the over-reported CC bandwidths.

Table-3   Bandwidth combinations for CA_n41A-n79A

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n41A-n79A | n418 n798 CA_n41A-n79A8 | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n41 | 10, 15, 20, 40, 50, 60 | 1 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 2 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |

In the cases above, to avoid inter-operability issue, if the legacy BCS which is not compatible with the signalled CC bandwidth is not reported, the legacy NW will miss this band combination. It will lead to performance degradation to legacy NW when the band combinations supported by the UE cannot be used. It is also not aligned with what we agreed when introducing BCS#4/5.

In addition, the option2 brings a high complexity for the UE when reporting the bandwidth capability. To avoid inter-operability issue and to achieve a better performance in legacy NW, the workload for the UE is high to determine what BCS could be reported for each band combination.

Observation 2: It brings performance degradation to the legacy NW and high complexity to the UE if only the supported BCS(s) compatible with the signalled CC bandwidth is reported.

In our view, a general solution is needed to address the backward compatibility issue and make the aggregated bandwidth mechanism feasible for all cases. Based on the analysis above, we found the key issue is, from legacy NW perspective, the original logic is used to determine the supported bandwidth for a CC, i.e. by validating the channelBW, the BCS, and the supportedBandwidth for a CC together. To avoid NBC, it is important to keep the capability reporting following the original logic to legacy NW, and introduce new mechanism of aggregated bandwidth in a dependent way only acknowledgeable to upgraded NW.

One solution is, the legacy supportedBandwidth field for a CC is still reported following the original logic for the legacy NW, for the upgraded NW, when the aggregated bandwidth is reported, the maximum CC bandwidth is considered to be same as the maximum supported channel bandwidth for corresponding bands, and the upgraded NW ignores the reported supportedBandwidth field in FSPC. It is up to UE how many CC bandwidth combinations are reported through the legacy way considering trade-off between signalling overhead and performance in legacy NW. One example is in table-4. In this way, backward compatibility can be ensured.

Table-4  CC bandwidth in legacy NW and in upgraded NW

| # |  | n41A max CC BW(MHz) | n79A max CC BW (MHz) | Aggregatd BW (MHz) |
| --- | --- | --- | --- | --- |
| 1 (First line of FS) | Legacy NW | 60 (reported in supportedBandwidth) | 100 (reported in supportedBandwidth) | / |
|  | Upgraded NW | 100 (determined implicitly according to channelBW of n41) | 100 (determined implicitly according to channelBW of n79) | 160 |
| 2 (Second line of FS) | Legacy NW | 100 (reported in supportedBandwidth) | 60 (reported in supportedBandwidth) | / |
|  | Upgraded NW | 100 (determined implicitly according to channelBW of n41) | 100 (determined implicitly according to channelBW of n79) | 160 |

Proposal 1: To avoid NBC, when the aggregated bandwidth is reported for a BC, the upgraded NW considers the maximum CC bandwidth is same as the maximum channel bandwidth reported for corresponding bands, and ignores the supportedBandwidth capability reported in FSPC.  The supportedBandwdith capability reporting follows the legacy principle.

Other signalling design

In previous discussion, there was concern on the aggregated bandwidth mechanism, because to utilize the new capability solution, the feature set is required to be applicable to all of the bandwidth combinations within the aggregated bandwidth, and all of the FeatureSetPerCCs e.g. MIMO layer, within the feature set are required to be identical to each other. If any capability parameter within the feature set has a different value between different bandwidth combinations, multiple feature sets are still needed.

Proposal 2: Different feature sets should be reported if different CC bandwidth combinations have different capabilities, e.g. MIMO layer.

In RAN4 LS, there are following descriptions on the aggregated bandwidth capability IE:

- If a band combo has mix of TDD and FDD CCs, the new IE would need to signal independent values for max TDD aggregated BW and for max FDD aggregated BW.

For the above bullet, the aggregated bandwidth for FDD and TDD CCs should be signalled separately. Then for a band combination with mixed FDD and TDD CCs, more than one {FDD, TDD} aggregated bandwidth pairs may be supported by the UE, e.g. {100, 20}, {90, 30}, {80, 40} and so on.

Observation 3: For a band combination, more than one {FDD, TDD} aggregated bandwidth pairs may be supported by the UE.

According to our analysis above, one possible reason to differentiate FDD/TDD may be the typical SCS for FDD and TDD is different, for the same bandwidth, the processing resources for FDD CCs and TDD CCs are different. Then with different FDD BW and TDD BW, the feature sets may be not identical to each other. For example, for a band combination {bandA CC1(FDD), bandB CC2(TDD)}, there are following capability sets:

Table-5  Example for different feature set for different bandwidth combinations

| # | Band combination | BW(mHz) | MIMO | Aggregated BW(mHz) |
| --- | --- | --- | --- | --- |
| 1 | bandA CC1(FDD) | 100 | 2 | 120 |
|  | band B CC2(TDD) | 20 | 4 |  |
| 2 | bandA CC1(FDD) | 20 | 2 |  |
|  | band B CC2(TDD) | 100 | 2 |  |

In this case, different feature sets are needed for the above two types of bandwidth combinations, in which the {FDD, TDD} aggregated bandwidth pairs are {100, 20} and {20, 100} respectively.

Then for a band combination with more than one {FDD, TDD} bandwidth pairs, a list of {FDD, TDD} bandwidth pairs could be reported for the band combination which share the same feature set. When the feature set cannot be shared for different FDD/TDD bandwidth pairs, separate BC entries or separate lines of feature set should be used for these bandwidth pairs.

Proposal 3: For a band combination with mixed FDD and TDD CCs, a list of {FDD, TDD} bandwidth pairs can be reported for the band combination which share the same feature set, and separate BC entries or separate lines of feature set are used when the feature set cannot be shared between different {FDD, TDD} bandwidth pairs.

Conclusion

In this contribution, we have following observations and proposals:

Observation 1: The signalling overhead exists when the supported aggregated bandwidth is lower than the maximum aggregated bandwidth defined by corresponding BCS.

Observation 2: It brings performance degradation to the legacy NW and high complexity to the UE if only the supported BCS(s) compatible with the signalled CC bandwidth is reported.

Observation 3: For a band combination, more than one {FDD, TDD} aggregated bandwidth pairs may be supported by the UE.

Proposal 1: To avoid NBC, when the aggregated bandwidth is reported for a BC, the upgraded NW considers the maximum CC bandwidth is same as the maximum channel bandwidth reported for corresponding bands, and ignores the supportedBandwidth capability reported in FSPC.  The supportedBandwdith capability reporting follows the legacy principle.

Proposal 2: Different feature sets should be reported if different CC bandwidth combinations have different capabilities, e.g. MIMO layer.

Proposal 3: For a band combination with mixed FDD and TDD CCs, a list of {FDD, TDD} bandwidth pairs can be reported for the band combination which share the same feature set, and separate BC entries or separate lines of feature set are used when the feature set cannot be shared between different {FDD, TDD} bandwidth pairs.

References

[1] R2-2302439, LS on UE signalling for the maximum aggregated bandwidth for FR1 CA, RAN4.