3GPP TSG-RAN WG2 Meeting#123                                	R2-230xxxx

Toulouse, France, Aug 21-25, 2023

Agenda item:	7.6.2.2

Source: 	Nordic Semiconductor ASA

Title: 	GNSS Validity duration Reporting

Document for:	Discussion and Approval

# Introduction

In RAN2#121bis it was agreed that UE can stay in RCC_CONNECTED when existing GNSS position becomes out-of-date if the UE enters a GNSS measurement gap. However, it is not agreed yet how the gap duration is configured to the UE. In RAN2#122 there is a working assumption that a new MAC CE is introduced to trigger connected UE to perform GNSS measurement. This MAC CE can be used to configure the gap duration (as part of the MAC CE payload).

RAN2#122 also had a working assumption that an UL MAC CE will be used to report the remaining GNSS validity duration.   As discussed, for example, in [1] and [2] there is a need for a new UE/NW timer during which the UE can report the new GNSS validity timer after the GNSS measurement gap has ended and the NW can expect the report from the UE before concluding the UE did not manage to obtain a new GNSS fix. It is open how this new timer could be configured to the UE.

Concerning the GNSS validity duration reporting it needs to be decided whether there should be a new SR for validity reporting or should an eNB simply transmit a UL grant for a UE for validity reporting purposes. Both solutions seem to have minimal specification impact. On the other hand, an eNB providing an UL grant would be more power efficient from the UE point of view.

Agreements in RAN2#121bis:

- There is no need for UE to provide GNSS position fix time duration in Msg3.

- RLM is suspended during the GNSS measurement gap while the UE is measuring GNSS

- UE can stay in RRC_CONNECTED state when current GNSS position becomes out-of-date if the UE enters a GNSS measurement gap. FFS whether the new GNSS measurement shall be started before, upon or after the current GNSS validity duration expiry

- For a UEs that cannot acquire system information and GNSS position at the same time, acquisition of SIB31 may be postponed until GNSS measurement is completed if the UE cannot complete acquisition of SIB31 before the start of GNSS measurement gap

- For the NB-IoT CP solution, UE will report the GNSS validity duration by using a MAC CE

- RAN2 can discuss UE autonomously reacquire GNSS during inactive state of C-DRX based on RAN1’s input in the next RAN2 meeting

- Working Assumption: GNSS validity duration UE reported after GNSS measurement is the remaining validity duration

Agreements in RAN2#122:

- Confirm the working assumption that GNSS validity duration UE reports is the remaining validity duration.

- The UE triggers GNSS measurement reporting every time upon completing the GNSS fix operation.

- When network triggers GNSS measurement initiation is up to network implementation.

- Add a note to state some AS operations are suspended when UE is performing GNSS measurement during GNSS measurement gap.

- GNSS fix time duration should be reported in 1) and 2):

- RRCConnectionReestablishmentComplete and RRCConnectionReestablishmentComplete-NB

- RRCConnectionReconfigurationComplete for HO case

(FFS whether there are some scenarios where this is not needed or whether there has to be some explicit NW indication to do so)

- Working Assumption: An UL MAC CE for GNSS validity duration reporting is used for NB-IoT user plane solution and eMTC UE as well, in addition to previously agreed NB-IoT control plane solution

- Working Assumption: A new DL MAC CE is introduced to trigger connected UE to perform GNSS measurement.

In this paper, we propose an implementation choice for delivering both the GNSS measurement gap duration and the new GNSS validity reporting timer value after ending the GNSS measurement gap.

# Discussion

## GNSS measurement gap and validity reporting timer configuration

The GNSS measurement gap duration is bounded by the UE reported GNSS position fix duration. Therefore, the GNSS measurement gap duration could share the same value definitions as the enumerated PositionFixDuration-r18 IE. Subsequently, 4 bits is enough to index all the defined values. This would leave another 4-bit field for signaling the GNSS validity reporting timer values in the GNSS Measurement Command MAC Control Element.

As discussed in [1] and [2] a new GNSS validity reporting timer would be needed that is started after the GNSS measurement gap has ended. The GNSS validity reporting timer values that can be indexed in the GNSS Measurement Command MAC Control Element are FFS.

Figure 6.1.3.xx-1 GNSS Measurement Command MAC Control Element

The GNSS measurement gap duration enumerated values are aligned with the GNSS-PositionFixDuration-r18 values.

The GNSS Measurement Command MAC Control Element has two 4-bit fields indicating the GNSS measurement gap duration and the GNSS validity reporting timer values.  The GNSS validity reporting timer values are FFS.

## GNSS validity duration reporting

Once the GNSS measurement gap has ended and before the GNSS validity reporting timer has expired, the UE would need to send the remaining GNSS validity duration report to the eNB. The reporting could be UE initiated, i.e. a new SR would be needed.  This solution would have minimal specification impact.

Another approach would be that the eNB continues giving one or more UL grants using the existing search space for the UE after the GNSS measurement gap has ended until the GNSS validity reporting timer expires or the eNB receives the report from the UE. This solution would also have minimal specification impact. However, compared to the SR-based solution the UL grant-based solution would be more power optimized from the UE point of view.

The eNB provides the UE with one or more UL grants for the remaining GNSS validity duration reporting. The UL grants for reporting purposes would start after the GNSS measurement gap has ended and last until the GNSS validity reporting timer has expired.

# Conclusion

In this paper, we proposed a signaling for configuring both the GNSS measurement gap duration and the GNSS validity reporting timer as well as how the remaining GNSS validity duration could be reported. Based on the discussion in the paper, we respectfully ask RAN2 to discuss and consider the following proposals:

- Proposal 1	The GNSS measurement gap duration enumerated values are aligned with the GNSS-PositionFixDuration-r18 values.

- Proposal 2	The GNSS Measurement Command MAC Control Element has two 4-bit fields indicating the GNSS measurement gap duration and the GNSS validity reporting timer values.  The GNSS validity reporting timer values are FFS.

- Proposal 3	The eNB provides the UE with one or more UL grants for the remaining GNSS validity duration reporting. The UL grants for reporting purposes would start after the GNSS measurement gap has ended and last until the GNSS validity reporting timer has expired.

# References

# R2-2305894 (Google, Issues for the GNSS Validity Reporting)

- R1-2308011 (Nordic, Improved GNSS operations for IoT NTN)