3GPP TSG-RAN WG2 Meeting #123	             R2-2308886

Toulouse, France, August 21-25, 2023

Agenda item:	    7.3.2

Source: 		CEWiT

Title: 	Cell DTX/DRX NES Techniques

Document for:	Discussion

# Introduction

The technique of Cell DTX/DRX for network energy saving was studied and the outcome is provided in TR 38.864 [1]. Regarding network configuring Cell DTX/DRX, some issues were raised about the configuration of multiple Cell DTX/DRX patterns in the previous meeting RAN2#122[2]. There were proposals for the handling of HARQ feedback as well. However, the proposals were not discussed in the meeting. In this contribution, we discuss details of multiple configuration for Cell DTX/DRX and impact of Cell DTX/DRX on HARQ feedback.

# Discussion

A Cell DTX/DRX pattern is a cycle of active and non-active periods for the gNB which can be configured per serving cell. In the active period of the cell DTX/DRX, the gNB switches on in intervals to perform the transmission and/or reception of signals and channels. The DTX/DRX reduces the ON time, transmissions and receptions of gNB, resulting in energy saving. The DTX/DRX should have a set of parameters associated with it. E.g., total duration, On duration, OFF duration, duty cycle, periodicity, etc. The value for each parameter of the cell DTX/DRX can be determined based on various factors such as load in network, connection density and UE arrival rate. The energy saving at gNB can be improved if the DTX/DRX can adapt to the network conditions. Hence, there should be multiple patterns for cell DTX/DRX of which one can be activated based on network condition. For e.g., a scenario for the requirement of multiple Cell DTX/DRX configurations is shown in fig.1. If the load is high and a number of UE are there with different cDRX patterns as shown in fig. 1(a), the cell DTX/DRX pattern should have a larger ON duration to accommodate all the UEs. Similarly if the load is low and only very few UEs are present, the cell DTX/DRX may adapt to a pattern with smaller ON duration as shown in fig. 1(b). Hence, Multiple Cell DTX/DRX configurations should be configured to the UE considering different network requirements. One configuration among the multiple configurations can be activated dynamically.

Proposal 1: Multiple Cell DTX/DRX configurations with different value of parameters should be provided to the UE. Dynamic signalling can be used to activate only one configuration at a time.

Fig 1. Multiple Cell DTX/DRX configurations.

(b)

The Cell DTX/DRX operations will also affect the generation of HARQ-ACK codebook at the UE. E.g., when candidate occasion for HARQ feedback overlaps with the non-active periods of cell DTX/DRX, then both gNB and UE are not expected to receive any signal/channel. In that case including NACK in HARQ codebook is redundant and it increases the UL overhead. Therefore, the candidate occasions for HARQ feedback overlapping with non-active period of DTX/DRX should be considered as non-candidate occasion for HARQ feedback and ACK/NACK should not be included in the HARQ codebook.

Observation 1: Including ACK/NACK feedback for candidate occasions for HARQ feedback overlapping with non-active period of DTX/DRX results in redundancy and increase UL overhead.

Proposal 2: The candidate occasions for HARQ feedback overlapping with non-active period of DTX/DRX should be treated as non-candidate occasion for HARQ feedback.

In case non active period of DTX/DRX pattern overlaps with HARQ feedback occasion, deprioritizing the HARQ feedback occasion over DTX/DRX pattern results in performance loss at UE. Further, reconfiguring the deprioritized HARQ feedback occasion results in additional signaling and redundancy. Therefore, mechanisms for reassigning the deprioritized HARQ feedback occasion after the DTX/DRX duration should be considered. For e.g., instead of rescheduling the deprioritized HARQ feedback occasion with all configurations, the HARQ feedback occasion can be reassigned by a time offset to the UE, so that the HARQ feedback will be performed after the offset. The remaining parameters of the HARQ feedback operation (e.g., mapping of candidate etc.) will remain the same as that of the initial configuration.

Observation 2: Following observation is made about deprioritizing HARQ feedback occasion over cell DTX/DRX

- Deprioritizing results in performance loss at UE

- Rescheduling the HARQ feedback occasion results in additional signaling and redundancy.

Proposal 3: Deprioritized HARQ feedback occasion should be reassigned after Cell DTX/DRX off duration using a time offset.

# Conclusion

Based on the above discussion, following proposals are made

Proposal 1: Multiple Cell DTX/DRX configurations with different value of parameters should be provided to the UE. Dynamic signaling can be used to activate only one configuration at a time.

Observation 1: Including ACK/NACK feedback for candidate occasions for HARQ feedback overlapping with non-active period of DTX/DRX results in redundancy and increase UL overhead.

Proposal 2: The candidate occasions for HARQ feedback overlapping with non-active period of DTX/DRX should be treated as non-candidate occasion for HARQ feedback.

Observation 2: Following observation is made about deprioritizing HARQ feedback occasion over cell DTX/DRX

- Deprioritizing results in performance loss at UE

- Rescheduling the HARQ feedback occasion results in additional signaling and redundancy.

Proposal 3: Deprioritized HARQ feedback occasion should be reassigned after Cell DTX/DRX off duration using a time offset.

# References

- TR 38.864, Study on network energy savings for NR, December 2022.

- Chair Notes, 3GPP TSG-RAN WG2 Meeting #122, Incheon, Korea, May 22-26, 2023