3GPP TSG-RAN WG2 Meeting #123	R2-2308900

Toulouse, France, August 21-25, 2023

Agenda Item:	7.7.4.2

Source:	Ericsson

Title:	Handover enhancements

Document for:	Discussion, Decision

# 1	Introduction

The work item for NR NTN enhancements in Rel-18 [1] emphasizes NTN-TN and NTN-NTN mobility and service continuity as an important area for improvements.

For handover enhancements, the WID points out the following two aspects to consider:

- “Specify NTN-NTN handover enhancements for RRC_CONNECTED UEs in the quasi-earth-fixed cell and earth-moving cell to reduce the signalling overhead.”

- “Study and, if needed, specify enhancement to Xn[/NG] signalling to support feeder link switch-over, CHO, e.g., exchange of necessary information between gNBs.”

In this paper, we address the first aspect above.

# 2	Discussion

## 2.1	RACH-less Handover

### 2.1.1	Previous agreements

In RAN2#121, it was agreed to support RACH-less handover for NTN in Rel-18. In RAN2#121bis-e, the following agreements were made on RACH-less handover:

- For initial UL transmission in RACH-less HO, support pre-allocated grant in RACH-less HO command.

- NTN RACH-less HO is supported for Intra-satellite handover with the same feeder link. i.e., with same gateway/gNB.

- NTN RACH-less HO can be supported for intra-satellite handover with different feeder links, i.e., with gateway/gNB switch, inter-satellite handover with gateway/gNB switch, and inter-satellite handover with same gateway/gNB.

- The pre-allocated grant is provided as type-1 CG

- RAN2 confirms the general UE procedure for NTN RACH-less HO

  - receive a RACH-less HO command which can include pre-allocated grant optionally. FFS N_TA is optional. (RRC)

  - start timer T304 for the target cell (RRC)

  - perform DL and UL synchronization, and start timer T430. FFS how to perform RACH-less UL synchronization to NTN target cell. (RRC, MAC)

  - start time alignment timer (MAC)

  - monitor target cell PDCCH for dynamic grant if pre-allocated grant is not configured in RACH-less HO command (MAC, PHY)

  - send initial UL transmission including RRCReconfigurationComplete message using the available UL grant (RRC, MAC, PHY)

  - consider RACH-less HO is completed upon receiving NW confirmation. FFS how to confirm RACH-less HO is successfully completed. (RRC, MAC)

  - stop timer T304 for the target cell. (RRC)

  - FFS whether to release UL grant if pre-allocated after RACH-less HO completion

  - FFS RACH-less HO failure handling, e.g. whether UE fallback to RACH-based HO to the target cell

  - FFS procedure for RACH-less HO combined with PCI unchanged or CHO if supported

- At least for pre-allocated grant, for the confirmation of RACH-less HO completion we reuse of LTE approach, i.e., UE Contention Resolution Identity MAC CE is used but UE ignores the content of this field. FFS if anything else is needed for dynamic grant

- Consider to support combining RACH-less HO with time-based CHO for NTN, taking into account the 1) validity of pre-allocated grant and potential waste of reserved resource; 2) when/how to provide dynamic grant in PDCCH

In RAN2#122, further agreements were made on RACH-less handover:

- In NTN RACH-less handover, NW either indicates NTA in the target cell is identical to the source cell, or the NTA explicitly provided by the NW is 0. RAN2 will not discuss the case where NTA does not equal to 0.

- From RAN2 perspective synchronization among source and target cells is not an issue in NTN RACH-less HO.

- Release pre-allocated UL grant after RACH-less HO completion.

- LTE approach (of confirming the HO completion) is reused for both pre-allocated grant and dynamic grant. FFS any enhancement to the confirmation of RACH-less HO completion, e.g. the NW does not send the UE Contention Resolution Identity MAC CE, and sends PDCCH/PDSCH addressed to C-RNTI.

- Remove "FFS how to perform RACH-less UL synchronization to NTN target cell", RAN2 assumes the UL sync handling in the target cell is the same in RACH-based HO and RACH-less HO, except how to acquire NTA (FFS on the spec impact, if any).

The objective of this section is to discuss the highlighted aspects above: the timing configuration of pre-allocated uplink grant, the possible fallback mechanism, and the combination with CHO.

### 2.1.2	Pre-allocated UL grant timing configuration

In RAN2#121bis-e, it was agreed that the pre-allocated grant is provided as type-1 configured grant (CG). In NR, configured grant eliminates the need to request and assign resources for each uplink packet transmission by pre-allocating resources to the UE. The timing configuration is determined by several RRC parameters, namely timeReferenceSFN and timeDomainOffset. The former, timeReferenceSFN, indicates the SFN used to determine the offset (timeDomainOffset) of a type-1 CG resource in the time domain [2]. This allows to support a greater variety of allocation patterns for type-1 CG, i.e., the CG periodicities with which the CG resource appears in different position in different frames.

Even though the pre-allocated uplink grant in RACH-less HO could also benefit from such flexibility, the enhanced complexity is not justified. Firstly, the purpose of this uplink grant is to provide initial access to the target cell. Secondly, the grant shall be determined by the target gNB, and the corresponding timing reference shall be associated with the target cell’s SFN. However, the description of timeReferenceSFN is tied to the serving cell SFN and the reception of the RRCReconfiguration message which contains the configured grant configuration. Therefore, re-using a similar timing configuration as type-1 CG, i.e., with the parameters timeReferenceSFN and timeDomainOffset, requires the source gNB to be informed (in case of inter-gNB handover) leading to potential changes in inter-node signaling and RAN3 impact.

The timing reference of a type-1 CG is defined by the reception of the RRCReconfiguration message containing the grant configuration.

In NTN RACH-less HO, the timing reference of the pre-allocated uplink grant shall be determined by the target gNB.

Introducing a reference SFN similar to timeReferenceSFN for pre-allocated UL grant introduces unnecessary complexity and specification impact (especially for RAN3).

RAN2 to discuss how to provide the timing configuration for the pre-allocated grant in RACH-less handover based on the target’s cell time reference (SFN).

The following excerpt from TS 38.331 [3] shows the wide range of values that can be configured for the uplink grant periodicity. On the one hand, some periodicity values have a granularity which is not needed for handover. For instance, 2 symbols and 7 symbols periodicity value leads to higher UL resource consumption while it does not reduce the overall HO latency which is dominated by other components such as DL sync with the target cell or UE processing time. On the other hand, 80, 160, 320, 640 slots periodicity value leads to a distinctly increased HO latency with 15KHz SCS (HO latency increase may be smaller with higher SCS). Thus, RAN2 should discuss which periodicity values apply to the case of NTN RACH-less HO pre-allocated UL grant.

NTN RACH-less HO pre-allocated UL grant does not require the wide range of periodicity values available for type 1 CG (e.g., symbol level or hundreds of slots).

--------------------------------------------- TS 38.331 ---------------------------------------------

periodicity Periodicity for UL transmission without UL grant for type 1 and type 2 (see TS 38.321 [3], clause 5.8.2). The following periodicities are supported depending on the configured subcarrier spacing [symbols]: 15 kHz: 2, 7, n*14, where n={1, 2, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 128, 160, 320, 640} 30 kHz: 2, 7, n*14, where n={1, 2, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 128, 160, 256, 320, 640, 1280} 60 kHz with normal CP 2, 7, n*14, where n={1, 2, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 128, 160, 256, 320, 512, 640, 1280, 2560} 60 kHz with ECP: 2, 6, n*12, where n={1, 2, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 128, 160, 256, 320, 512, 640, 1280, 2560} 120 kHz: 2, 7, n*14, where n={1, 2, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 128, 160, 256, 320, 512, 640, 1024, 1280, 2560, 5120} 480 and 960 kHz: n*14, where n={1, 2, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 128, 160, 256, 320, 512, 640, 1024, 1280, 2560, 5120} In case of SDT, the network does not configure periodicity values less than 5ms.

--------------------------------------------- TS 38.331 ---------------------------------------------

An associated problem with the lack of a parameter such timeReferenceSFN is the possibility that a certain periodicity value results in that the pre-allocated UL grant resource appears in different position in different frames. For example, this can happen with the periodicity values of 4, 8, 16, 32, 64, 128 slots with 15KHz SCS A simple solution without major specification impact is to restrict the range of possible values for RACH-less handover to those in which this issue does not happen, e.g., 1, 2, 5, 10, 20, 40, or 80 slots.

In NTN, the granularity to provide the periodicity value of the pre-allocated uplink grant for RACH-less handover is number of slots.

Based on type-1 CG, RAN2 to select which periodicity values should be adopted for pre-allocated UL grant in NTN RACH-less handover, e.g., 1, 2, 5, 10, 20, 40, or 80 slots.

### 2.1.3	Fallback mechanism to regular Random Access

During RAN2#121bis-e, a few companies proposed the possible fallback to RACH-based HO (CBRA) after failing to use the allocated grant, either pre-allocated or dynamically allocated, provided that T304 has not yet expired. We consider that the sole purpose of introducing RACH-less handover in NTN is to mitigate congestion due to limited RACH resources. Hence, a fallback mechanism would probably worsen this problem if UEs, which have been allocated reserved resources, are allowed to use the already congested channel. Legacy mechanisms (e.g., RLF) are sufficient to deal with the exceptions when UEs cannot use their grants.

During RACH-less handover, there is no real benefit of a fallback to RACH-based handover in NTN.

RAN2 does not specify a fallback mechanism NTN RACH-less handover in Rel-18.

### 2.1.4	Time Alignment Timer

TimeAlignmentTimer (TAT) determines how long the MAC entity of a UE considers the TA information (e.g., NTA) provided by the network valid. The network configures its value per TAG (Timing Advance Group) either via dedicated signalling or broadcast. In RAN2#122, it was agreed that “RAN2 assumes the UL sync handling in the target cell is the same in RACH-based HO and RACH-less HO”. This agreement implies that, similar to RACH-based HO, a UE performing a RACH-less HO autonomously calculates and pre-compensates timing advance in NTN.

A UE performing RACH-less HO can autonomously calculate and pre-compensate timing advance in NTN.

In NTN, the role of TAT is not a determining factor since TA validity is guaranteed by T430 which dictates how long the ephemeris and common TA parameters hold accurate. Therefore, during an NTN RACH-less HO, TAT shall not be a limiting factor to execute the handover (initial access) and its behaviour should be adapted such that the TA validity is guaranteed by T430. RAN2 can further discuss the details of how to adapt TAT mechanism so that its value is not applied during initial access in NTN RACH-based HO.

In NTN, TA validity is guaranteed by T430.

During NTN RACH-less HO, for initial access, TAT shall not limit TA validity nor the handover execution.

### 2.1.5	RACH-less Conditional handover

In RAN2#121bis-e, it was agreed to further consider combining RACH-less HO with time-based CHO for quasi-Earth fixed cells in NTN. Two of the topics that should be addressed are the validity of pre-allocated grant to minimize the potential waste of reserved resources and the provision of dynamic grant in PDCCH. Regarding the first topic, it would be up to gNB implementation to ensure an optimal allocation of reserved resources. The gNB is aware of the duration of CHO time window [T1, T2] (via inter-node signalling) and just needs to ensure that the pre-allocated grant is available to the UE within [T1, T2]. In a similar manner, for dynamic grant, the gNB will send the corresponding PDCCH carrying the uplink grant only during [T1, T2]. In conclusion, UE should not expect any allocation out of the assigned window.

In NTN RACH-less conditional handover, network ensures the pre-allocated grant, and the provision of dynamic grant happens within the CHO time window [T1, T2].

Adopt RACH-less conditional handover in NR NTN Rel-18. It is up to gNB implementation how to assign an optimal allocation and minimize reserved resources waste.

### 2.1.6	Reusing NTN RACH-less configuration in other WI

During the post email discussion handling the running RRC CR in RAN2#122, a few companies raised the point of considering forward compatibility in NTN RACH-less handover signalling. In this way, the field parameters or IEs that describe NTN RACH-less handover should be defined in a general manner so that they might be re-used for RACH-less in TN or mobile IAB. As a result, the following FFS was added:” whether the name, description and/or conditional presence of ntn-RACH-lessHO can be modified if other WIs reuse the NTN signalling for RACH-less (e.g., Mobile IAB)”.

Unlike in terrestrial networks or other WI, NTN RACH-less is facilitated by the provision of ephemeris and common TA parameters of the target satellite. This allows the UE to make an accurate calculation of the Timing Advance to make an initial transmission. This feature might not be common for other WI in TN. Thus, RAN2 should focus on tangible facts and define NTN RACH-less handover signalling exclusive to NTN.

RAN2 to design a signalling for RACH-less handover which is exclusive for NTN, i.e., without considering whether it may be re-used by other WI in the future.

## 2.2	Reusing PCI after service link switch

In RAN2#119bis-e, [4] proposed another mobility enhancement for a particular scenario: service link (satellite) switch in a quasi-Earth fixed cell whenever both satellites (leaving and incoming) are connected to the same gNB (see the following Figure 2). The core of the proposal is to reuse PCI during the service link switch, i.e., in the figure below, this means that both green and blue cells would have an identical PCI. The main advantage is that a UE does not have to perform a L3 Handover, with an obvious saving in signalling and processing power, and can continue using almost the same configuration (e.g., no key change).

Figure 2. Service link switch in quasi-Earth fixed cells [2].

In RAN2#121bis-e, it was agreed that “In quasi-earth fixed cell case, for hard satellite switch in the same SSB frequency and same gNB (no key change), satellite switching without PCI changing (not requiring L3 mobility) is supported, unless major technical issues are identified by RAN1”, and a corresponding LS was sent to RAN1. In RAN1#113 RAN1 replied that “From RAN1 perspective, no feasibility issue is identified for hard satellite switching without PCI change” [5], this means satellite switching without PCI changing for the hard switch case is feasible and will be supported in Rel.18.

Satellite switching without PCI changing for the hard switch scenario is feasible and will be supported in Rel-18.

### 2.2.1	Baseline procedure

During previous discussions, there seems to be different perspectives on the way both network and UE behave in the unchanged PCI scenario. The overall procedure would go as follows for the hard switch case:

- Network broadcasts assistance information to inform the UE about this type of switch.

- Upon t-service-17 expires, the UE makes use of the assistance information it has received and applies the NTN-Config-r17 information for the new cell (i.e., the cell after the switch) and restarts the validity timer associated with the ephemeris and Common TA parameters.

- The UE obtains DL synchronization in the new cell by receiving PSS and SSS.

- The UE acquires SIB19 after the switch to collect any remaining relevant information.

- The UE recalculates its TA, based on the ephemeris and Common TA parameters of the new cell (i.e. after the switch) and the UE’s own location. This TA should be accurate enough for the UE to transmit, e.g. on the PUCCH or the PUSCH, but the UE may also optionally perform a random access in the new cell to get its TA adjusted by the network.

- If configure to do so, the UE may also report its TA to the network using the Timing Advance Report MAC CE.

Adopt the described procedure as a baseline for the hard service link switch unchanged PCI scenario.

### 2.2.2	Implementation aspects

One issue is how to implement mobility in this scenario. In our view since this is an intra-gNB satellite switching without PCI changing nor key change, L1 mobility can be applied, i.e., the UE does not need to send a HO complete or switch complete command in the new satellite using either RRC signaling or MAC CE. Instead, the NW could schedule UL and DL transmission in the new satellite after the new satellite starts to serve the area, for UL the NW may either initiate a random-access procedure using PDCCH order or schedule a dynamic UL grant or activate a CG Type 2 for the UE’s UL transmission.

In the unchanged PCI scenario, UE does not need to send a confirmation to the network when it successfully connects to the target cell.

### 2.2.3	Backwards compatibility

Another open issue is how to support mobility of legacy (Release 17) UEs in this scenario. One option is regular handover in which, as presented in [6], the handover command needs to be sent to each UE after the last SSB from the serving satellite has reached all the UEs in the cell. Otherwise, a UE will re-synchronize again to the same source satellite, given the PCI and overall configuration are identical. An added problem with this solution is the implementation difficulty: HO commands cannot be spread in time and the gNB would be forced to send a high number of HO command in a very short period of time.

In the unchanged PCI scenario, the use of regular handover to handle Rel-17 RRC_CONNECTED UEs might pose implementation challenges.

A more feasible solution is to rely on time-based CHO, where T1 could be set to value later the time that the new satellite starts to serve the area. However, this is an optional feature. Thus, legacy UEs not supporting time-based CHO, and which cannot receive the HO command (after the last SSB from the serving satellite) due to a potential temporary congestion will need to rely on RLF and RRC Re-establishment for every service link switch. In this latter case, the interruption time will increase significantly. Therefore, we ask RAN2 to discuss whether sending the HO command after the last SSB from the serving satellite is a viable solution. If not, whether any enhancement should be introduced to support mobility for legacy UEs (particularly for UEs not supporting time-based CHO).

In the unchanged PCI scenario, the only feasible option for legacy UEs not supporting time-based CHO is RLF and Re-establishment which will result in a significant increase of interruption time.

Discuss how Rel-17 NTN UEs can use regular handover mechanism in the unchanged PCI service link switch.

# 3	Conclusions

In the previous sections we made the following observations:

Observation 1	The timing reference of a type-1 CG is defined by the reception of the RRCReconfiguration message containing the grant configuration.

Observation 2	In NTN RACH-less HO, the timing reference of the pre-allocated uplink grant shall be determined by the target gNB.

Observation 3	Introducing a reference SFN similar to timeReferenceSFN for pre-allocated UL grant introduces unnecessary complexity and specification impact (especially for RAN3).

Observation 4	NTN RACH-less HO pre-allocated UL grant does not require the wide range of periodicity values available for type 1 CG (e.g., symbol level or hundreds of slots).

Observation 5	During RACH-less handover, there is no real benefit of a fallback to RACH-based handover in NTN.

Observation 6	A UE performing RACH-less HO can autonomously calculate and pre-compensate timing advance in NTN.

Observation 7	In NTN, TA validity is guaranteed by T430.

Observation 8	In NTN RACH-less conditional handover, network ensures the pre-allocated grant, and the provision of dynamic grant happens within the CHO time window [T1, T2].

Observation 9	Satellite switching without PCI changing for the hard switch scenario is feasible and will be supported in Rel-18.

Observation 10	In the unchanged PCI scenario, the use of regular handover to handle Rel-17 RRC_CONNECTED UEs might pose implementation challenges.

Observation 11	In the unchanged PCI scenario, the only feasible option for legacy UEs not supporting time-based CHO is RLF and Re-establishment which will result in a significant increase of interruption time.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 to discuss how to provide the timing configuration for the pre-allocated grant in RACH-less handover based on the target’s cell time reference (SFN).

Proposal 2	In NTN, the granularity to provide the periodicity value of the pre-allocated uplink grant for RACH-less handover is number of slots.

Proposal 3	Based on type-1 CG, RAN2 to select which periodicity values should be adopted for pre-allocated UL grant in NTN RACH-less handover, e.g., 1, 2, 5, 10, 20, 40, or 80 slots.

Proposal 4	RAN2 does not specify a fallback mechanism NTN RACH-less handover in Rel-18.

Proposal 5	During NTN RACH-less HO, for initial access, TAT shall not limit TA validity nor the handover execution.

Proposal 6	Adopt RACH-less conditional handover in NR NTN Rel-18. It is up to gNB implementation how to assign an optimal allocation and minimize reserved resources waste.

Proposal 7	RAN2 to design a signalling for RACH-less handover which is exclusive for NTN, i.e., without considering whether it may be re-used by other WI in the future.

Proposal 8	Adopt the described procedure as a baseline for the hard service link switch unchanged PCI scenario.

Proposal 9	In the unchanged PCI scenario, UE does not need to send a confirmation to the network when it successfully connects to the target cell.

Proposal 10	Discuss how Rel-17 NTN UEs can use regular handover mechanism in the unchanged PCI service link switch.

# References

RP-222654, NR NTN (Non-Terrestrial Networks) enhancements, Thales, RAN#97, September 2022.

TS 36.331, v17.4.0, “Radio Resource Control (RRC) Protocol specification (Release 17)”

TS 38.331, v17.4.0, “Radio Resource Control (RRC) Protocol specification (Release 17)”

R2-2210862, Report from [AT119bis-e][119][NR NTN Enh] HO enhancements, Nokia, RAN2#119bis-e, October 2022.

R1-2306209, Draft reply LS to RAN2 on unchanged PCI, Moderator (CATT), RAN1#113.

R2-2304147, Considerations on unchanged PCI solution, Sequans, RAN2#121bis-e, April 2023.