3GPP TSG RAN WG2#123bis	R2-2309411

Xiamen, China, 9th - 13th October 2023

3GPP TSG RAN WG1 #114	R1-2308429

Toulouse, France, August 21 - 25, 2023

Title:	LS on Rel-17 PUCCH repetition enhancements

Response to:	-

Release:	Rel-17

Work Items:	NR_IIOT_URLLC_enh-Core

Source:	RAN1

To:	RAN2

CC:

Contact Person:

#### Name:	Klaus Hugl

E-mail Address:	klaus.hugl@nokia.com

Attachment:	               -

1. Overall Description:

PUCCH repetition for short PUCCH formats (PUCCH formats 0 & 2) is supported in Rel-17 based on the following earlier RAN1 agreements:

Agreement (@ RAN1#104-e): Support PUCCH repetition for PUCCH formats 0 and 2 at least for sub-slot based PUCCH repetition. FFS: Support for slot-based PUCCH repetition Agreement (@RAN1#106-e) Support slot-based PUCCH repetition for PUCCH Format 0 and Format 2 also for single TRP operation. - The support is subject to independent UE capability indication Agreement (@RAN1#106b-e) For PUCCH repetition enhancements: - Support inter-slotFrequencyHopping for PUCCH repetition operation of PUCCH Format 0 and Format 2 for slot-based PUCCH configurations. - Support inter-subslot Frequency Hopping for PUCCH repetition operation of PUCCH Formats 0, 1, 2, 3 and 4 for 7OS slot-based PUCCH configurations. - The UE applies the inter-subslot FH operation from sub-slot to sub-slot, if configured with inter-slotFrequencyHopping in the respective PUCCH_config. - (Working Assumption) Support inter-subslot Frequency Hopping for PUCCH repetition operation of PUCCH Format 0 and Format 2 for 2OS slot-based PUCCH configurations. - The UE applies the inter-subslot FH operation from sub-slot to sub-slot, if configured with inter-slotFrequencyHopping in the respective PUCCH_config. - Note: As for Rel-15, the configuration / enabling of inter-slotFrequencyHopping and intraSlotFrequencyHopping is not supported. Agreement (RAN1#107-e) Confirm the following RAN1 working assumption from RAN1#106bis-e with the additional agreement on UE capability (in RED): - (Working Assumption) Support inter-subslot Frequency Hopping for PUCCH repetition operation of PUCCH Format 0 and Format 2 for 2OS slot-based PUCCH configurations. - The UE applies the inter-subslot FH operation from sub-slot to sub-slot, if configured with inter-slotFrequencyHopping in the respective PUCCH_config.. - Support single UE capability indication of inter-subslot FH for PUCCH repetition operation.

While the additional support of PUCCH repetition for PUCCH format 0 has been captured in the description in 38.331, the description on the support for PUCCH format 2 is currently still missing in the related field descriptions.

Therefore, RAN1 kindly requests RAN2 to consider the following proposed changes to the field description of PUCCH-FormatConfig in the Rel-17 version of TS 38.331 to also correctly capture the PUCCH repetition support for PUCCH format 2:

PUCCH-FormatConfig, PUCCH-FormatConfigExt field descriptions

additionalDMRS If the field is present, the UE enables 2 DMRS symbols per hop of a PUCCH Format 3 or 4 if both hops are more than X symbols when FH is enabled (X=4). And it enables 4 DMRS symbols for a PUCCH Format 3 or 4 with more than 2X+1 symbols when FH is disabled (X=4). The field is not applicable for format 0, 1 and 2. See TS 38.213 [13], clause 9.2.2.

interslotFrequencyHopping If the field is present, the UE enables inter-slot frequency hopping when PUCCH Format 0, 1, 2, 3 or 4 is repeated over multiple slots. For longa PUCCH over multiple slots, the intra and inter slot frequency hopping cannot be enabled at the same time for a UE. The field is not applicable for format 2. See TS 38.213 [13], clause 9.2.6.

maxCodeRate Max coding rate to determine how to feedback UCI on PUCCH for format 2, 3 or 4. The field is not applicable for format 0 and 1. See TS 38.213 [13], clause 9.2.5.

maxCodeRateLP Max coding rate to determine how to feedback UCI on PUCCH for format 2, 3 or 4. The field is not applicable for format 0 and 1. This field configures additional max code rate in the second entry of PUCCH-ConfigurationList-r16 for multiplexing low-priority (LP) HARQ-ACK and high-priority (HP) UCI in a PUCCH as described Clause 9.2.5.3 of TS 38.213 [13]. The field is absent for the first entry of PUCCH-ConfigurationList-r16.

nrofSlots Number of slots with the same PUCCH. When the field is absent the UE applies the value n1. The field is not applicable for format 2. See TS 38.213 [13], clause 9.2.6.

pi2BPSK If the field is present, the UE uses pi/2 BPSK for UCI symbols instead of QPSK for PUCCH. The field is not applicable for format 0, 1 and 2. See TS 38.213 [13], clause 9.2.5.

rb-SetIndex Indicates the RB set where PUCCH resource is allocated.

simultaneousHARQ-ACK-CSI If the field is present, the UE uses simultaneous transmission of CSI and HARQ-ACK feedback with or without SR with PUCCH Format 2, 3 or 4. See TS 38.213 [13], clause 9.2.5. When the field is absent the UE applies the value off. The field is not applicable for format 0 and 1.

2. Actions:

To RAN WG2

ACTION: Take the above into account in the next update of the Rel-17 version of 38.331.

3. Date of Next RAN WG1 Meetings:

TSG-RAN WG1 Meeting #114bis		October 9 to 13, 2023		Xiamen, CN

TSG-RAN WG1 Meeting #115			November 13 to 17, 2023	Chicago, US