3GPP TSG RAN WG2#123bis	R2-2309421

Xiamen, China, 9th - 13th October 2023

3GPP TSG RAN WG1 #114	R1-2308566

# Toulouse, France, August 21st –25th, 2023

Title:	Reply LS to RAN2 on unchanged PCI

Response to:	R1-2304323(R2-2304273)

Release:	Rel-18

Work Item:	NR_NTN_enh-Core

Source:	RAN1

To:	RAN2

Cc:

Attachment:	no

Contact Person:

Name:	Deshan Miao

E-mail Address:	miaodeshan@catt.cn

# Overall description

RAN1 thanks RAN2 for the LS R1-2304323(R2-2304273) on unchanged PCI issue and would like to provide the following response to the question from RAN2:

Question 2: If it is feasible to support soft satellite switching without PCI change?

Reply:

Under the following conditions:

- UE is not required to connect to two satellites simultaneously during soft satellite switching.

- Interference avoidance/mitigation between two satellites may potentially be done by gNB implementation at least to ensure non-colliding SSB with same PCI at UE side.

- UE is provided with the information on new common TA, K_mac, ephemeris and cell-specific K-offset are applied during resynchronization to new satellite.

- UE may be provided with the information if needed to detect the SSB of the new satellite for soft satellite switching.

- The same UE behavior may be applied for soft satellite switching and hard satellite switching.

RAN1 concludes it is feasible for soft satellite switching without PCI change.

# Actions:

To RAN2:

RAN1 respectfully asks RAN2 to take the above information into account for future work.

# Date of Next TSG-RAN1 Meetings:

RAN1#114bis	09th -13rd Oct 2023	Xiamen, CN

RAN1#115	13rd -17th Nov 2023	Chicago, US