3GPP TSG RAN WG2#123bis	R2-2309451

Xiamen, China, 9th - 13th October 2023

3GPP TSG RAN WG4 Meeting#108	R4-2314351

Toulouse, France, August 21 – 25, 2023

Title:	LS on NR SL unlicensed LBT failures UE behavior

Release:	Release 18

Work Item:	NR_SL_enh2-Core

Source:	TSG RAN WG4

To:	TSG RAN WG2

Cc:	TSG RAN WG1

Contact person:

Name:		  Santhan Thangarasa

E-mail Address:	santhan<dot>thangarasa<at>@ericsson<dot>com

# 1	Overall description

RAN4 has discussed RRM requirements for following sidelink procedure:

- Procedure for initiation/cease of SLSS transmissions

Based on the discussion, RAN4 has agreed to define measurement delay requirements as function of CCA failures (L1) during the evaluation period (Tevaluate,SLSS_CCA), where

- L1 is the number of occasions containing sidelink synchronization reference signal of the SyncRef UE not available at the UE during Tevaluate,SLSS_CCA due to the CCA failures; L1, max is the maximum number of allowed occasions containing sidelink synchronization reference signal of the SyncRef UE not available at the UE during Tevaluate,SLSS_CCA due to the CCA failures where L1 ≤ L1, max.

When the extension of evaluation period reaches L1, max, the number of available occasions containing sidelink synchronization reference signals during Tevaluate,SLSS_CCA is not enough to derive an accurate PSBCH-RSRP measurement result, which cannot be used to compared with syncTxThreshO and futher determine whether to initiate/cease SLSS transmission.

RAN4 would like to ask RAN2 which of the following options regarding the UE behaviour is reasonable from RAN2 perspective:

- The UE measures PSBCH-RSRP and evaluates whether to initiate/cease SLSS transmissions within Tevaluate,SLSS_CAA, and upon exceeding the maximum allowed CCA failures during Tevaluate,SLSS_CCA, the UE shall:

  - Opton 1: cease all SLSS transmissions,

  - Option 2: initiate SLSS transmissions

  - Option 3: UE keeps current SLSS transmission status.

# 2	Actions

To TSG RAN2

ACTION: RAN4 respectfully asks RAN2 provide feedback on the expected UE actions upon exceeding the maximum allowed CCA failures during the evaluation period.

# 3	Dates of next TSG RAN WG4 meetings

TSG RAN WG4 Meeting #108-bis	9th  - 13th October 2023	Xiamen, China

TSG RAN WG4 Meeting #109	13th – 17th November 2023	Chicago, US