3GPP TSG-RAN WG2 Meeting #123bis	R2-2309524

# Xiamen, China, 9 – 13 October 2023

Agenda item:		7.5.5

Source:	Xiaomi

Title:	UE capabilities for XR

Document for:		Discussion and Decision

Introduction

In RAN2#123 meeting, following guidance was provided on how to progress XR UE capability discussion:

UE capabilities will be discussed in the next meeting(s) based on company inputs. Companies are encouraged to provide also Stage-3 details of their proposals, e.g. draftCRs on the capabilities to allow better comparison of the proposals. Interested companies bringing documents to this AI should contact specification rapporteur to consolidate their proposals offline.

XR UE capability rapporteur coordinated offline among interested companies and provided a contribution R2-2310148 [1] with common proposals/agreements as well as discussion points.

In this contribution, we discuss the open points (as from [1]) of UE capabilities for XR, and the aspects (e.g. pre-requisites) not covered by [1].

Discussion

XR awareness

In [1], dynamic UE capability for XR was discussed:

In addition, XR awareness by AS layer is a bit distinct from traditional UE capabilities as it has a dependency to the XR Application. This may be easily achievable for some applications, but not for others. For example, the UE may not be able to do PDU set identification in case upper layers utilize a protocol which performs packet encryption such as QUIC or IPSec, or when the application does not employ RTP header extensions on which the UE normally relies for PDU set identification. So even if the UE indicates a capability to perform PDU set based discarding in AS layer, it may not be able to do that eventually, depending on which application is actually running. In summary, XR awareness in UL and UE support of related capabilities may be a more dynamic kind of information. How this is defined would require further discussion, but it seems agreeable that this scenario should be addressed.

Proposal 2: To discuss how to handle the possibility that UE’s XR awareness may be dynamic and dependent on the XR application.

First of all, although UE might not be able to identify PDU set / PSI purely based on protocol headers (e.g. due to encryption of headers or absence of RTP header extensions), it is possible for UE to get information of PDU set / PSI via inter-layer interaction, which is UE internal operation and up to UE implementation.

Observation 1: Inter-layer interaction within UE may help UE to obtain information of PDU set / PSI.

In the remaining of this section, we investigate the impact on related UE capabilities from dynamic / application-specific nature, assuming that above inter-layer interaction might not be always available.

XR related features/capabilities are UL traffic information, PDU set discard, and PSI based discard. Related proposals from [1] are as follows:

Proposal 3: At least a new optional UE capability signaling (e.g., xr-AssistanceInfo) is defined for the new UE assistance information related to XR traffic requires i.e., jitter range, burst arrival time, and UL data burst periodicity per QoS flow.

Proposal 4: A new optional UE capability signaling (e.g., discardPDU-Set) is defined to identify Rel-18 UEs supporting PDU set based discard operation (i.e. pdu-SetDiscard configuration, as per the current running RRC CR), and another new optional UE capability signaling (e.g., discardPSI) is defined to identify Rel-18 UEs supporting PSI based discard (i.e. psi-BasedDiscard configuration, as per the current running RRC CR).

UE only reports the UL traffic information if the related measurement is available, as from RRC running CR R2-2309315 [2] below.

2>	for each PDU session for which the UE intends to provide UL traffic information in this UEAssistanceInformation message:

3>	set pdu-SessionID to the value of the concerned PDU session ID;

3>	for each QoS flow of this PDU session for which the UE intends to provide UL traffic information in this UEAssistanceInformation message:

4>	set qfi to the value of the concerned QFI;

4>	if the jitter range measurement is available; and

4>	if the UE did not provide jitter range since it was configured to provide UL traffic information, or if the measured jitter range has changed since the last transmission of the UEAssistanceInformation message containing jitterRange:

5>	set jitterRange to the latest measured value of the jitter range;

4>	if the burst arrival time measurement is available; and

4>	if the UE did not provide burst arrival time since it was configured to provide UL traffic information, or if the measured burst arrival time has changed since the last transmission of the UEAssistanceInformation message containing burstArrivalTime:

5>	set burstArrivalTime to the latest measured value of the burst arrival time;

4>	if the traffic periodicity measurement is available; and

4>	if the UE did not provide traffic periodicity since it was configured to provide UL traffic information, or if the measured traffic periodicity has changed since the last transmission of the UEAssistanceInformation message containing trafficPeriodicity:

5>	set trafficPeriodicity to the latest measured value of the traffic periodicity.

Therefore the reporting of UL traffic information can already handle the dynamic aspects of XR traffic, e.g. when data burst information is not available.

Proposal 1: The reporting of UL traffic information can already handle the dynamic aspects of XR traffic (e.g. when data burst information is not available), and additional enhancements are not needed.

For PDU Set discard and PSI based discard, UE cannot perform related discard operation if PDU set or PSI cannot be identified. Discard related enhancements can improve the system capacity, but there are no inter-operability issues if discard operations cannot be performed when configured. There is nothing the network or UE can do if the PDU set of PSI information are not available at UE side.

Proposal 2: No additional mechanisms are needed to address the issue that UE cannot perform PDU Set discard or PSI based discard due to that UE cannot identify PDU set or PSI.

Pre-requisites

As from [1], RAN2 will introduce 7 XR related capabilities: xr-AssistanceInfo, discardPDU-Set, discardPSI, supportOfNewBS-Table, supportOfDelayReporting, supportOfEnhancedDRX, supportOfDisableHARQ-RTT-CG. Among them, capabilities xr-AssistanceInfo, discardPDU-Set, discardPSI, supportOfNewBS-Table, supportOfDelayReporting do not have pre-requisites. Capabilities supportOfEnhancedDRX and supportOfDisableHARQ-RTT-CG are related to DRX and CG, and discussion of pre-requisites is needed.

For rational number DRX cycle (supportOfEnhancedDRX), following is the related proposal in [1]:

Proposal 7: A new optional UE capability signaling (e.g., supportOfEnhancedDRX) is defined to identify Rel-18 UEs supporting C-DRX enhancements including the support of non-integer (i.e. rational) DRX periodicity and addressing the SFN wrap around.

From the running MAC CR [3], rational number DRX cycle is related to both long DRX cycle and short DRX cycle, whose capabilities are copied below.

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| longDRX-Cycle Indicates whether UE supports long DRX cycle as specified in TS 38.321 [8]. | UE | Yes | Yes | No |
| secondaryDRX-Group-r16 Indicates whether UE supports secondary DRX group as specified in TS 38.321 [8]. | UE | No | Yes | No |
| shortDRX-Cycle Indicates whether UE supports short DRX cycle as specified in TS 38.321 [8]. | UE | Yes | Yes | No |

Since both longDRX-Cycle and shortDRX-Cycle are mandatory capabilities, there is no need to define the pre-requisite of UE capability supportOfEnhancedDRX. This is inline with the Rel-16 capability secondaryDRX-Group-r16, which does not have pre-requisites either.

For retransmission-less CG (supportOfDisableHARQ-RTT-CG), following is the related proposal in [1]:

Proposal 8: A new optional UE capability signaling (e.g., supportOfDisableHARQ-RTT-CG) is defined to identify UE supporting retransmission-less CG enhancement (which allows disabling the HARQ RTT timer per CG configuration).

Retransmission-less CG (supportOfDisableHARQ-RTT-CG) is related to both DRX and CG. Similar to the discussion of rational number DRX above, there is no need to have (long or short) DRX cycle as pre-requisite. It is expected that UE supporting retransmission-less CG needs to support at least one type of CG (either type 1 or type 2). Following capabilities are defined for configured grants from Rel-16 and forward:

- Configured grant support in licensed band: per band capabilities configuredUL-GrantType1-v1650 and configuredUL-GrantType2-v1650. These capabilities were introduced to replace per UE capabilities configuredUL-GrantType1 and configuredUL-GrantType2 (as from CR R2-2106644).

- Configured grant support in shared spectrum: per UE capabilities configuredUL-GrantType1-r16 and configuredUL-GrantType2-r16.

Retransmission-less CG should have at least one of configured grant capabilities as pre-requisite.

Proposal 3: The UE supporting retransmission-less CG (supportOfDisableHARQ-RTT-CG) shall also support configuredUL-GrantType1-v1650 or configuredUL-GrantType2-v1650 or configuredUL-GrantType1-r16 or configuredUL-GrantType2-r16. Other RAN2 related XR capabilities (xr-AssistanceInfo, discardPDU-Set, discardPSI, supportOfNewBS-Table, supportOfDelayReporting, supportOfEnhancedDRX) do not have pre-requisites.

Conclusion

In this contribution, we discuss the UE capabilities for XR, and have the following observations:

Observation 1: Inter-layer interaction within UE may help UE to obtain information of PDU set / PSI.

We propose the following:

Proposal 1: The reporting of UL traffic information can already handle the dynamic aspects of XR traffic (e.g. when data burst information is not available), and additional enhancements are not needed.

Proposal 2: No additional mechanisms are needed to address the issue that UE cannot perform PDU Set discard or PSI based discard due to that UE cannot identify PDU set or PSI.

Proposal 3: The UE supporting retransmission-less CG (supportOfDisableHARQ-RTT-CG) shall also support configuredUL-GrantType1-v1650 or configuredUL-GrantType2-v1650 or configuredUL-GrantType1-r16 or configuredUL-GrantType2-r16. Other RAN2 related XR capabilities (xr-AssistanceInfo, discardPDU-Set, discardPSI, supportOfNewBS-Table, supportOfDelayReporting, supportOfEnhancedDRX) do not have pre-requisites.

- References

[1] R2-2310148, Intel Corporation, Qualcomm Incorporated, Xiaomi, InterDigital, Nokia, Nokia Shanghai Bell, vivo, OPPO, Huawei, HiSilicon, Ericsson, "Summary on UE Capabilities for Rel-18 XR WI"

[2] R2-2309315, Huawei et al, "Introduction of XR enhancements into TS 38.331 (running CR)"

[3] R2-2309316, Qualcomm, " Introduction of XR enhancements to TS 38.321"