3GPP TSG-RAN WG2 Meeting #123bis	                     		  R2-2309608

Xiamen, China, Oct 9th – 13th, 2023

Source:	CATT

Title:	LPP and RRC impacts to enable Carrier Phase Positioning

Agenda Item:	7.2.5

Document for:	Discussion and Decision

1 Introduction

In RAN1 #112, #112bis-e, #113, and #114 meeting, RAN1 has discussed the carrier phase positioning and the agreements made in RAN1 were included in the LS sent to RAN2 at RAN1 #114 meeting [1]. This contribution will discuss the questions on the LS (Reply LS on PRU Procedures) which is not specified in the LS yet, and the impact of RAN1 agreements to LPP and RRC protocols.

2 Discussion

2.1 Discussion on the questions on LS from RAN1

Missed information in RAN1 LS

RAN1 sent their agreements on CPP in R1-2308644 “Reply LS on PRU Procedures” [1].

We will discuss the missing information, which are not included in RAN1 LS, but are needed for RAN2 to finalize the LPP protocol.

When a UE reports the RSCP/RSCPD together with the legacy measurement, the associated DL PRS resource IDs are expected to be reported. But, there are no clear parameters of RSCP/RSCPD in RRC parameters. Considering RSCPD can only be measured on one PFL while RSTD can be measured across multiple PFLs, it seems that the associated PRS resources of RSCPD can be different from those of the main RSTD measurement (not the additional measurement of RSTD).

So here is the question on the measurement report:

- Question1: Can the associated DL PRS resource ID(s) and resource Set ID(s) of RSCPD be different from the associated DL PRS resource ID(s) of RSTD when UE report these measurements? The same question applies to RSCP and UE Rx-Tx time difference.

Agreement

Rel-17 LOS/NLOS indication (when indicated) applies for the carrier phase measurement(s) in the same report.

Because LOS/NLOS indication applies for carrier phase measurement(s), if the associated resource of RSCPD and RSTD can be different, then there will be different LOS/NLOS indications associated with different resources.

- Question2: If answer of Q1 is yes, does UE provide additional LOS/NLOS indication, which are associated with the RSCPD but not with the RSTD?

For DL-TDOA, UE may provide up to 3 additional RSTD measurements per pair of TRPs. For CPP there is no such parameter in RRC parameter list from RAN1 on additional measurement of RSCP/RSCPD. In our view, RAN2 should ask RAN1 whether there are additional measurements of RSCP and RSCPD in the measurement report.

- Question3: For DL-TDOA, UE may provide up to 3 additional RSTD measurements per pair of TRPs. For CPP, is there any additional RSCP measurement per TRP or additional RSCPD measurement per pair of TRPs?

So we propose to send a reply LS to RAN1 for the measurement report aspect, asking the two questions.

Proposal 1: RAN2 agree to send a reply LS to RAN1 for the questions about measurement report of RSCP/RSCPD:

- Question 1: Can the associated resource ID and resource Set ID in the report of RSCPD be different from the associated resource ID and resource Set ID in RSTD when UE report these measurements? The same question applies to RSCP and UE Rx-Tx time difference report.

- Question 2: If answer of Q1 is yes, is the LOS/NLOS indication associated with the resource of RSCPD also required?

- Question3: For DL-TDOA, UE may provide up to 3 additional RSTD measurements per pair of TRPs. For CPP, is there any additional RSCP measurement per TRP or additional RSCPD measurement per pair of TRPs?

2.2 Measurement report of RSCP/RSCPD

There are impacts on LPP protocol including three aspects:

- New measurement report of RSCP/RSCPD in Multi-RTT/DL-TDOA measurement report.

- New measurement request on RSCP/RSCPD including indicated DL PRS resource set(s) occurring within indicated time window(s)

- Assistance data of PRU CPP info for UE-based positioning

Below please find the agreements on measurement report of RSCP/RSCPD from RAN1.

Agreement

For NR DL reference signal carrier phase difference (RSCPD) measurement for NR CPP, the RSCPD is defined as the difference of RSCPs measured from the DL PRS signals from target TRP and reference TRP.

Agreement

For NR carrier phase positioning, at least support the following approach: enable a UE/TRP to report carrier phase measurements together with the legacy positioning measurements to LMF

Agreement

Rel-17 LOS/NLOS indication (when indicated) applies for the carrier phase measurement(s) in the same report.

Agreement

If a UE reports RSCPD measurements together with RSTD measurements in a measurement report element, the reference TRP for RSCPD is the same as the reference TRP reported for RSTD.

- The target and the reference TRP are in the same PFL

Agreement

Each DL RSCP/RSCPD measurement instance is obtained with Nsample=1 sample only.

Agreement

For the timestamp associated with a reported RSCP/RSCPD measurement, NR-TimeStamp, with the granularity of a slot, currently defined in TS 37.355, can be reused as the timestamp.

Subject to UE capability, a UE may optionally provide an OFDM symbol index in the timestamp.

Note: It is up to RAN2/RAN3 how to signal the timestamp

Agreement

When DL RSCPD/RSCP measurements are reported together with the DL RSTD/ UE Rx – Tx time difference measurements, the DL RSCPD/RSCP measurements are obtained from a single DL PFL only.

Note: From RAN1’s perspective, the reporting of the carrier phase measurements from one DL PFL has no impact on the reporting of the DL RSTD and/or UE Rx – Tx time difference measurements from the same DL PFL or other DL PFLs.

Agreement

Support UE/TRP to report the phase quality indication for the RSCP/RSCPD measurements. The phase quality indication includes the following fields:

- phase quality index

- phase quality resolution

The values of the phase quality index and phase quality resolution are left for RAN4.

The impact of measurement report in LPP can be found as below in NR-DL-TDOA-MeasElement and NR-Multi-RTT-SignalMeasurementInformation.

NR-DL-TDOA-MeasElement-r16 ::= SEQUENCE {

***skip the unchange***

]],

[[

nr-DL-TDOA-CarrierPhaseMeasurements-r18

NR-DL-TDOA-CarrierPhaseMeasurements-r18			OPTIONAL

]]

}

NR-DL-TDOA-CarrierPhaseMeasurements-r18::= SEQUENCE (SIZE (1..FFS)) OF

NR-DL-TDOA-CarrierPhaseMeasurementElement-r18

NR-DL-TDOA-CarrierPhaseMeasurementElement-r18 ::= SEQUENCE {

nr-DL-PRS-ResourceID-RSCPD-r18		NR-DL-PRS-ResourceID-r16			OPTIONAL,

nr-DL-PRS-ResourceSetID-RSCPD-r18	NR-DL-PRS-ResourceSetID-r16			OPTIONAL,

nr-TimeStamp-RSCPD-r18				NR-TimeStamp-r16,

nr-RSCPD-r18						FFS CHOICE {			},

...

} OPTIONAL,

nr-PhaseQuality-r18			NR-PhaseQuality-r18 OPTIONAL,

nr-los-nlos-Indicator-r17			CHOICE {

perTRP-r17						LOS-NLOS-Indicator-r17,

perResource-r17					LOS-NLOS-Indicator-r17

} 																	OPTIONAL,

...

}

Editor Notes:

1.the number of report CarrierPhaseMeasurementElement is FFS, waiting for RAN1 further agreement.

2.the value of RSCPD is FFS, waiting for RAN1 and RAN4 agreement.

nr-TimeStamp-RSCPD This field specifies the time instance at which the RSCPD measurement is performed. The reference TRP for RSCPD is the same as the reference TRP reported for RSTD.

nr-RSCPD This field specifies the NR DL reference carrier phase difference measurement, as defined in TS 38.215 [36]. Mapping of the measured quantity is defined as in TS 38.133 [46]. The target and the reference TRP are in the same PFL.

nr-PhaseQuality This field specifies the target device′s best estimate of the quality of the measurement.

NR-Multi-RTT-CarrierPhaseMeasurements-r18::= SEQUENCE (SIZE (1..FFS)) OF

NR-Multi-RTT-CarrierPhaseMeasurementElement-r18

NR-Multi-RTT-CarrierPhaseMeasurementElement-r18 ::= SEQUENCE {

{

nr-DL-PRS-ResourceID-RSCP-r18		NR-DL-PRS-ResourceID-r16			OPTIONAL,

nr-DL-PRS-ResourceSetID-RSCP-r18			NR-DL-PRS-ResourceSetID-r16			OPTIONAL,

nr-TimeStamp-RSCP-r18						NR-TimeStamp-r16,

nr-RSCP-r18						FFS CHOICE {			},

...

} OPTIONAL,

nr-PhaseQuality-r18			NR-PhaseQuality-r18 OPTIONAL,

nr-los-nlos-Indicator-r17			CHOICE {

perTRP-r17						LOS-NLOS-Indicator-r17,

perResource-r17					LOS-NLOS-Indicator-r17

} 																	OPTIONAL,

...

}

Editor Notes:

1.the number of report CarrierPhaseMeasurementElement is FFS, waiting for RAN1 further agreement.

2.the value of RSCP is FFS waiting for RAN1 further agreement.

Proposal 2: The TP of carrier phase measurement report will be discussed in LPP running CR.

2.3 Indicated time window(s) for measurement request and transmitting SRS

RAN1 indicated that there will be a mechanism to ensure simultaneous measurements/transmissions (e.g. in the same slot(s)) for multiple UEs, including a target UE and a PRU in the LS [1]. There are impacts on simultaneous measurements in LPP and transmissions (e.g. in the same slot(s)) impacts for UEs in RRC separately.

Current RAN1 specifications do not support a mechanism to ensure simultaneous measurements/transmissions (e.g. in the same slot(s)) for multiple UEs, including a target UE and a PRU.

RAN1 will continue discussions on what enhancements to LPP, NRPPa, and/or RAN signaling are necessary to support simultaneous measurements of the same DL-PRS for multiple UEs, including a target UE and a PRU; and to support simultaneous transmission of SRS for multiple UEs, including a target UE and a PRU.

Note: The enhancements might or might not have RAN1 specification impact.

2.3.1 Indicated time window(s) in LPP

The indicated time window(s) is defined by RAN1 to measure the same DL PRS for improvement of positioning accuracy.

Below please find the related agreement from RAN1 which has impact on LPP protocol in request location information message.

Agreement

To enable LMF to request the UEs, including target UE and PRU(s), to perform measurements on indicated DL PRS resource set(s) occurring within indicated time window(s), each time window is defined with the following parameters:

- The start of the time window, which is indicated by a combination of subframe number, slot offset and symbol index

- The duration of the time window, which is given by a number of consecutive slots/symbols

FFS: the number of consecutive slots/symbols

- (Optional) The periodicity of the time window, which is defined similar to IE NR-DL-PRS-Periodicity-and-ResourceSetSlotOffset in TS 37.355.FFS: the maximum number of the windows

Agreement

When an LMF requests the UEs, including target UE and PRU(s), to perform measurements on indicated DL PRS resource set(s) occurring within indicated time window(s)

- The duration of a time window can be configured as follows:

- {1, 2, 4, 6, 8, 12, 16} slots.

- the number of the time windows can be:

- {1, 2}

- FFS: {4, 8}

So a common IE for the IndicatedResourceSet-and-TimeWindow will be introduced including below features:

- Start of the time window based on each TRP, including the subframe number, slot offset and symbol index

- Periodicity of time window

- Duration of the time window

- Numbers of the time window

–	NR-IndicatedResourceSet-and-TimeWindow

The IE NR-IndicatedResourceSet-and-TimeWindow provides a set of indicated DL PRS resource set(s) occurring within indicated time window(s) which is configured from server to target UE to perform carrier phase measurements.

-- ASN1START

NR-IndicatedResourceSet-and-TimeWindow-r18  ::= SEQUENCE {

nr-IndicatedResourceSet-and-TimeWindow-List-r18		SEQUENCE (SIZE (1.. nrMaxSetsPerTrpPerFreqLayer-r16)) OF NR-IndicatedResourceSetTimeWindow-r18					OPTIONAL, -- Need ON

}

NR-IndicatedResourceSetTimeWindow-r18 ::= SEQUENCE {

nr-IndicatedResourceSetID-r18	NR-DL-PRS-ResourceSetID-r16		OPTIONAL, -- Need ON

nr-StartSFN-TimeWindow-r18		INTEGER (0..1023) 				OPTIONAL, -- Need ON

nr-Periodicity-and-SlotOffsetTimeWindow-r18			NR-DL-PRS-Periodicity-and-SlotOffset-r16																OPTIONAL, -- Need ON

nr-SymbolOffsetTimeWindow-r18	INTEGER (0..12) 					OPTIONAL, -- Need ON

nr-durationTimeWindow-r18		ENUMERATED { n1, n2, n4, n6, n8, n12, n16, ... } 																					OPTIONAL, -- Need ON

nr-numberTimeWindow-r18		ENUMERATED { n1, n2, FFS4, FFS8}	OPTIONAL, -- Need ON

...

}

-- ASN1STOP

Proposal 3: The TP of carrier phase measurement request will be discussed in LPP running CR.

2.3.2 Indicated time window(s) for transmission of SRS

Below please find the related agreement from RAN1 which has impact on RRC protocol to enable UE and PRU to send the SRS in the indicated time window(s) from LMF to gNB. However it seems that gNB can configure proper SRS according to the requested transmission time windoe from LMF to gNB, instead of configuring time window to UE again. It’s up to gNB implementation to configure SRS for UEs.

Agreement

Confirm the following working assumption with modification made in RAN1#113:

Working assumption

To enable LMF to optionally request the serving gNB of a UE to configure the transmission of the UL positioning SRS resources from the UE within indicated time window(s), support:

- Option 1D: Each of the time windows is defined with the following parameters:

  - The start of the time window, which is indicated by a combination of system frame number, slot offset and symbol index with respect to the SFN initialization time

  - The duration of the time window, which is given by a number of consecutive slots/symbols

    - FFS: the number of the consecutive slots/symbols

  - (Optional) The periodicity of the time window, which is defined similar to IE PeriodicitySRS in “Requested SRS Transmission Characteristics” in TS 38.455.

- FFS: the maximum number of the windows

Agreement

Confirm the following working assumption with modification made in RAN1#113:

Working assumption

To enable LMF to optionally request the serving gNB of a UE to configure the transmission of the UL positioning SRS resources from the UE within indicated time window(s), support:

- Option 1D: Each of the time windows is defined with the following parameters:

  - The start of the time window, which is indicated by a combination of system frame number, slot offset and symbol index with respect to the SFN initialization time

  - The duration of the time window, which is given by a number of consecutive slots/symbols

    - FFS: the number of the consecutive slots/symbols

  - (Optional) The periodicity of the time window, which is defined similar to IE PeriodicitySRS in “Requested SRS Transmission Characteristics” in TS 38.455.

- FFS: the maximum number of the windows

Proposal 4:RAN2 to discuss if it’s up to gNB implementation to configure proper SRS to UEs according to the request from LMF without any RRC impact.

2.4 Assistance Data of PRU for UE-based positioning

The carrier phase measurement of PRU will improve the accuracy when the location is calculated. So it is agreed in RAN1 that the DL carrier phase measurement reported by a PRU will be provided to target UE when UE-based carrier phase positioning, which is different from other positioning methods. Below please find the related agreement in RAN1.

Agreement

For UE-based carrier phase positioning, when LMF forwards the DL carrier phase measurement reported by a PRU to a target UE, the timestamp associated with the PRU carrier phase measurements should also be forwarded in positioning assistance data.

Agreement

For UE-based carrier phase positioning, support enabling LMF to forward the DL carrier phase measurement reported by a PRU, with additional information of the same PRU to a target UE for UE-based carrier phase positioning in the positioning assistance data.

- Note: Whether the forwarded DL carrier phase measurement is DL RSCP and/or DL RSCPD depends at least on which of them is (are) supported by UE capability.

- additional information of the same PRU includes at least PRU location.

FFS: additional PRU information, e.g. the AoD of PRU to each TRP, etc.

A Common IE of NR-PRU-DL-Info will be introduced accordingly.

#### –	NR-PRU-DL-Info

The IE NR-PRU-DL-Info is used by the location server to provide DL carrier phase measurement information reported by a PRU, with additional information of this PRU to a target UE.

-- ASN1START

NR-PRU-DL-Info-r18 ::= SEQUENCE (SIZE (1..maxMeasInstances-r17)) OF

NR-PRU-DL-MeasurementInformation-r18

NR-PRU-DL-MeasurementInformation-r18 ::= SEQUENCE {

dl-PRS-ReferenceInfo-r16		DL-PRS-ID-Info-r16,

nr-PRU-DL-MeasList-r18 			NR-PRU-DL-MeasList-r18,

...

}

Proposal 5: The TP of carrier phase measurement of PRU from LMF to UE will be discussed in LPP running CR.

2.5 Support of CPP in RRC_INACITVE mode

UE also can measure carrier phase in RRC_INACTIVE state according to RAN1 agreement as below,

Agreement

Support the reuse of existing physical layer procedures for DL positioning (e.g., DL-TDOA) with the necessary enhancements in measurement configuration, request and report (e.g., adding the configuration related to the NR DL CPP) for both UE-based and UE-assisted NR DL carrier phase positioning, including

•	UE in RRC_CONNECTED state with measurement gap.

•	FFS: UE in RRC_CONNECTED state without measurement gap

•	UE in RRC_ INACTIVE state

The start of the indicated time window which is associated with DL-PRS ResourceSet is sfn according to RAN1 LS.

Agreement

To enable LMF to request the UEs, including target UE and PRU(s), to perform measurements on indicated DL PRS resource set(s) occurring within indicated time window(s), each time window is defined with the following parameters:

- The start of the time window, which is indicated by a combination of system frame number, slot offset and symbol index

- The duration of the time window, which is given by a number of consecutive slots/symbols

FFS: the number of consecutive slots/symbols

- (Optional) The periodicity of the time window, which is defined similar to IE NR-DL-PRS-Periodicity-and-ResourceSetSlotOffset in TS 37.355.FFS: the maximum number of the windows

The start of the time window for CPP can be provided to UE in RequestLocationInformation before UE enters to RRC_INACTIVE. It’s up to LMF to set the proper sfn of time window for the UEs.

In the figure, the DL-PRS is assumed to be transmitted with the periodicity of one SF, and the time window with the periodicity of two SFs. In this case, the start time of the window can be the same as the start time of the first DL-PRS transmission time, and can also be the start time of the send DL-PRS transmission time as shown in the figure indicated by the SFN and the slot number as in RAN1’s agreement. The time window can be configured with the start time, duration, and period, regardless of the RRC status (RRC_CONNECTED, RRC_IDLE or RRC_INACTIVE).

Proposal 6: The time window can be configured with the start time as sfn, duration, and period, regardless of the RRC status (RRC_CONNECTED, RRC_IDLE or RRC_INACTIVE).

3 Conclusions

In this contribution, we have discusses the LPP impact and TPs based on the RAN1 progress on carrier phase positioning.

Proposal 1: RAN2 agree to send a reply LS to RAN1 for the questions about measurement report of RSCP/RSCPD:

- Question 1: Can the associated resource ID and resource Set ID in the report of RSCPD be different from the associated resource ID and resource Set ID in RSTD when UE report these measurements? The same question applies to RSCP and UE Rx-Tx time difference report.

- Question 2: If answer of Q1 is yes, is the LOS/NLOS indication associated with the resource of RSCPD also required?

- Question3: For DL-TDOA, UE may provide up to 3 additional RSTD measurements per pair of TRPs. For CPP, is there any additional RSCP measurement per TRP or additional RSCPD measurement per pair of TRPs?

Proposal 2: The TP of carrier phase measurement report will be discussed in LPP running CR.

Proposal 3: The TP of carrier phase measurement request will be discussed in LPP running CR.

Proposal 4:RAN2 to discuss if it’s up to gNB implementation to configure proper SRS to UEs according to the request from LMF without any RRC impact.

Proposal 5: The TP of carrier phase measurement of PRU from LMF to UE will be discussed in LPP running CR.

Proposal 6: The time window can be configured with the start time as sfn, duration, and period, regardless of the RRC status (RRC_CONNECTED, RRC_IDLE or RRC_INACTIVE).

4 Reference

- R1-2308644 Reply LS on PRU Procedures, RAN1, CATT

- R1-2308545 Session notes for 9.5 (Expanded and Improved NR Positioning) Ad-Hoc Chair (Huawei) RAN1#114

- R1-2306144 Session notes for 9.5 (Study on expanded and improved NR positioning) Ad-Hoc Chair (Huawei) RAN1#113

- R1-2304170 Session notes for 9.5 (Study on expanded and improved NR positioning) Ad-Hoc Chair (Huawei) RAN1#112-bis-e

- 3GPP TS 37.355 LTE Positioning Protocol (LPP) (Release 17) V17.5.0