3GPP TSG-RAN WG2 Meeting #123bis	R2-2309609

Xiamen, China, October 9th – 13th, 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.355 | CR | 0466 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of existing SSR IEs in A-GNSS for BDS system |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, CAICT, CMCC, China Telecom, China Unicom, Huawei, ZTE Corporation, MediaTek Inc., OPPO, xiaomi, vivo, Spreadtrum |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh-Core |  |  |  |  |  | Date: |  |  | 2023-09-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification。 of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The exsiting SSR IEs GNSS-SSR-OrbitCorrections, GNSS-SSR-ClockCorrections are not completed for BDS SSR. The existing SSR related IEs are defined based on RTCM SSR information which doesn’t include all the SSR information for BDS, so the content in BDS Precise Point Positioning(PPP) Service Signal PPP-B2b (which including SSR information) should be referenced. According to BDS PPP-B2b ICD, two additional parameters IODN and IOD corr should be included in GNSS-SSR-OrbitCorrections and GNSS-SSR-ClockCorrections to let UE to check the correction information issues. So it is proposed to add these two parameters GNSS-SSR-OrbitCorrections and GNSS-SSR-ClockCorrections in 37.355. Impact analysis Architecture options LTE, NR SA, NSA Impacted functionality: Positioning assistance data transfer of SSR Inter-operability: If only the network is implemented according to the CR and the UE is not, no interoperability problems are foreseen. If only the UE is implemented according to the CR and the network is not, no interoperability problems are foreseen. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - ICD specification of BDS PPP-B2b is added in section 2 as reference. - Two additional parameters IODN and IOD corr are included in GNSS-SSR-OrbitCorrections and GNSS-SSR-ClockCorrections according to PPP-B2b ICD. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | SSR for A-BDS is not completed which may degrade the positioning accuracy if the issues of related data are not aligned. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 6.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of change

GNSS-SSR-ClockCorrections 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 36.305: "Stage 2 functional specification of User Equipment (UE) positioning in E-UTRAN".

[3]	3GPP TS 23.271: "Functional stage 2 description of Location Services (LCS)".

[4]	IS-GPS-200, Revision D, Navstar GPS Space Segment/Navigation User Interfaces, March 7th, 2006.

[5]	IS-GPS-705, Navstar GPS Space Segment/User Segment L5 Interfaces, September 22, 2005.

[6]	IS-GPS-800, Navstar GPS Space Segment/User Segment L1C Interfaces, September 4, 2008.

[7]	IS-QZSS, Quasi Zenith Satellite System Navigation Service Interface Specifications for QZSS, Ver.1.1, July 31, 2009.

[8]	Galileo OS Signal in Space ICD (OS SIS ICD), Issue 1.2, February 2014, European Union.

[9]	Global Navigation Satellite System GLONASS Interface Control Document, Version 5.1, 2008.

[10]	Specification for the Wide Area Augmentation System (WAAS), US Department of Transportation, Federal Aviation Administration, DTFA01-96-C-00025, 2001.

[11]	RTCM-SC104, RTCM Recommended Standards for Differential GNSS Service (v.2.3), August 20, 2001.

[12]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); "Radio Resource Control (RRC); Protocol specification".

[13]	3GPP TS 25.331: "Radio Resource Control (RRC); Protocol Specification".

[14]	3GPP TS 44.031: "Location Services (LCS); Mobile Station (MS) - Serving Mobile Location Centre (SMLC) Radio Resource LCS Protocol (RRLP)".

[15]	3GPP TS 23.032: "Universal Geographical Area Description (GAD)".

[16]	3GPP TS 36.211: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical Channels and Modulation".

[17]	3GPP TS 36.214: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer – Measurements".

[18]	3GPP TS 36.133: "Evolved Universal Terrestrial Radio Access (E-UTRA); Requirements for support of radio resource management".

[19]	3GPP TS 23.003: "Numbering, addressing and identification".

[20]	OMA-TS-LPPe-V1_0, LPP Extensions Specification, Open Mobile Alliance.

[21]	3GPP TS 36.101: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception".

[22]	ITU-T Recommendation X.691 (07/2002) "Information technology - ASN.1 encoding rules: Specification of Packed Encoding Rules (PER)" (Same as the ISO/IEC International Standard 8825-2).

[23]	BDS-SIS-ICD-B1I-3.0: "BeiDou Navigation Satellite System Signal In Space Interface Control Document Open Service Signal B1I (Version 3.0)", February, 2019.

[24]	ATIS-0500027: "Recommendations for Establishing Wide Scale Indoor Location Performance", May 2015.

[25]	Bluetooth Special Interest Group: "Bluetooth Core Specification v4.2", December 2014.

[26]	IEEE 802.11, Part 11: "Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications".

[27]	IETF RFC 6225, "Dynamic Host Configuration Protocol Options for Coordinate-Based Location Configuration Information", July 2011.

[28]	3GPP TS 36.213: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer procedures".

[29]	"Earth Gravitational Model 96 (EGM96)", National Geospatial-Intelligence Agency, NASA.

[30]	RTCM Standard 10403.3: "Differential GNSS (Global Navigation Satellite Systems) Services" – Version 3, October 7, 2016.

[31]	IGS ANTEX: "The Antenna Exchanged Format" – version 1.4, September 15, 2010.

[32]	Federal Information Processing Standards Publication 197, "Specification for the ADVANCED ENCRYPTION STANDARD (AES)", November 26, 2001.

[33]	NIST Special Publication 800-38A, "Recommendation for Block Cipher Modes of Operation Methods and Techniques", 2001.

[34]	3GPP TS 38.101-2: "NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone".

[35]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[36]	3GPP TS 38.215: "NR; Physical layer measurements".

[37]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[38]	IRNSS Signal-In-Space (SPS) Interface Control Document (ICD) for standard positioning service version 1.1, Aug 2017.

[39]	BDS-SIS-ICD-B1C-1.0: "BeiDou Navigation Satellite System Signal In Space Interface Control Document Open Service Signal B1C (Version 1.0)", December, 2017.

[40]	3GPP TS 38.305: "NG Radio Access Network (NG-RAN); Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

[41]	3GPP TS 38.211: "3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Physical channels and modulation".

[42]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[43]	IS-QZSS-L6-001, Quasi-Zenith Satellite System Interface Specification – Centimetre Level Augmentation Service, Cabinet Office, November 5, 2018.

[44]	3GPP TR 38.901: "Technical Specification Group Radio Access Network; Study on channel model for frequencies from 0.5 to 100 GHz".

[45]	3GPP TS 38.214: "NR; Physical layer procedures for data".

[46]	3GPP TS 38.133: "NR; Requirements for support of radio resource management".

[47]	3GPP TS 38.300: "NR; NR and NG-RAN Overall Description; Stage 2".

[48]	3GPP TS 38.213: "NR; Physical layer procedures for control".

[49]	BDS-SIS-ICD-B2a-1.0: "BeiDou Navigation Satellite System Signal In Space Interface Control Document Open Service Signal B2a (Version 1.0)", December, 2017.

[50]	BDS-SIS-ICD-B3I-1.0: "BeiDou Navigation Satellite System Signal In Space Interface Control Document Open Service Signal B3I (Version 1.0)", February, 2018.

[51]	NMEA standard 0183, Version 4.11, November 2018.

[XX]	BDS-SIS-ICD-PPP-B2b-1.0: "BeiDou Navigation Satellite System Signal In Space Interface Control Document Precise Point Positioning Service Signal PPP-B2b (Version 1.0)", July, 2020.

The next change

–	GNSS-SSR-OrbitCorrections

The IE GNSS-SSR-OrbitCorrections is used by the location server to provide radial, along-track and cross-track orbit corrections together with integrity information. The target device may use the SSR-OrbitCorrectionList to compute a satellite position correction to be combined with the satellite position calculated from broadcast ephemeris.

The parameters provided in IE GNSS-SSR-OrbitCorrections – except for ORBIT-IntegrityParameters and SSR-IntegrityOrbitBounds –  are used as specified for SSR Clock Messages (e.g., message type 1057 and 1063) in [30] and apply to all GNSSs.

-- ASN1START

GNSS-SSR-OrbitCorrections-r15 ::= SEQUENCE {

epochTime-r15						GNSS-SystemTime,

ssrUpdateInterval-r15				INTEGER (0..15),

satelliteReferenceDatum-r15			ENUMERATED { itrf, regional, ... },

iod-ssr-r15							INTEGER (0..15),

ssr-OrbitCorrectionList-r15			SSR-OrbitCorrectionList-r15,

...,

[[

orbit-IntegrityParameters-r17	ORBIT-IntegrityParameters-r17	OPTIONAL -- Need OR

]]

}

SSR-OrbitCorrectionList-r15 ::= SEQUENCE (SIZE(1..64)) OF SSR-OrbitCorrectionSatelliteElement-r15

SSR-OrbitCorrectionSatelliteElement-r15 ::= SEQUENCE {

svID-r15							SV-ID,

iod-r15								BIT STRING (SIZE(11)),

delta-radial-r15					INTEGER (-2097152..2097151),

delta-AlongTrack-r15				INTEGER (-524288..524287),

delta-CrossTrack-r15				INTEGER (-524288..524287),

dot-delta-radial-r15				INTEGER (-1048576..1048575)		OPTIONAL, -- Need ON

dot-delta-AlongTrack-r15			INTEGER (-262144..262143)		OPTIONAL, -- Need ON

dot-delta-CrossTrack-r15			INTEGER (-262144..262143)		OPTIONAL, -- Need ON

...,

[[

ssr-IntegrityOrbitBounds-r17	SSR-IntegrityOrbitBounds-r17	OPTIONAL, -- Cond Integrity1

iodn-r17						BIT STRING (SIZE(10))			OPTIONAL, -- Cond BDS

iod-Corr-r17					INTEGER (0..7)					OPTIONAL  -- Cond BDS

]]

}

ORBIT-IntegrityParameters-r17 ::= SEQUENCE {

probOnsetConstFault-r17					INTEGER (0..255),

meanConstFaultDuration-r17				INTEGER (1..3600),

probOnsetSatFault-r17					INTEGER (0..255),

meanSatFaultDuration-r17				INTEGER (1..3600),

orbitRangeErrorCorrelationTime-r17		INTEGER (0..255)			OPTIONAL, -- Need OR

orbitRangeRateErrorCorrelationTime-r17	INTEGER (0..255)			OPTIONAL, -- Cond Integrity2

...

}

SSR-IntegrityOrbitBounds-r17 ::= SEQUENCE {

meanOrbitError-r17				RAC-OrbitalErrorComponents-r17,

stdDevOrbitError-r17			RAC-OrbitalErrorComponents-r17,

meanOrbitRateError-r17			RAC-OrbitalErrorComponents-r17,

stdDevOrbitRateError-r17		RAC-OrbitalErrorComponents-r17,

...

}

RAC-OrbitalErrorComponents-r17	::= SEQUENCE {

radial-r17			INTEGER (0..255),

alongTrack-r17		INTEGER (0..255),

crossTrack-r17		INTEGER (0..255)

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| Integrity1 | The field is mandatory present if ORBIT-IntegrityParameters is present; otherwise it is not present. |
| Integrity2 | The field is mandatory present if orbitRangeErrorCorrelationTime is present; otherwise it is not present. |
| BDS | The field is optionally present, Need ON, for BDS only; otherwise it is not present. |

GNSS-SSR-OrbitCorrections field descriptions

epochTime This field specifies the epoch time of the orbit corrections. The gnss-TimeID in GNSS-SystemTime shall be the same as the GNSS-ID in IE GNSS-GenericAssistDataElement.

ssrUpdateInterval This field specifies the SSR Update Interval. The SSR Update Intervals for all SSR parameters start at time 00:00:00 of the GPS time scale. A change of the SSR Update Interval during the transmission of SSR data should ensure consistent data for a target device. See table Value of ssrUpdateInterval to SSR Update Interval relation below. NOTE 1.

satelliteReferenceDatum This field specifies the satellite refence datum for the orbit corrections.

iod-ssr This field specifies the Issue of Data number for the SSR data. A change of iod-ssr is used to indicate a change in the SSR generating configuration.

iodn This field specifies the issue number of satellite clock and ephemeris broadcast by the downlink signals of GNSS as specified in [XX].

iod-Corr This field specifies the issue number of the orbit correction and the clock correction as specified in [XX].

svID This field specifies the satellite for which the orbit corrections are provided.

iod This field specifies the IOD value of the broadcast ephemeris for which the orbit corrections are valid (see IE GNSS-NavigationModel). NOTE 2.

delta-radial This field specifies the radial orbit correction for broadcast ephemeris. NOTE 3. Scale factor 0.1 mm; range ±209.7151 m.

delta-AlongTrack This field specifies the along-track orbit correction for broadcast ephemeris. NOTE 3. Scale factor 0.4 mm; range ±209.7148 m.

delta-CrossTrack This field specifies the cross-track orbit correction for broadcast ephemeris. NOTE 3. Scale factor 0.4 mm; range ±209.7148 m.

dot-delta-radial This field specifies the velocity of radial orbit correction for broadcast ephemeris. NOTE 3. Scale factor 0.001 mm/s; range ±1.048575 m/s.

dot-delta-AlongTrack This field specifies the velocity of along-track orbit correction for broadcast ephemeris. NOTE 3. Scale factor 0.004 mm/s; range ±1.048572 m/s.

dot-delta-CrossTrack This field specifies the velocity of cross-track orbit correction for broadcast ephemeris. NOTE 3. Scale factor 0.004 mm/s; range ±1.048572 m/s.

probOnsetConstFault This field specifies the Probability of Onset of Constellation Fault per Time Unit where a constellation fault is at least two satellites being faulty simultaneously due to the same event. This field specifies the onset probability that the residual range or range rate error exceeds a bound created using the minimum allowed inflation factor Kmin, and bounding parameters as mean + Kmin * stdDev where Kmin = normInv(irMaximum / 2), with irMaximum as provided in IE GNSS-Integrity-ServiceParameters. The probability is calculated by P=10-0.04n [hour-1] where n is the value of probOnsetConstFault and the range is 10-10.2 to 1 per hour.

meanConstFaultDuration This field specifies the Mean Constellation Fault Duration which is the mean duration between when a constellation fault occurs, and the user is alerted by IE GNSS-RealTimeIntegrity (or the integrity violation is over). Scale factor 1 s; range 1-3600 s.

probOnsetSatFault This field specifies the Probability of Onset of Satellite Fault per Time Unit which is the probability of occurrence of satellite error to exceed the residual error bound for more than the Time to Alert (TTA). This field specifies the onset probability that the residual range or range rate error exceeds a bound created using the minimum allowed inflation factor Kmin, and bounding parameters as mean + Kmin * stdDev where Kmin = normInv(irMaximum / 2), with irMaximum as provided in IE GNSS-Integrity-ServiceParameters. The probability is calculated by P=10-0.04n [hour-1] where n is the value of probOnsetSatFault and the range is 10-10.2 to 1 per hour.

meanSatFaultDuration This field specifies the Mean Satellite Fault Duration which is the mean duration between when a satellite fault occurs, and the user is alerted by IE GNSS-RealTimeIntegrity (or the integrity violation is over). Scale factor 1 s; range 1-3,600 s.

orbitRangeErrorCorrelationTime This field specifies the Orbit Range Error Correlation Time which is the upper bound of the correlation time of the satellite residual range error due to orbit. The time is calculated using: t=10i, &i≤1801800+100(i-180), 180<&i≤234 7200+1000i-234, &i>234 [s] Range is 1-28,200 s.

orbitRangeRateErrorCorrelationTime This field specifies the Orbit Range Rate Error Correlation Time which is the upper bound of the correlation time of the satellite residual range rate error due to orbit. The time is calculated using: t=10i, &i≤1801800+100(i-180), 180<&i≤234 7200+1000i-234, &i>234 [s] Range is 1-28,200 s.

meanOrbitError This field specifies the Mean Orbit Error bound in satellite radial, along-track and cross-track coordinates, which are the mean values for a set of three overbounding models that bound the residual orbit error in satellite radial, along-track and cross-track directions. Each mean is calculated using: μ=0.01i, &i≤2002+0.1(i-200), 200<&i≤230 5+0.5i-230, &i>230 [m] Range is 0-17.5 m.

stdDevOrbitError This field specifies the Standard Deviation Orbit Error bound in satellite radial, along-track and cross-track coordinates, which are the standard deviation values for a set of three overbounding models that bound the residual orbit error in satellite radial, along-track and cross-track directions. Each standard deviation is calculated using: σ=0.01i, &i≤2002+0.1(i-200), 200<&i≤230 5+0.5i-230, &i>230 [m] Range is 0-17.5 m.

meanOrbitRateError This field specifies the Mean Orbit Rate Error in satellite radial, along-track and cross-track coordinates, which are the mean values for a set of three overbounding models that bound the residual satellite orbit rate error in satellite radial, along-track and cross-track directions. Scale factor 0.001 m/s; range 0-0.255 m/s.

stdDevOrbitRateError This field specifies the Standard Deviation Orbit Rate Error in satellite radial, along-track and cross-track coordinates, which are the standard deviation values for a set of three overbounding models that bound the residual satellite orbit rate error in satellite radial, along-track and cross-track directions. Scale factor 0.001 m/s; range 0-0.255 m/s.

NOTE 1:	The update intervals are aligned to the GPS time scale for all GNSSs in order to allow synchronous operation for multiple GNSS services. This means that the update intervals may not be aligned to the beginning of the day for another GNSS. Due to the leap seconds, this is generally the case for GLONASS.

NOTE 2:	In the cases that gnss-ID indicates 'gps' or 'qzss', the iod refers to the NAV broadcast ephemeris (GPS L1 C/A or QZSS QZS-L1, respectively, in table GNSS to iod Bit String(11) relation in IE GNSS-NavigationModel).

NOTE 3:	The reference time t0 is epochTime + ½ × ssrUpdateInterval. The reference time t0 for ssrUpdateInterval '0' is epochTime.

Value of ssrUpdateInterval to SSR Update Interval relation

| Value of ssrUpdateInterval | SSR Update Interval |
| --- | --- |
| 0 | 1 second |
| 1 | 2 seconds |
| 2 | 5 seconds |
| 3 | 10 seconds |
| 4 | 15 seconds |
| 5 | 30 seconds |
| 6 | 60 seconds |
| 7 | 120 seconds |
| 8 | 240 seconds |
| 9 | 300 seconds |
| 10 | 600 seconds |
| 11 | 900 seconds |
| 12 | 1800 seconds |
| 13 | 3600 seconds |
| 14 | 7200 seconds |
| 15 | 10800 seconds |

–	GNSS-SSR-ClockCorrections

The IE GNSS-SSR-ClockCorrections is used by the location server to provide clock correction parameters together with integrity information. The target device may use the SSR-ClockCorrectionList to compute a clock correction to be applied to the broadcast satellite clock parameters, identified by iod of corresponding GNSS-SSR-OrbitCorrections.

The parameters provided in IE GNSS-SSR-ClockCorrections – except for CLOCK-IntegrityParameters and SSR-IntegrityClockBounds – are used as specified for SSR Clock Messages (e.g., message type 1058 and 1064) in [30] and apply to all GNSSs.

-- ASN1START

GNSS-SSR-ClockCorrections-r15 ::= SEQUENCE {

epochTime-r15						GNSS-SystemTime,

ssrUpdateInterval-r15				INTEGER (0..15),

iod-ssr-r15							INTEGER (0..15),

ssr-ClockCorrectionList-r15			SSR-ClockCorrectionList-r15,

...,

[[

clock-IntegrityParameters-r17	CLOCK-IntegrityParameters-r17		OPTIONAL   -- Need OR

]]

}

SSR-ClockCorrectionList-r15 ::= SEQUENCE (SIZE(1..64)) OF SSR-ClockCorrectionSatelliteElement-r15

SSR-ClockCorrectionSatelliteElement-r15 ::= SEQUENCE {

svID-r15							SV-ID,

delta-Clock-C0-r15					INTEGER (-2097152..2097151),

delta-Clock-C1-r15					INTEGER (-1048576..1048575)			OPTIONAL, -- Need ON

delta-Clock-C2-r15					INTEGER (-67108864..67108863)		OPTIONAL, -- Need ON

...,

[[

ssr-IntegrityClockBounds-r17	SSR-IntegrityClockBounds-r17		OPTIONAL, -- Need OR

iod-Corr-r17					INTEGER (0..7)						OPTIONAL  –- Cond BDS

]]

}

CLOCK-IntegrityParameters-r17 ::= SEQUENCE {

clockRangeErrorCorrelationTime-r17		INTEGER (0..255),

clockRangeRateErrorCorrelationTime-r17	INTEGER (0..255),

...

}

SSR-IntegrityClockBounds-r17 ::= SEQUENCE {

meanClock-r17							INTEGER (0..255),

stdDevClock-r17							INTEGER (0..255),

meanClockRate-r17						INTEGER (0..255),

stdDevClockRate-r17						INTEGER (0..255),

...

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| BDS | The field is optionally present, Need ON, for BDS only; otherwise it is not present. |

GNSS-SSR-ClockCorrections field descriptions

epochTime This field specifies the epoch time of the clock corrections. The gnss-TimeID in GNSS-SystemTime shall be the same as the GNSS-ID in IE GNSS-GenericAssistDataElement.

ssrUpdateInterval This field specifies the SSR Update Interval. The SSR Update Intervals for all SSR parameters start at time 00:00:00 of the GPS time scale. A change of the SSR Update Interval during the transmission of SSR data should ensure consistent data for a target device. See table Value of ssrUpdateInterval to SSR Update Interval relation in IE GNSS-SSR-OrbitCorrections.

iod-ssr This field specifies the Issue of Data number for the SSR data. A change of iod-ssr is used to indicate a change in the SSR generating configuration.

iod-Corr This field specifies the issue number of the orbit correction and the clock correction as specified in [XX].

svID This field specifies the satellite for which the clock corrections are provided.

delta-Clock-C0 This field specifies the C0 polynomial coefficient for correction of broadcast satellite clock. NOTE 1. Scale factor 0.1 mm; range ±209.7151 m.

delta-Clock-C1 This field specifies the C1 polynomial coefficient for correction of broadcast satellite clock. NOTE 1. Scale factor 0.001 mm/s; range ±1.048575 m/s.

delta-Clock-C2 This field specifies the C2 polynomial coefficient for correction of broadcast satellite clock. NOTE 1. Scale factor 0.00002 mm/s2; range ±1.34217726 m/s2.

clockRangeErrorCorrelationTime This field specifies the Clock Range Error Correlation Time which is the upper bound of the correlation time of the satellite residual range error due to clock. The time is calculated using: t=10i, &i≤1801800+100(i-180), 180<&i≤234 7200+1000i-234, &i>234 [s] Range is 1-28,200 s.

clockRangeRateErrorCorrelationTime This field specifies the Clock Range Rate Error Correlation Time which is the upper bound of the correlation time of the satellite residual range rate error due to clock. The time is calculated using: t=10i, &i≤1801800+100(i-180), 180<&i≤234 7200+1000i-234, &i>234 [s] Range is 1-28,200 s.

meanClock This field specifies the Mean Clock Error bound which is the mean value for an overbounding model that bounds the residual clock error. The bound is meanClock + K * stdDevClock and shall be so that the probability of it to be exceeded shall be lower than IRallocation for irMinimum < IRallocation < irMaximum, where K = normInv(IRallocation / 2) and irMinimum, irMaximum as provided in IE GNSS-Integrity-ServiceParameters. This IRallocation is a fraction of the Target Integrity Risk that represents the integrity risk budget available. The mean is calculated using: μ=0.01i, &i≤2002+0.1(i-200), 200<&i≤230 5+0.5i-230, &i>230 [m] Range is 0-17.5 m.

stdDevClock This field specifies the Standard Deviation Clock Error bound which is the standard deviation for an overbounding model that bounds the residual clock error. The standard deviation is calculated using: σ=0.01i, &i≤2002+0.1(i-200), 200<&i≤230 5+0.5i-230, &i>230 [m] Range is 0-17.5 m.

meanClockRate This field specifies the Mean Clock Rate Error bound which is the mean value for an overbounding model that bounds the residual clock rate error. The bound is meanClockRate + K * stdDevClockRate and shall be so that the probability of it to be exceeded shall be lower than IRallocation for irMinimum < IRallocation < irMaximum, where K = normInv(IRallocation / 2) and irMinimum, irMaximum as provided in IE GNSS-Integrity-ServiceParameters. This IRallocation is a fraction of the Target Integrity Risk that represents the integrity risk budget available. Scale factor 0.001 m/s; range 0.000-0.255 m/s.

stdDevClockRate This field specifies the Standard Deviation Clock Rate Error bound which is the standard deviation for an overbounding model that bounds the residual clock rate error. Scale factor 0.001 m/s; range 0.000-0.255 m/s.

NOTE 1:	The reference time t0 is epochTime + ½ × ssrUpdateInterval. The reference time t0 for ssrUpdateInterval '0' is epochTime.

The end