3GPP TSG-RAN WG2 Meeting #123bis                                                   R2-2309615

Xiamen, China, 9th - 13th Oct, 2023

Source:              CATT

Title:	             Remaining Issues on Multi-path

Agenda Item:	7.9.4

Document for:	Discussion and Decision

Introduction

In this contribution, the key remaining issues of multi-path will be further discussed.

Discussion

Issues specific for scenario 1

Indirect path release

Regarding to the indirect path release, one open issue is that whether the PC5-RRC connection is immediately released after the indirect path release or it can be maintained before buffered data on the indirect path is delivered.

In our understanding, similar as the indirect path to direct path switching in Rel-17 U2N relay, the timing to execute the PC5-RRC link release is up to UE implementation.

Proposal 1: For scenario 1, for indirect path release, the timing to release the PC5-RRC connection can be left to UE implementation.

Measurement event for indirect path change

For indirect path to indirect path switching for U2N Relay, event Z1(Serving L2 U2N Relay UE becomes worse than threshold1 and Candidate L2 U2N Relay UE becomes better than threshold2) is introduced. It is obvious that this new event can also be applied to indirect path change for multi-path scenario 1.

Proposal 2: For scenario 1, event Z1 can be used for indirect path change of multi-path.

Relay UE’s state transition

In RAN2#121 meeting, the following agreement was reached:

For bringing the idle/inactive relay UE to RRC_CONNECTED, the legacy Rel-17 behaviour (Alt 1 in the proposal) is not disabled for indirect path addition when split SRB1 is configured.  A PC5-RRC trigger is specified at least for other cases.

And in RAN2#121 bis meeting, the following agreements were reached:

When split SRB1 with duplication is configured, the remote UE sends the RRCReconfigurationComplete message to gNB via both paths for Scenario 1.

When one of the following conditions is met, the remote UE sends the RRCReconfigurationComplete message to gNB via the direct path for Scenario 1. FFS on need for additional condition.

-	when primary RLC entity of split SRB1 is on direct path

-	when non-split SRB1 is configured on direct path

Based on the above, it is obvious both the Rel-17 behavior and new PC5-RRC signaling are supported. Hence, it should further make clear when the Remote UE uses the Rel-17 behavior and when the Remote UE uses the new PC5-RRC signaling. Based on the agreements reached on RAN2#121bis, it is obvious that if split SRB1 with duplication is configured, the legacy Rel-17 behavior can be reused; otherwise, new PC5-RRC signaling can be used.

Proposal 3:  For scenario 1, in order to bring the IDLE/INACTIVE Relay UE to CONNECTED, if split SRB1 with PDCP duplication is configured, the legacy R17 behavior can be used; otherwise, new PC5-RRC signaling can be used.

Based on Proposal 3, it is obvious that the selection between legacy R17 behavior and new PC5-RRC signaling totally depends on gNB implementation. But when gNB makes decision, it should at least know the Relay UE’s RRC state and whether the Relay UE supports new PC5-RRC signaling. Considering in Rel-18, only intra-gNB multi-path is considered, it means the Relay UE and Remote UE should be under the same gNB, hence,  gNB can aware the Relay UE’s RRC state based on the measurement report from the Remote UE. But gNB cannot aware whether the IDLE/INACTIVE Relay UE supports new PC5-RRC signaling or not. Hence, Remote UE should get the relay UE’s release/capability from Relay UE and report it to gNB.

Proposal 4:  For scenario 1, Remote UE should acquire the release/capability of the Relay UE and indicate it to gNB.

Relay UE HO

In RAN2#122 meeting, the following agreement was reached on Relay UE HO:

For Scenario-1/2, not pursue remote UE notifying network upon reception of notification message indicating relay UE handover. FFS whether rely on network to release configuration of relay UE at remote UE before relay UE handover, or rely on remote UE to suspend the indirect path upon reception of notification message indicating relay UE handover.

In RAN2#123 meeting, during the offline discussion [4], the following two options are summarized:

- Option 1: Rely on network to release configuration of Relay UE at Remote UE before Relay UE handover.

- Option 2: Rely on Remote UE to suspend the indirect path upon reception of notification message indicating Relay UE handover.

In Rel-18, only intra-gNB multi-path is considered, one gNB serves both the Relay UE and Remote UE. If the gNB determines to handover the Relay UE, it is feasible for the gNB to release the Relay configuration at Remote UE, this release can even happen before Relay UE sends notification message to Remote UE. In case of the network does not release the Relay UE configuration, once the Remote UE receives the notification from Relay UE, it can also suspend the indirect path as indicated in Option 2.

Proposal 5: For scenario 1, when Relay UE performs handover, network can release the Relay configuration at the Remote UE before Relay UE handover.

Proposal 6: For scenario 1, upon Remote UE receiving the notification message indicating Relay UE handover, the Remote UE suspends the indirect path if it has not been released.

Issues specific for scenario 2

Support of Case G for scenario 2

Regarding to scenario 2, the following working assumption was reached:

Working assumption:

Support case G for scenario 2 for RRC_CONNECTED target relay UE.

Whether case G should be supported mainly depends on the following two aspects:

- Issue 1: Whether there is use case to support more than one Relay UE for a Remote UE?

- Issue 2: Whether the additional specification effort is acceptable?

For issue 1), from the perspective of use case, considering the interface between Relay UE and Remote UE are ideal, the Relay UE and Remote UE should be co-located or wired connected. In this case, the main benefit of introducing multiple candidate Relay UEs is to improve the robustness, e.g., if once the indirect path failed, it can be recovered by using another Relay UE.

For issue 2), if multiple candidate Relay UEs are introduced, the main specification effort is who performs the target Relay UE selection? Remote UE or gNB? If Remote UE selects the target Relay UE, it is totally depends on Remote UE implementation and no specification effort. If gNB selects the target Relay UE, Remote UE should report the multiple candidate Relay UEs to gNB and gNB should indicated the selected target Relay UE to the Remote UE which is similar as the Case G of scenario 1 except that the fields contained in the measurement report may be different and the trigger for measurement report is different. It is obvious that the additional specification effort is limited.

Proposal 7: Confirm the working assumption that support case G for scenario 2 for RRC_CONNECTED target relay UE.

If case G is supported, it means that multiple candidate relay UEs can be supported. Hence when Remote UE reports Relay UE’s ID to gNB, multiple Relay UEs can be reported.

Proposal 8: For scenario 2, when Remote UE reports Relay UE’s ID to gNB, multiple Relay UEs can be reported.

IDLE/INACTIVE Relay UE ID reporting for scenario 2

In RAN2#121bis meeting, regarding to scenario 2, the following agreement was reached:

In scenario 2, if both remote and relay UE are in RRC_CONNECTED, the remote UE reports relay UE’s ID to gNB for indirect path addition.  Need for reporting in the idle/inactive case can be further discussed.  FFS what ID is used.

In RAN2#123 meeting, it was agreed that:

For Scenario 2, leave it to relay and remote UE implementation on how to trigger the RRC_IDLE/RRC_INACTIVE target relay UE to initiate RRC connection establishment procedure.

Based on the above agreements, it is obvious that the candidate Relay UE can be idle and inactive for scenario 2. In order to solve the Relay UE ID reporting issue, there are mainly two options:

- Option 1: Remote UE can trigger Relay UE into CONNECTED firstly, and then report the C-RNTI.

- Option 2: Remote UE can report the Relay UE ID even if the Relay UE is in IDLE/INACTIVE.

  - Option 2.1: The Relay UE ID can be Relay UE’s 5G-RUTI (for IDLE) or I-RNTI(for INACTIVE) [1];

  - Option 2.2:   A new Relay UE other than C-RNTI and how to assign/exchange the ID between the Remote UE and Relay UE on non-3GPP connection is up to UE implementation[2];

  - Option 2.3: gNB preconfigured local ID [3].

For Option 2, the main benefit of introducing multiple candidate Relay UEs is to let gNB select/reselect a more proper target Relay UE considering the Relay UE’s Uu link quality and Relay UE’s load. But if the reported candidate Relay UEs are in IDLE/INACTIVE, gNB cannot aware the Relay UE’s Uu link, the benefit of introducing multiple candidate Relay UEs cannot be achieved. Furthermore, Option 2 will introduce new Relay UE ID other than C-RNTI, it should further discuss which Relay UE ID should be used, which will need more discussion.

Considering the limited time and the benefit of supporting IDLE/INACTIVE candidate Relay UE is unclear, it had better adopt option 1.

Proposal 9: For scenario 2, Remote UE can report the candidate Relay UEs’s ID only when the candidate Relay UE is in RRC_CONNECTED state.

Which message is used to send the candidate Relay UEs’s ID?

In the previous meeting, it was agreed that the remote UE reports relay UE’s ID to gNB for indirect path addition. But which message is used to send the candidate Relay UE’s ID has not been discussed. In the last RAN2 meeting, three options are mentioned in [2]:

- Option 1. Using UE information procedure.

- Option 2. Using measurement framework.

- Option 3. Using UAI framework.

As mentioned in [2], for Option 1, the gNB request the Remote UE to report the associated Relay UE information. And then, the Remote UE includes the Relay UE ID in the UE information response. It means this is a new procedure. For Option 2, it should also define new measurement event. Compared with Option 1 and Option 2, Option 3 has the less specification effort. Hence, we slightly prefer Option 3.

Proposal 10: For scenario 2, Remote UE can report the candidate Relay UEs’s ID using UAI.

Which node decides the indirect path change?

For scenario 1, the indirect path change is determined by gNB. For scenario 2, considering the serving Relay UE is in RRC_CONNECTED, the gNB can know the Uu link quality of Relay UE. Regarding to the ideal link, if it cannot work, either the Remote UE or the Relay UE can report it to gNB. Hence, it had better let gNB decide the indirect path change, which is also compatible with scenario 1.

Proposal 11: For scenario 2, same as scenario 1, left gNB implementation to determine the indirect path change.

Multi-path authorization

In [5], SA2 reply to RAN2 that multi-path transmission authorization and subscription function for Scenario 2 in all cases is out of SA2 scope. It means the multi-path authorization and subscription should not impact CN.

In addition, in RAN2#119 meeting, it was already agreed that:

RAN2 deprioritizes discussion on authorization and association mechanism between remote UE and relay UE in scenario 2.

Hence, it is suggested the authorization for scenario 2 left to UE implementation.

Proposal 12: For scenario 2, no further RAN2 work on multi-path authorization is needed.

RRC connection re-establishment

In RAN2#121 meeting, it was agreed that:

In scenario 1, when a remote UE configured with multi-path initiates the RRC re-establishment procedure, the remote UE does not perform re-establishment directly into a multi-path configuration.

But it is still unclear whether the above principle can also be applied to scenario 2. In our understanding, there is no strong motivation to differ scenario 1 and scenario 2.

Proposal 13: For scenario 2, when a remote UE configured with multi-path initiates the RRC re-establishment procedure, the remote UE does not perform re-establishment directly into a multi-path configuration.

Issues common for scenario 1 and scenario 2

Failure report for scenario 1/2

In RAN2#121bis-e meeting, it was agreed:

A remote UE in RRC_CONNECTED, upon reception of Uu RLF indication from the relay UE, suspends transmissions on the indirect path and informs the network if SRB1 is available on the direct path and not suspended, otherwise triggers re-establishment.

In RAN2#122 meeting, it was further agreed:

For Scenario-1/2, when reporting direct-path failure via indirect-path, use MCGFailureInformation message. FFS on whether additional IE needs to be introduced.

Based on the above agreements, regarding to the failure report, there are still some remaining issues need to be discussed:

- Issue 1: When reporting indirect path failure on direct path, which signaling is used?

- Issue 2: Whether additional IE needs to be introduced in MCGFailureInformation?

For Issue 1, there are three options:

- Option 1: SidelinkUEInformationNR message;

- Option 2: MCGFailureInformation message;

- Option 3: A new message.

For Option 1, in legacy Rel-17, SidelinkUEInformationNR can be used when PC5 RLF is detected. But it is not applicable for the Relay UE Uu link failure since the failure happens on Uu not PC5. Furthermore, considering the interface between Remote UE and Relay UE is ideal not PC5, Option 1 is not applicable for scenario 2. Hence, Option 1 can be excluded.

For Option 1 and Option 3, considering it was agreed that both direct path and indirect path belong to MCG, hence, it is slightly prefers to reuse MCGFailureInformation message.

Proposal 14: When reporting indirect path failure, MCGFailureInformation message can be used.

For Issue 2, according to the current TS38.331, the following failure type was defined:

failureType-r16                   ENUMERATED {t310-Expiry, randomAccessProblem, rlc-MaxNumRetx, t312-Expiry-r16, lbt-Failure-r16, beamFailureRecoveryFailure-r16,                                         bh-RLF-r16, spare1}

For direct path failure, the case is same as legacy, no additional failure type is needed.

For indirect path failure, in order to identify the Uu link of Relay UE failure (for scenario 1), PC5 link between Relay and Remote UE failure (for scenario 1) and ideal link failure (for scenario 2), it had better introduce new failure types.

Proposal 15: Introducing new failure types (e.g., sl-Failure, n3c-IdealLink-Failure, relayUu-Failure) in MCGFailureInformation.

Path activation/deactivation for scenario 1/2

In RAN2#121bis-e meeting, it was agreed to reuse primary path and primary RLC entity for MP split bearer.

The concept of the ‘primary path and primary RLC entity’ is adopted for each MP split bearer configuration according to the existing definition.

For legacy CA, SCells are configured by gNB via RRC signaling, and can be activated or deactivated via MAC CE in order to achieve power saving benefit. However, for multi-path scenario, direct path and indirect path can be in the same cell. Hence, the cell activation/deactivation concept is not suitable any more. Therefore, from the perspective of power saving, it is suggested to introduce path activation/deactivation mechanism.

Similar as SCell activation/deactivation, path activation/deactivation can be implemented by MAC CE. Since Uu MAC CE can’t be transmitted to the remote UE via the indirect path. Path activation/deactivation can be done via MAC CE over direct path.

Proposal 16: Path activation/deactivation is introduced for multi-path for scenario 1/2.

Proposal 17: Introduce Path Activation/Deactivation MAC CE over direct path to control the activation/deactivation of indirect path.

PDCP duplication

Regarding to the PDCP duplication, in RAN2#122 meeting, the following agreements were reached:

For Scenario-1/2, PDCP duplication of DRB is controlled by legacy Duplication Activation/Deactivation MAC CE and Duplication RLC Activation/Deactivation MAC CE delivered via direct path.

For Scenario-1/2, RRC sets the initial state of PDCP duplication for split SRB/DRB as in legacy.

In RAN2#123 meeting, it was further agreed that:

In packet duplication for scenario 1, the PDCP entity need not indicate to the Uu RLC entity to discard the PDCP PDU when the PC5 RLC entity acknowledges the transmission of the PDCP PDU.  FFS if this requirement can be stronger (“shall not”), to be discussed in CR development.

In packet duplication for scenario 1, in the case where Uu RLC entity at the remote UE acknowledges the transmission

Based on the above agreements, there are still some open issues regarding to PDCP duplication, listed below:

- Issue 1: Whether CA duplication is applied to the direct path?

- Issue 2: Whether CA duplication is applied to the indirect path?

- Issue 3: In case of PDCP duplication, how many legs can be supported?

For Issue 1, in legacy Uu, UE can aggregate multiple serving cells. With the introduction of SL relay, it is not reasonable to restrict the direct path can only have one serving cell. That is to say, direct path can have multiple serving cells and CA duplication should be supported.

Proposal 18:  For scenario 1 and scenario 2, CA duplication can be supported in the direct path.

For Issue 2, there are two hops: PC5/ideal hop and Relay UE’s Uu hop. The analysis should be performed for each hop:

- For the ideal hop of scenario 2, CA duplication cannot be supported since the interface between the Remote UE and Relay UE is not in 3GPP scope.

- For the PC5 hop of scenario 1, considering SL CA is still under discussion in Rel-18 SL evolution work item, hence, it had better not consider SL CA for the hop between Relay UE and Remote UE in Rel-18.

- For the Relay UE’s Uu hop, if CA is not considered between the Remote UE and Relay UE, it is meaningless to consider CA duplication in Relay UE’s Uu interface.

Proposal 19: For scenario 1 and scenario 2, CA duplication is not supported in the indirect path.

For Issue 3, based on the analysis on Issue 1 and Issue 2, it is obvious that at most three legs can be supported: one on the indirect path and at most two on the direct path.

Proposal 20: For scenario 1 and scenario 2, if PDCP duplication is used, at most three duplication legs can be supported.

Conclusion

According to the analysis in section 2, it is proposed:

Proposal 1: For scenario 1, for indirect path release, the timing to release the PC5-RRC connection can be left to UE implementation.

Proposal 2: For scenario 1, event Z1 can be used for indirect path change of multi-path.

Proposal 3:  For scenario 1, in order to bring the IDLE/INACTIVE Relay UE to CONNECTED, if split SRB1 with PDCP duplication is configured, the legacy R17 behavior can be used; otherwise, new PC5-RRC signaling can be used.

Proposal 4:  For scenario 1, Remote UE should acquire the release/capability of the Relay UE and indicate it to gNB.

Proposal 5: For scenario 1, when Relay UE performs handover, network can release the Relay configuration at the Remote UE before Relay UE handover.

Proposal 6: For scenario 1, upon Remote UE receiving the notification message indicating Relay UE handover, the Remote UE suspends the indirect path if it has not been released.

Proposal 7: Confirm the working assumption that support case G for scenario 2 for RRC_CONNECTED target relay UE.

Proposal 8: For scenario 2, when Remote UE reports Relay UE’s ID to gNB, multiple Relay UEs can be reported.

Proposal 9: For scenario 2, Remote UE can report the candidate Relay UEs’s ID only when the candidate Relay UE is in RRC_CONNECTED state.

Proposal 10: For scenario 2, Remote UE can report the candidate Relay UEs’s ID using UAI.

Proposal 11: For scenario 2, same as scenario 1, left gNB implementation to determine the indirect path change.

Proposal 12: For scenario 2, no further RAN2 work on multi-path authorization is needed.

Proposal 13: For scenario 2, when a remote UE configured with multi-path initiates the RRC re-establishment procedure, the remote UE does not perform re-establishment directly into a multi-path configuration.

Proposal 14: When reporting indirect path failure, MCGFailureInformation message can be used.

Proposal 15: Introducing new failure types (e.g., sl-Failure, n3c-IdealLink-Failure, relayUu-Failure) in MCGFailureInformation.

Proposal 16: Path activation/deactivation is introduced for multi-path for scenario 1/2.

Proposal 17: Introduce Path Activation/Deactivation MAC CE over direct path to control the activation/deactivation of indirect path.

Proposal 18:  For scenario 1 and scenario 2, CA duplication can be supported in the direct path.

Proposal 19: For scenario 1 and scenario 2, CA duplication is not supported in the indirect path.

Proposal 20: For scenario 1 and scenario 2, if PDCP duplication is used, at most three duplication legs can be supported.

Reference

- R2-2307745	 Open issues on multi-path relay for scenario 1 and scenario 2	   Qualcomm

- R2-2308206  Remaining issues on multi-path operation	  Huawei, HiSilicon

- R2-2308324  Considerations on multi-path scenario 2     CMCC

- R2-2308949  Offline 402 on A.I 7.9.4 Multi-path relaying      Nokia

- R2-2307057	Reply LS to SA2 on authorization for multi-path Scenario 2 (S2-2307707; contact: LGE)   SA2	LS in	Rel-18	NR_SL_relay_enh, 5G_ProSe_Ph2	To:RAN2  Cc:RAN3