3GPP TSG-RAN WG2 Meeting #123bis                                              	R2-2309663

Xiamen, China, 9th– 13th Sep, 2023

Source:	CATT, Turkcell

Title:	Considerations on data collection of AIML for NR air-interface

Agenda Item:	7.16.2.2

Document for:	Discussion and Decision

Introduction

For data collection of AI/ML for NR air-interface, to coordinate the LS to RAN1, a lot of agreements were achieved in RAN2#122 meeting. And then the LS out was approved in [1]. The LS has 2 parts of contents:

Part A: RAN2 Assumptions on data collection that require RAN1 confirmation

Part B: Aspects of data collection that require RAN1 feedback/inputs

In part A, 4 assumptions are listed and need RAN1 to confirm:

- Assumption 1: RAN2 assumes that for the data collection in some scenarios (e.g., internal data up to implementation or the existing data are enough), possibly no RAN2 specification effort is needed in some scenarios;

- Assumption 2: For the latency requirement of data collection, RAN2 assumes for all types of offline model training, there is no latency requirement for data collection. But for model inference or (real-time) model monitoring, there is a latency requirement for data collection;

- Assumption 3: RAN2 assumes that the analysis/selection of the data collection frameworks should focus on the RRC_CONNECTED state (for both data generation and reporting). Analysis and potential enhancement of the non-connected state can be revisited when needed;

- Assumption 4: For the data generation entity and termination entity deployed at different entities, RAN2 made the assumptions on the generated / terminated entities for model training/inference/monitoring per use case.

In part B, RAN2 would like RAN1 to provide feedback/inputs on the following essential aspects per LCM purpose (i.e., model training, inference and monitoring) for each (sub)use case, and the LCM sidedness should also be considered:

- Data content

- Typical data size (value or value range) of the identified data content

- Reporting type (e.g., periodic, event triggered, other) of the identified data content

- Typical latency requirement (value or value range) to transfer the identified data content

And then based on RAN1 feedback, RAN2 could facilitate the discussion on data collection for further progress.

Since RAN1 only replied Part A in [2], and RAN1 email discussion about Part B will conclude on September 28th,   in this contribution, first we intend to further analyze the data collection requirement based on the replied views of RAN1 LS Part A, and then to discuss whether and how to apply or enhance the existing frameworks to achieve the AI/ML model LCM purposes.

Discussion

  - Considerations based on RAN1 reply LS Part A

For Assumption 1 (possibly no RAN2 specification effort is needed in some scenarios)

RAN1 made some modifications that the description of “(real-time)” has been removed, and some sub-use cases are provided for examples:

- For model inference of the UE-sided model, input data for model inference is available inside the UE. - For UE-side (real-time) performance monitoring of the UE-sided model, in some cases, e.g., for CSI prediction and beam prediction, performance metrics are available inside the UE. UE can independently monitor a model's performance without any data input from NW. - Note: RAN1’s understanding is that “data input” in the above does not include assistance information that a model may additionally use for performance metric calculation

RAN1 confirmed that for the UE-sided model, the input data for model inference or for model monitoring can be available inside the UE.

For Assumption 2 (latency requirement of data collection)

RAN1 confirmed that for all types of offline model training (i.e., UE- /NW-/ two-sided model training), there is no latency requirement. But for model inference or (real-time) model monitoring, there have latency requirement when required data comes from other entities.

For Assumption 3 (whether focus on the RRC_CONNECTED state)

RAN1 confirms RAN2’s Assumption 3 for CSI compression, CSI prediction, beam prediction and Positioning use cases. For positioning, it is noted that existing specification supports DL PRS measurement and UE positioning in both RRC_CONNECTED and RRC_INACTIVE state.

Observation 1: For positioning, RAN1 confirms existing specification supports DL PRS measurement and UE positioning in both RRC_CONNECTED and RRC_INACTIVE state.

For Assumption 4 (data generation entity and termination entity deployed at different entities)

For the data generation entity and termination entity deployed at different entities, RAN1 separated CSI enhancement use case into CSI compression and CSI prediction parts, and explained them separately.

And for the UE-part of two-sided model inference or UE-side model inference, input data is internally available at UE, and cannot be generated by gNB/LMF and terminated at UE.

Observation 2: For the UE-part of two-sided model inference or UE-side model inference, RAN1 only agree that input data is internally available at UE.

Summary of RAN1 reply

Based on the RAN1 response for all the 4 assumptions, we list the key word in the table as below:

Table 1 Need of the input data between UE and NW node(s)

|  |  | CSI compression | CSI prediction | Beam management | Positioning enhancement (in RRC_CONNECTED and RRC_INACTIVE) |
| --- | --- | --- | --- | --- | --- |
| For model training (no latency requirement), input data can be generated by |  | UE/gNB | UE | UE/gNB | UE/PRU/gNB/LMF |
| For model inference | at UE side | internally available at UE | internally available at UE | internally available at UE | internally available at UE |
|  | at NW/gNB side | UE -> gNB | Open Issue1 | UE -> gNB | internally available at gNB |
|  | at LMF side | N/A | N/A | N/A | UE/gNB -> LMF |
| For performance monitoring | at UE side | Open Issue 2 (Almost exclude by RAN1) | available inside UE, based on assumption1 | available inside UE, based on assumption1 | Open Issue3 available inside the UE (if needed) |
|  | at NW/gNB side | UE -> gNB | UE -> gNB | UE -> gNB | generated by at least gNB |
|  | at LMF side | N/A | N/A | N/A | UE/gNB -> LMF |

In order to make the content in the table more complete, 3 open issues are raised:

Open issue 1 (For model inference): For CSI prediction, whether and how to require the input data for model inference in network side?

This case is not in the modified RAN1 reply LS, since in RAN1 agreement of RAN1#111 meeting:

Agreement Time domain CSI prediction using UE sided model is selected as a representative sub-use case for CSI enhancement.

So the CSI prediction can only be inferenced in the UE side, and the model inference at network side has been excluded from RAN1 agreement.

Observation 3: Model inference at NW side for CSI prediction has been excluded by RAN1.

Open issue 2 (For performance monitoring): For CSI compression, whether and how the metrics and the input data of performance monitoring can be required for UE side monitoring?

In LS assumption 1, RAN1 only take the sub-use cases of CSI prediction and beam prediction for example, so whether and how to require the metrics of performance monitoring for the other cases of UE side monitoring should be clarified.

In the agreement of RAN2#122 meeting:

RAN 2 assumes that for the data collection in some scenarios (e.g., internal data up to implementation or the existing data are enough), possibly no RAN2 specification effort is needed in some scenarios, e.g. (not exhaustive): - - For model inference of UE-sided model, input data for model inference is available inside the UE. - - For UE-side (real time) monitoring of UE-sided model, performance metrics are available inside the UE. UE can independently monitor a model's performance without any data input from NW.

RAN2’s agreement does not provide the detailed sub-use cases.

For CSI compression, whether to perform monitoring at the UE side had intense debate in RAN1. In RAN1#112bis meeting, it has been agreed that:

Agreement In CSI compression using two-sided model use case, for UE-side monitoring, further study potential specification impact on triggering and means for reporting the monitoring metrics, including periodic/semi-persistent and aperiodic reporting, and other reporting initiated from UE.

But some companies later doubt that how the UE could calculate the monitoring metric for the two-sided model, and think the resumed CSI should not be sent to the UE side from the NW side. From the RAN1 reply LS, it can be seen that all the sentence of input data “generated by gNB and terminated at UE” are removed, so here we can deduce that the input data from NW for UE side monitoring of CSI compression is also not allowed by RAN1.

Observation 4: Input data “generated by gNB and terminated at UE” is not allowed by RAN1.

Open issue 3 (For performance monitoring): For positioning enhancement, whether and how the metrics and the input data of performance monitoring can be required for UE side monitoring?

For UE-sided positioning enhancement (case 1/2a), since the performance monitoring for CSI prediction and beam prediction can be inside the UE, the performance monitoring for positioning case 1/2b can also be inside the UE, and the metrics and the input data of performance monitoring at UE side (if needed) are available inside the UE.

Observation 5: The metrics and the input data of performance monitoring at UE side for UE-sided positioning enhancement (case 1/2a) are available inside the UE.

So based on the observations above, it is proposed that:

Proposal 1: For the UE-part of two-sided model inference or UE-side model inference, input data is internally available at UE, and cannot be generated by gNB/LMF and terminated at UE.

Proposal 2: For CSI prediction, the model inference is only performed at UE side, and the input data is internally available at UE.

Proposal 3: For CSI compression, the input data from NW for UE side monitoring is not allowed, so the UE side performance monitoring is impractical.

Proposal 4: For UE-sided positioning enhancement (case 1/2a), the metrics and the input data of performance monitoring at UE side (if needed) are available inside the UE.

  - Applicability and enhancement of existing frameworks

The applicability and enhancement of existing frameworks should be discussed based on both RAN1 reply LS Part A and Part B. Since Part B will conclude on September 28th by RAN1, in this section, we only discuss whether and how these existing frameworks should be applied or enhanced for AI/ML usage based on our consideration.

Logged MDT

Logged MDT is a non-real time report based on UE recording of the cell/beam measurement results for a long time, e.g. 2 hours. If we allow the UE to report in a single RRC report multiple measurements, the logged MDT is the most appropriate mechanism to carry multiple measurement results from UE to the network side. Logged MDT is introduced based on the Trace mechanism in LTE R10, so the trace relate index such as traceReference and traceRecordingSessionRef is mandatory configured in the logged MDT configuration. Even the measurement result collected by the logged MDT can be utilized by the gNB node, the results should still be sent to the OAM collection entity of TCE. Moreover, the logged MDT can only be performed in IDLE/INACTIVE mode, and the logged MDT in CONNECTED mode is not support since in LTE.

Observation 6: Logged MDT cannot be performed in CONNECTED mode now, and the trace related ID is mandatory in the configuration and reporting.

Proposal 5: Logged MDT can be used for offline model training based data collection if the conditions as below can be satisfied:

- Support of CONNECTED state logged MDT;

- Allowed by OAM to use MDT to collect AI related data besides the existing MDT results.

Immediate MDT and L3 measurements

The differences between these two frameworks are:

- Immediate MDT needs UE to best effort report the location information besides the L3 RRM measurement result;

- Immediate MDT also needs to be performed under the trace framework, i.e. the MDT configuration should be required from OAM, and the collected data from UE needs to be sent to TCE by the gNB.

Immediate MDT and L3 measurements can be performed in CONNECTED mode which could satisfied the Layer 3 signalling with ten milliseconds level latency, so these two frameworks may be used for transmitting the data for offline model training, model inference and performance monitoring. But since the “Typical latency requirement” in LS part B has not been response by RAN1, and the L1 measurement (CSI reporting) with e.g. slot level latency seems more appropriate for model inference and performance monitoring, these two frameworks can be considered for offline model training currently. Legacy L3 RRM measurement report is used for HO decision which carries RSRP/RSRQ/SINR result for event triggered measurement or periodically reported measurement, and it does not support UL segmentation in the current spec. Thus, if the size of the AI related data transmitted in the air-interface is larger than the PDCP SDU size limit of 9000 bytes, to transmit the data with current RRM measurement report may need enhancement.

Proposal 6: Immediate MDT and L3 RRM measurements can be used as baseline for data collection of offline model training if satisfying the conditions as below, respectively:

- For L3 RRM measurements, the transmitted data size should be limited;

- For Immediate MDT, allowed by OAM to use MDT to collect AI related data besides the existing MDT results.

L1 measurement (CSI reporting)

L1 measurement result is based on UCI with explicit format. In our understanding, due to the resource limitation, the UCI may not carry large amount of raw data for model training input, e.g., Precoding matrix, CIR, PDP. So for offline training which may needs large size data transmission but is not time-sensitive, L1 measurement seems not appropriate. However, using CSI reporting for ground-truth CSI transmission or for extended beam ID number transmission is still under discussion in RAN1, so it is also possible to use this framework for data collection of model training. For model inference or model monitoring, the information may need to be sent to the network side immediately, thus the L1 measurement can be used.

Proposal 7: L1 measurement can be used for data collection of time-sensitive purpose, e.g. model inference or model monitoring, or can be enhanced by carrying e.g. ground-truth CSI, for model training.

UAI

The UAI message may be sent to the network if it was configured to do so, or upon some criterion for a UE is met based on the network configuration. The message itself can be considered to send anything that the network requests. Since it is also a Layer 3 signalling with ten milliseconds level latency, and the configuration and report method are flexible, it can be used for the data collection of offline model training. But in order to use this framework, enhanced data for AI/ML in the network configuration e.g. other-Config in RRCReconfiguration is needed.

Proposal 8: UAI can be used for model training based data collection with enhanced data for AI/ML by network configuration.

Early measurements

Early measurements can only be performed in IDLE or INACTIVE mode and the result can be reported in CONNECTED mode. The UE should send available indicator to the gNB and wait for the network request. This mechanism does not need OAM configuration. RAN1 explained in the reply LS [2] the existing specification supports DL PRS measurement and UE positioning in both RRC_CONNECTED and RRC_INACTIVE state. But since the early measurement should be performed only in IDLE/INACTIVE state which does not match all the other use cases, in order to make the framework applicable to multiple cases with universality, this framework can be de-prioritized.

Proposal 9: De-prioritize early measurements for AIML based data collection.

LPP

For the use cases of CSI feedback enhancement and for beam management which are terminated at gNB node at network side, other frameworks involving both UE and gNB can be utilized. But for the positioning accuracy enhancement case 1/2a/2b which involves UE reporting for positioning accuracy enhancement use case, the gNB can be transparent for the data transmission between UE and LMF. Thus, to extend current LPP specification is a simpler and easier way to transmit the AI related data. Furthermore based on the same reasons, for the positioning accuracy enhancement case 3a/3b, to extend current LPPa specification could be considered.

Proposal 10: LPP specification can be extended for positioning accuracy enhancement case 1/2a/2b, and LPPa specification can be extended for case 3a/3b, for model training, model inference and model monitoring.

Conclusion

According to the analysis in section 2, we propose:

For Considerations based on RAN1 reply LS Part A

Observation 1: For positioning, RAN1 confirms existing specification supports DL PRS measurement and UE positioning in both RRC_CONNECTED and RRC_INACTIVE state.

Observation 2: For the UE-part of two-sided model inference or UE-side model inference, RAN1 only agree that input data is internally available at UE.

Observation 3: Model inference at NW side for CSI prediction has been excluded by RAN1.

Observation 4: Input data “generated by gNB and terminated at UE” is not allowed by RAN1.

Observation 5: The metrics and the input data of performance monitoring at UE side for UE-sided positioning enhancement (case 1/2a) are available inside the UE.

Proposal 1: For the UE-part of two-sided model inference or UE-side model inference, input data is internally available at UE, and cannot be generated by gNB/LMF and terminated at UE.

Proposal 2: For CSI prediction, the model inference is only performed at UE side, and the input data is internally available at UE.

Proposal 3: For CSI compression, the input data from NW for UE side monitoring is not allowed, so the UE side performance monitoring is impractical.

Proposal 4: For UE-sided positioning enhancement (case 1/2a), the metrics and the input data of performance monitoring at UE side (if needed) are available inside the UE.

For Applicability and enhancement of existing frameworks

Observation 6: Logged MDT cannot be performed in CONNECTED mode now, and the trace related ID is mandatory in the configuration and reporting.

Proposal 5: Logged MDT can be used for offline model training based data collection if the conditions as below can be satisfied:

- Support of CONNECTED state logged MDT;

- Allowed by OAM to use MDT to collect AI related data besides the existing MDT results.

Proposal 6: Immediate MDT and L3 RRM measurements can be used as baseline for data collection of offline model training if satisfying the conditions as below, respectively:

- For L3 RRM measurements, the transmitted data size should be limited;

- For Immediate MDT, allowed by OAM to use MDT to collect AI related data besides the existing MDT results.

Proposal 7: L1 measurement can be used for data collection of time-sensitive purpose, e.g. model inference or model monitoring, or can be enhanced by carrying e.g. ground-truth CSI, for model training.

Proposal 8: UAI can be used for model training based data collection with enhanced data for AI/ML by network configuration.

Proposal 9: De-prioritize early measurements for AIML based data collection.

Proposal 10: LPP specification can be extended for positioning accuracy enhancement case 1/2a/2b, and LPPa specification can be extended for case 3a/3b, for model training model inference and model monitoring.

Reference

- R2-2306906, LS out on Data Collection Requirements and Assumptions RAN2.

- R1-2308730, RAN2 LS reply on data collection RAN1.