3GPP TSG-RAN2 Meeting # 123bis	R2-2309686

# Xiamen, China, October 9 - 13

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 1665 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding default SL DRX configuration in determination procedure in 5.28.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO, Lenovo, Qualcomm, vivo, ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh-Core |  |  |  |  |  | Date: |  |  | 2023-09-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - For sl-DRX-GC-BC-Cycle/sl-DRX-GC-BC-OndurationTimer/sl-DRX-GC-InactivityTimer determination, the case where default SL DRX configuration being adopted is missed, i.e., for the SL DRX parameters to be used for a particular GC/BC destination, - RRC specification defines how to determine the DRX configurations for each QoS flow including the SL DRX configurations cam be mapped to dedicated QoS profiles and cannot be mapped to dedicated QoS profiles (default SL DRX configuration); - MAC specification defines how to determine the SL DRX parameters to be used for a Destination when one or multiple SL DRX mapped to the QoS flows is configured. The current specification only captured the case where the determination is based on the values associated with SL-QoS-Profile in sl-DRX-GC-BC-PerQoS-List while the values configured in sl-DefaultDRX-GC-BC-r17 are all missed and should be thus added. For the following 4 cases, Case-1: A single Cycle/Onduration/Inactivity timer that mapped to SL-QoS-Profile is associated with the destination, select the single value; Case-2: A single Cycle/Onduration/Inactivity time (configured in default SL DRX) that cannot be mapped to SL-QoS-Profile is associated with the destination, select the single default value; Case-3: Multiple Cycle/Onduration/Inactivity timer including only non-default values associated with the destination, select the longest/shortest value among the non-default configurations; Case-4: Multiple Cycle/Onduration/Inactivity timer including both non-default values and default value associated with the destination, select the longest/shortest value among both default and non-default configurations; As shown in the above 4 cases, case-1 and case-3 are existing cases that only related to non-default SL DRX values, while for case-2 and case-4, all the default configuration related things are missed, which is not correct. - In 5.28.2, it is not appropriate to say “interested cast type” since cast type is associated with destination but not interested by UE, so shoule be reworded. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Adding default SL DRX configuration in sl-DRX-GC-BC-Cycle/sl-DRX-GC-BC-OndurationTimer/sl-DRX-GC-InactivityTimer determination procedure in 5.28.2. - Changing “interested cast type is associated to” to “associated cast type is” in 5.28.2. Impact analysis Impacted 5G architecture options: NR SA Impacted functionality SL DRX Inter-operability: If the network is implemented according to this CR while the UE is not, there is no inter-operability issue because this CR is to add a missing case and rewording. If the UE is implemented according to this CR while the network is not, there is no inter-operability issue because this CR is to add a missing case and rewording. If one UE is implemented according to this CR while the other UE is not, there is no inter-operability issue because this CR is to add a missing case and rewording. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The determination of sl-DRX-GC-BC-Cycle/sl-DRX-GC-BC-OndurationTimer/sl-DRX-GC-InactivityTimer is not correct since missing of consideration of default SL DRX configuration - The “interest cast type” amy cause confusion. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.28.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Change

### 5.28.2	Behaviour of UE receiving SL-SCH Data

When SL DRX is configured, the Active Time includes the time while:

-	sl-drx-onDurationTimer/sl-DRX-GC-BC-OndurationTimer or sl-drx-InactivityTimer/sl-DRX-GC-InactivityTimer is running; or

-	sl-drx-RetransmissionTimer/sl-DRX-GC-RetransmissionTimer is running; or

-	period of sl-LatencyBoundCSI-Report configured by RRC in case SL-CSI reporting MAC CE is not received; or

-	the time between the transmission of the request of SL-CSI reporting and the reception of the SL-CSI reporting MAC CE in case SL-CSI reporting MAC CE is received; or

-	Slot(s) associated with the announced periodic transmission(s) by the UE transmitting SL-SCH Data; or

-	the time between transmission/reception of Direct Link Establishment Request message (TS 24.587 [28]) or ProSe Direct Link Establishment Request message (TS 24.554 [29]) and reception of RRCReconfigurationSidelink message including initial DRX configuration or the link establishment procedure being aborted by upper layer; or

-	the time between transmission of RRCReconfigurationSidelink message including initial DRX configuration and reception of corresponding RRCReconfigurationCompleteSidelink or RRCReconfigurationFailureSidelink message.

When one or multiple SL DRX is configured, the MAC entity shall:

1>	if a single sl-DRX-GC-BC-Cycle that is mapped with one or multiple SL-QoS-Profile in sl-DRX-GC-BC-PerQoS-List or a single sl-DRX-GC-BC-Cycle that is configured in sl-DefaultDRX-GC-BC and cannot be mapped with any SL-QoS-Profile in sl-DRX-GC-BC-PerQoS-List is associated to a Destination and the associated cast type is groupcast or broadcast:

2>	select the sl-DRX-GC-BC-Cycle that is mapped with one or multiple SL-QoS-Profile in sl-DRX-GC-BC-PerQoS-List or the sl-DRX-GC-BC-Cycle configured in sl-DefaultDRX-GC-BC associated with the Destination.

1>	else if multiple sl-DRX-GC-BC-Cycle that include one or multiple sl-DRX-GC-BC-Cycle that are mapped with one or multiple SL-QoS-Profile in sl-DRX-GC-BC-PerQoS-List and, if applicable, one sl-DRX-GC-BC-Cycle that is configured in sl-DefaultDRX-GC-BC and cannot be mapped with any SL-QoS-Profile in sl-DRX-GC-BC-PerQoS-List are associated to a Destination Layer-2 ID and the associated cast type is groupcast or broadcast:

2>	select the sl-DRX-GC-BC-Cycle whose length is the shortest one among multiple sl-DRX-GC-BC-Cycle that are mapped with one or multiple SL-QoS-Profile in sl-DRX-GC-BC-PerQoS-List and sl-DRX-GC-BC-Cycle configured in sl-DefaultDRX-GC-BC associated with the Destination Layer-2 ID.

1>	if a single sl-DRX-GC-BC-OndurationTimer that is mapped with one or multiple SL-QoS-Profile in sl-DRX-GC-BC-PerQoS-List or a single sl-DRX-GC-BC-OndurationTimer that is configured in sl-DefaultDRX-GC-BC and cannot be mapped with any SL-QoS-Profile in sl-DRX-GC-BC-PerQoS-List is associated to a Destination and the associated cast type is groupcast or broadcast:

2>	select the sl-DRX-GC-BC-OndurationTimer that is mapped with one or multiple SL-QoS-Profile in sl-DRX-GC-BC-PerQoS-List or the sl-DRX-GC-BC-OndurationTimer configured in sl-DefaultDRX-GC-BC associated with the Destination.

1>	else if multiple sl-DRX-GC-BC-OndurationTimer that include one or multiple sl-DRX-GC-BC-OndurationTimer that are mapped with one or multiple SL-QoS-Profile in sl-DRX-GC-BC-PerQoS-List and, if applicable, one sl-DRX-GC-BC-OndurationTimer that is configured in sl-DefaultDRX-GC-BC and cannot be mapped with any SL-QoS-Profile in sl-DRX-GC-BC-PerQoS-List are associated to a Destination Layer-2 ID and the associated cast type is groupcast or broadcast:

2>	select the sl-DRX-GC-BC-OndurationTimer whose length is the longest one among multiple sl-DRX-GC-BC-OndurationTimer that are mapped with one or multiple SL-QoS-Profile in sl-DRX-GC-BC-PerQoS-List and sl-DRX-GC-BC-Cycle configured in sl-DefaultDRX-GC-BC associated with the Destination Layer-2 ID.

1>	if a single sl-DRX-GC-InactivityTimer that is mapped with one or multiple SL-QoS-Profile in sl-DRX-GC-BC-PerQoS-List or a single sl-DRX-GC-InactivityTimer that is configured in sl-DefaultDRX-GC-BC and cannot be mapped with any SL-QoS-Profile in sl-DRX-GC-BC-PerQoS-List is associated to a Destination and the associated cast type is groupcast:

2>	select the sl-DRX-GC-InactivityTimer that is mapped with one or multiple SL-QoS-Profile in sl-DRX-GC-BC-PerQoS-List or the sl-DRX-GC-InactivityTimer configured in sl-DefaultDRX-GC-BC associated with the Destination:

1>	else if multiple sl-DRX-GC-InactivityTimer that include one or multiple sl-DRX-GC-InactivityTimer that are mapped with one or multiple SL-QoS-Profile in sl-DRX-GC-BC-PerQoS-List and, if applicable, one sl-DRX-GC-InactivityTimer that is configured in sl-DefaultDRX-GC-BC and cannot be mapped with any SL-QoS-Profile in sl-DRX-GC-BC-PerQoS-List of a Destination Layer-2 ID and the associated cast type is groupcast:

2>	select sl-DRX-GC-InactivityTimer whose length is the longest one among multiple sl-DRX-GC-InactivityTimer that are mapped with one or multiple SL-QoS-Profile in sl-DRX-GC-BC-PerQoS-List and sl-DRX-GC-InactivityTimer configured in sl-DefaultDRX-GC-BC associated with the Destination Layer-2 ID.

1>	if an sl-drx-HARQ-RTT-Timer expires:

2>	if the data of the corresponding Sidelink process was not successfully decoded or if the HARQ feedback (i.e., negative acknowledgement) is not transmitted for unicast due to UL/SL prioritization:

3>	start the sl-drx-RetransmissionTimer/sl-DRX-GC-RetransmissionTimer for the corresponding Sidelink process in the first slot after the expiry of sl-drx-HARQ-RTT-Timer.

When the cast type is groupcast or broadcast as indicated by upper layer, or the cast type is unicast for the reception of Direct Link Establishment Request message [28] or ProSe Direct Link Establishment Request message [29] as indicated by upper layer, or for the reception of discovery message [26], the sl-drx-StartOffset and sl-drx-SlotOffset are derived from the following equations:

sl-drx-StartOffset (ms) = Destination Layer-2 ID modulo sl-DRX-GC-BC-Cycle (ms).

sl-drx-SlotOffset (ms) = (Destination Layer-2 ID modulo the number of slots in one subframe)
/ (the number of slots in one subframe) (ms).

1>	if the SL DRX cycle is used, and [(DFN × 10) + subframe number] modulo (sl-drx-Cycle or sl-DRX-GC-BC-Cycle) = sl-drx-StartOffset:

2>	start sl-drx-onDurationTimer/sl-DRX-GC-BC-OndurationTimer after sl-drx-SlotOffset from the beginning of the subframe.

1>	if an SL DRX is in Active Time:

2>	monitor the SCI (i.e., 1st stage SCI and 2nd stage SCI).

2>	if the SCI indicates a new SL transmission:

3>	if Source Layer-1 ID of the SCI is equal to the 8 LSB of the intended Destination Layer-2 ID and Destination Layer-1 ID of the SCI is equal to the 16 LSB of the intended Source Layer-2 ID and the cast type indicator in the SCI is set to unicast:

4>	start or restart sl-drx-InactivityTimer for the corresponding Source Layer-2 ID and Destination Layer-2 ID pair in the first slot after SCI reception.

3>	if Destination Layer-1 ID of the SCI (i.e., 2nd stage SCI) is equal to the 16 LSB of the intended Destination Layer-2 ID and the cast type indicator in the SCI is set to groupcast:

4>	start or restart sl-DRX-GC-InactivityTimer for the corresponding Destination Layer-2 ID in the first slot after SCI reception.

2>	if the SCI indicates an SL transmission:

3>	if a next retransmission opportunity is indicated in the SCI:

4>	derive the sl-drx-HARQ-RTT-Timer from the retransmission resource timing of the next retransmission resource in the SCI.

3>	else if PSFCH resource is configured for the SL grant associated to the SCI:

4>	set the sl-drx-HARQ-RTT-Timer based on sl-drx-HARQ-RTT-Timer1 configured by upper layer if the cast type associated with the SCI is unicast or sl-DRX-GC-HARQ-RTT-Timer1 configured by upper layer if the cast type associated with the SCI is groupcast when HARQ feedback is enabled, or based on sl-drx-HARQ-RTT-Timer2 configured by upper layer if the cast type associated with the SCI is unicast or sl-DRX-GC-HARQ-RTT-Timer2 configured by upper layer if the cast type associated with the SCI is groupcast when HARQ feedback is disabled, for resource pool configured with PSFCH.

3>	else (i.e., if PSFCH resource is not configured for the SL grant associated to the SCI):

4>	set the sl-drx-HARQ-RTT-Timer as 0 slots.

3>	if PSFCH resource is not configured for the SL grant associated to the SCI:

4>	start the sl-drx-HARQ-RTT-Timer for the corresponding Sidelink process in the slot following the end of PSSCH transmission (i.e., currently received PSSCH).

3>	if PSFCH resource is configured for the SL grant associated to the SCI:

4>	if HARQ feedback is enabled by the SCI and the cast type associated with the SCI is unicast; or

4>	if HARQ feedback is enabled by the SCI and the cast type associated with the SCI is groupcast and positive-negative acknowledgement is selected;

5>	start the sl-drx-HARQ-RTT-Timer for the corresponding Sidelink process in the first slot after the end of the corresponding PSFCH transmission carrying the SL HARQ feedback; or

5>	start the sl-drx-HARQ-RTT-Timer for the corresponding Sidelink process in the first slot after the end of the corresponding PSFCH resource for the SL HARQ feedback when the SL HARQ feedback is not transmitted due to UL/SL prioritization.

4>	if HARQ feedback is enabled by the SCI and the cast type associated with the SCI is groupcast and negative-only acknowledgement is selected;

5>	start the sl-drx-HARQ-RTT-Timer for the corresponding Sidelink process in the first slot after the end of the corresponding PSFCH transmission carrying the SL HARQ feedback; or

5>	start the sl-drx-HARQ-RTT-Timer for the corresponding Sidelink process in the first slot after the end of the corresponding PSFCH resource for the SL HARQ feedback when the SL HARQ feedback is not transmitted due to UL/SL prioritization; or

5>	start the sl-drx-HARQ-RTT-Timer for the corresponding Sidelink process in the first slot after the end of the corresponding PSFCH resource for the SL HARQ feedback when the SL HARQ feedback is a positive acknowledgement.

4>	if HARQ feedback is disabled by the SCI and the resource(s) for one or more retransmission opportunities is not scheduled in the SCI:

5>	start the sl-drx-HARQ-RTT-Timer for the corresponding Sidelink process in the slot following the end of PSFCH resource.

4>	if HARQ feedback is disabled by the SCI and the resource(s) for one or more retransmission opportunities is scheduled in the SCI:

5>	start the sl-drx-HARQ-RTT-Timer for the corresponding Sidelink process in the slot following the end of PSSCH transmission (i.e., currently received PSSCH).

NOTE:	Void.

3>	stop the sl-drx-RetransmissionTimer/sl-DRX-GC-RetransmissionTimer for the corresponding Sidelink process.

1>	if an SL DRX Command MAC CE is received for the Source Layer-2 ID and Destination Layer-2 ID pair of a unicast:

2>	stop sl-drx-onDurationTimer for the Source Layer-2 ID and Destination Layer-2 ID pair of a unicast;

2>	stop sl-drx-InactivityTimer for the Source Layer-2 ID and Destination Layer-2 ID pair of a unicast.

End of Change