3GPP TSG-RAN2#123bis 	R2-2309691

Xiamen, 9th Oct – 13th Oct, 2023

Agenda item:	7.20.3 (NR_MIMO_evo_DL_UL-Core)

Source:	LG Electronics Inc.

Title: 	Discussion on Unified State MAC CE for mDCI

Document for:	Discussion and Decision

1.	Introduction

For unified TCI framework extension, RAN1 made agreements for mDCI based mTRP operation as follows in RAN1#110bis.

Agreement: On unified TCI framework extension for M-DCI based MTRP: - The existing TCI field in a DCI format 1_1/1_2 (with or without DL assignment) associated with one coresetPoolIndex value can indicate the joint/DL/UL TCI state(s) specific to the same coresetPoolIndex value - FFS: The UE shall apply the indicated joint/DL/UL TCI state(s) specific to a coresetPoolIndex value to channel(s)/signal(s) that have explicit or implicit association with the same coresetPoolIndex value - A coresetPoolIndex value field is included in TCI state activation command (MAC-CE) to indicate that the mapping between the activated TCI state(s) and the TCI codepoint(s) is specific to which coresetPoolIndex value

Based on RAN1 agreement, RAN2 made some agreements for Unified TCI state activation/deactivation MAC CE for mDCI based mTRP.

In this contribution, we present our view on the further discussion point for Unified TCI state A/D MAC CE for mDCI based mTRP and suggest the format.

2.	Discussion

For mDCI based mTRP operation, RAN2 agreed to revise the legacy Unified TCI state Activation/Deactivation MAC CE.

RAN2 confirm the following working assumption as an agreement: - Revise the legacy unified TCI state activation/deactivation MAC CE by adding a “CORESET Pool ID” field to support mDCI based mTRP operation.

Currently, there are six R bits in Unified TCI state Activation/Deactivation MAC CE. In order to implement CORESET Pool ID, we think one of R bit can be replaced by the field of CORESET Pool Index.

The following figure shows one example. Note that for mDCI based mTRP, Unified TCI state Activation/Deactivation MAC CE is not different between joint mode and separate mode.

Figure 1. Example of Unified TCI state Activation/Deactivation MAC CE for mDCI based mTRP

Proposal 1. For mDCI based mTRP operation, one of R bit in Unified TCI state Activation/Deactivation MAC CE can be replaced by the field of CORESET Pool Index.

Next, whether to use separate MAC CE format is used for joint mode and separate mode can be discussed. In Rel-18 mDCI based mTRP operation, no new enhancement is required for joint mode and separate mode. Thus, Unified TCI stat Activation/Deactivation MAC CE revised with CORESET Pool Index is enough and separate MAC CE format is not needed.

Proposal 2. For mDCI based mTRP operation, same MAC CE format is used for joint mode and separate mode.

Then, whether to use new eLCID for Unified TCI state Activation/Deactivation MAC CE for mDCI based mTRP needs to be discussed.

In our understanding, both Rel-18 TCI operation and legacy TCI operation can be co-existed for a serving cell, and both the legacy Unified TCI state Activation/Deactivation MAC CE and Unified TCI state Activation/Deactivation MAC CE for mDCI based mTRP need to be applied for the serving cell.

According to [1], applyIndicatedTCIState-r18 IE is introduced to indicate whether to apply Rel-18 TCI state or not and applyIndicatedTCIState-r18 IE is configured per Resource or Resource set. Thus, there is a case where some resource/resource set follows Rel-18 TCI state and the other resource/resource set follows Rel-17 TCI operation.

For example, ControlResourceSet Id#1 is configured with applyIndicatedTCIState-r18 and ControlResourceSet Id#2 is configured with followUnifiedTCI-State-r17. For ControlResourceSet Id#1, TCI states activated by Rel-18 MAC CE are mapped, i.e. Unified TCI state Activation/Deactivation MAC CE for mDCI based mTRP, and for ControlResourceSet Id#2, TCI states activated in Rel-17 MAC CE are mapped, i.e. legacy Unified TCI state Activation/Deactivation MAC CE.

Therefore, both the legacy Unified TCI state Activation/Deactivation MAC CE and Unified TCI state Activation/Deactivation MAC CE for mDCI based mTRP should be differentiated in the serving cell, and we think each MAC CE should have different eLCID, i.e., new one eLCID is introduced for Unified TCI state Activation/Deactivation MAC CE for mDCI based mTRP.

Proposal 3. For mDCI based mTRP operation, new one eLCID is used for Unified TCI state Activation/Deactivation MAC CE.

From Proposal 1-3, we provide TP in Annex.

Proposal 4. Adopt TP as in Annex.

3.	Conclusion

In this document, we present our view on impact of mTRP on MAC CE, and made proposals as follows.

Proposal 1. For mDCI based mTRP operation, one of R bit in Unified TCI state Activation/Deactivation MAC CE can be replaced by the field of CORESET Pool Index.

Proposal 2. For mDCI based mTRP operation, same MAC CE format is used for joint mode and separate mode.

Proposal 3. For mDCI based mTRP operation, new one eLCID is used for Unified TCI state Activation/Deactivation MAC CE.

Proposal 4. Adopt TP as in Annex.

4.	Reference

- R2-2309434	LS on Rel-18 higher-layers parameter list (R1-2308674; contact: Ericsson)

5.	Annex

#### 6.1.3.XX	Unified TCI States Activation/Deactivation MAC CE for mDCI based mTRP

The Unified TCI States Activation/Deactivation MAC CE for mDCI based mTRP is identified by a MAC subheader with eLCID as specified in Table 6.2.1-1b. It has a variable size consisting of following fields:

-	Serving Cell ID: This field indicates the identity of the Serving Cell for which the MAC CE applies. The length of the field is 5 bits. If the indicated Serving Cell is configured as part of a simultaneousU-TCI-UpdateList1, simultaneousU-TCI-UpdateList2, simultaneousU-TCI-UpdateList3 or simultaneousU-TCI-UpdateList4 as specified in TS 38.331 [5], this MAC CE applies to all theServing Cells in the set simultaneousU-TCI-UpdateList1, simultaneousU-TCI-UpdateList2, simultaneousU-TCI-UpdateList3 or simultaneousU-TCI-UpdateList4, respectively;

-	DL BWP ID: This field indicates a DL BWP for which the MAC CE applies as the codepoint of the DCI bandwidth part indicator field as specified in TS 38.212 [9]. The length of the BWP ID field is 2 bits;

-	UL BWP ID: This field indicates a UL BWP for which the MAC CE applies as the codepoint of the DCI bandwidth part indicator field as specified in TS 38.212 [9]. If value of unifiedTCI-StateType in the Serving Cell indicated by Serving Cell ID is joint, this field is considered as the reserved bits. The length of the BWP ID field is 2 bits;

-	CORESET Pool ID: This field indicates CORESET Pool Index by coresetPoolIndex as specified in TS 38.331 [5]. If CORESET Pool ID is set to 0, it indicates that the received Unified TCI state Activation/Deactivation MAC CE is applied to coresetPoolIndex 0, otherwise, it indicates that the received Unified TCI state Activation/Deactivation MAC CE is applied to coresetPoolIndex 1;

-	Pi: This field indicates whether each TCI codepoint has multiple TCI states or single TCI state. If Pi field is set to 1, it indicates that ith TCI codepoint includes the DL TCI state and the UL TCI state. If Pi field is set to 0, it indicates that ith TCI codepoint includes only the DL/joint TCI state or the UL TCI state. The codepoint to which a TCI state is mapped is determined by its ordinal position among all the TCI state ID fields;

-	D/U: This field indicate whether the TCI state ID in the same octet is for joint/downlink or uplink TCI state. If this field is set to 1, the TCI state ID in the same octet is for joint/downlink. If this field is set to 0, the TCI state ID in the same octet is for uplink;

-	TCI state ID: This field indicates the TCI state identified by TCI-StateId as specified in TS 38.331 [5]. If D/U is set to 1, 7-bits length TCI state ID i.e. TCI-StateId as specified in TS 38.331 [5] is used. If D/U is set to 0, the most significant bit of TCI state ID is considered as the reserved bit and remainder 6 bits indicate the TCI-UL-State-Id as specified in TS 38.331 [5]. The maximum number of activated TCI states is 16;

-	R: Reserved bit, set to 0.

Figure 6.1.3.XX-1: Unified TCI state activation/deactivation MAC CE for mDCI based mTRP

6.2.1	MAC subheader for DL-SCH and UL-SCH

…Unnecessary part omitted…

Table 6.2.1-1a Values of two-octet eLCID for DL-SCH

| Codepoint | Index | LCID values |
| --- | --- | --- |
| 0 to (216 – 1) | 320 to (216 + 319) | Identity of the logical channel |

Table 6.2.1-1b Values of one-octet eLCID for DL-SCH

| Codepoint | Index | LCID values |
| --- | --- | --- |
| 0 to 226 | 64 to 290 | Reserved |
| 227 | 291 | Serving Cell Set based SRS TCI State Indication MAC CE |
| 228 | 292 | SP/AP SRS TCI State Indication MAC CE |
| 229 | 293 | BFD-RS Indication MAC CE |
| 230 | 294 | Differential Koffset |
| 231 | 295 | Enhanced SCell Activation/Deactivation MAC CE with one octet Ci field |
| 232 | 296 | Enhanced SCell Activation/Deactivation MAC CE with four octet Ci field |
| 233 | 297 | Unified TCI States Activation/Deactivation MAC CE |
| 234 | 298 | PUCCH Power Control Set Update for multiple TRP PUCCH repetition MAC CE |
| 235 | 299 | PUCCH spatial relation Activation/Deactivation for multiple TRP PUCCH repetition MAC CE |
| 236 | 300 | Enhanced TCI States Indication for UE-specific PDCCH |
| 237 | 301 | Positioning Measurement Gap Activation/Deactivation Command |
| 238 | 302 | PPW Activation/Deactivation Command |
| 239 | 303 | DL Tx Power Adjustment |
| 240 | 304 | Timing Case Indication |
| 241 | 305 | Child IAB-DU Restricted Beam Indication |
| 242 | 306 | Case-7 Timing advance offset |
| 243 | 307 | Provided Guard Symbols for Case-6 timing |
| 244 | 308 | Provided Guard Symbols for Case-7 timing |
| 245 | 309 | Serving Cell Set based SRS Spatial Relation Indication |
| 246 | 310 | PUSCH Pathloss Reference RS Update |
| 247 | 311 | SRS Pathloss Reference RS Update |
| 248 | 312 | Enhanced SP/AP SRS Spatial Relation Indication |
| 249 | 313 | Enhanced PUCCH Spatial Relation Activation/Deactivation |
| 250 | 314 | Enhanced TCI States Activation/Deactivation for UE-specific PDSCH |
| 251 | 315 | Duplication RLC Activation/Deactivation |
| 252 | 316 | Absolute Timing Advance Command |
| 253 | 317 | SP Positioning SRS Activation/Deactivation |
| 254 | 318 | Provided Guard Symbols |
| 255 | 319 | Timing Delta |
| XXX | XXX | Unified TCI States Activation/Deactivation MAC CE for mDCI based mTRP |

…Unnecessary part omitted…