3GPP TSG-RAN WG2 Meeting #123bis                                                 R2-2309697

Xiamen, China, 9th-13th October, 2023

Source:              CATT

Title:	             Discussion on LCID selection for feature combination

Agenda Item:	7.19.3

Document for:	Discussion and Decision

Introduction

In R18 a general CCCH LCID extension solution is supposed to be discussed in main session as below.

- 7.0.2 CCCH LCID extension Contributions should focus on general CCCH LCID extension solution (e.g. cross-WI). How to increase the signalling in RACH msg 3 and if a common solution is needed to be specified.

And besides CCCH LCID extension, another thing to address is the selection of LCID for feature combination. It may also be discussed in main session, but some initial discussion could be arranged in a WI-specific way.

Discussion

In R17 a feature combination is defined to be associated with a set of Random Access resources, and the following features are added:

FeatureCombination information element

-- ASN1START

-- TAG-FEATURECOMBINATION-START

FeatureCombination-r17 ::= SEQUENCE {

redCap-r17                 ENUMERATED {true}                                    OPTIONAL,  -- Need R

smallData-r17              ENUMERATED {true}                                    OPTIONAL,  -- Need R

nsag-r17                   NSAG-List-r17                                        OPTIONAL,  -- Need R

msg3-Repetitions-r17       ENUMERATED {true}                                    OPTIONAL,  -- Need R

spare4                     ENUMERATED {true}                                    OPTIONAL,  -- Need R

spare3                     ENUMERATED {true}                                    OPTIONAL,  -- Need R

spare2                     ENUMERATED {true}                                    OPTIONAL,  -- Need R

spare1                     ENUMERATED {true}                                    OPTIONAL   -- Need R

}

NSAG-List-r17 ::= SEQUENCE (SIZE (1.. maxSliceInfo-r17)) OF NSAG-ID-r17

-- TAG-FEATURECOMBINATION-STOP

-- ASN1STOP

When RACH is triggered by a feature combination, it means all features in this combination are initiated. And among these possible features (i.e., R17 Redcap, R17 SDT, R17 slicing, R17 coverage enhancement (Msg3 repetition)), only Redcap involves the new LCID usage for CCCH and CCCH1.

In R18 the situation is becoming more complicated. One reason is R18 Redcap and R18 coverage enhancements (Msg1 repetition) WIs both agreed to introduce a new feature in feature combination for RACH partitioning as below.

- Redcap: Add a new value “enhRedCap-r18” in FeatureCombination-r17 One FeatureCombination-r17 should not set both redCap-r17 and enhRedCap-r18 as true

Coverage enhancement： - Msg1 repetition with different repetition number {2, 4, 8} are treated a separate feature, and a RACH partition is associated with a specific repetition number (Stage 3 details are FFS, e.g. we should not use all the spare values in the current IE)

Observation 1: R18 Redcap and R18 coverage enhancements (Msg1 repetition) WIs both agreed to introduce a new feature in feature combination for RACH partitioning.

Another reason is the LCID usage for early identification requirements in Msg3 have been raised in three WIs as below (also discussed in our companion paper [1]):

PUCCH repetition for NTN

It was agreed that:

*** possible options for RAN2 discussion *** Option 1: indicate by one R bit in a MAC subheader in Msg3 MAC PDU Option 2: new LCIDs for CCCH and CCCH1 for a non-RedCap UE in Msg3 MAC PDU Option 3: a new MAC CE with fixed size of zero bits in Msg3 MAC PDU Option 4: re-purpose the spare bit currently present in the RRC messages contained in UL CCCH or UL CCCH1. - We no longer consider options 3 and 4 - RAN2 confirms that the request/capability of PUCCH repetition for Msg4 HARQ-ACK via Msg3 higher layer signaling is feasible (can rediscuss if we cannot converge on a specific solution). Proposal 1: RAN2 to discuss how to coordinate the use of the remaining LCIDs between WIs. Proposal 2: RAN2 to discuss solutions to extend the LCID values without increasing the message size. - To be discussed in the common session in the main room

In order to indicate CCCH and CCCH1 separately, 2 new LCIDs will be occupied in NTN.

In RAN#101, the following proposal was endorsed [RAN#101 chairman notes, RP-232665 Offline discussion summary on Rel-18 NR-NTN PUCCH enhancements for Msg4 HARQ-ACK]

- PUCCH repetition discussed in Rel-18 NR NTN coverage enhancement is supported for the PUCCH transmissions when dedicated PUCCH resource configuration is not provided - No additional RAN1 work to support this behavior is assumed - It is confirmed that DAI field for dynamic indication of repetition factor can indicate '1' when multiple values configured by the gNB for repetition factors include '1' - FG 44-1 is used to indicate the support of this feature

Based on the agreement above, PUCCH repetition can be applied to the common PUCCH resources when dedicated PUCCH resource configuration is not provided. In this case, the PUCCH repetition can be applied for Msg4 HARQ-ACK for CBRA where C-RNTI can be transmitted in Msg3. Therefore, one LCID indicating C-RNTI together with PUCCH repetition for Msg4 HARQ-ACK for C-RNTI should also be reserved.

Hence, if LCID is used to indicate PUCCH repetition request/capability, 3 new LCIDs should be occupied in Rel-18 NTN.

eRedCap

In Rel-18 eRedCap, it was agreed that

All R18 eRedCap UEs uses the two new LCIDs for Msg3/MsgA PUSCH for CCCH/CCCH1 during Random Access, i.e., both those with peak rate reduction + BB BW reduction, and those with only peak rate reduction.

It can be seen that 2 new LCIDs which is similar in Rel-17 redCap will be used to indicate Msg3/MsgA PUSCH for CCCH/CCCH1.

Restricted UE capability in MUSIM

In Rel-18 MUSIM, it was agreed that:

Proposal 1: During RRC setup and RRC resume procedure, UE indicates its capability is temporary restricted by sending CCCH message using dedicated LCID. Proposal 2: During RRC re-establishment procedure, UE indicates its capability is temporary restricted by sending CCCH message using dedicated LCID. Using LCIDs would avoid any problems for RRC resume procedure. However, there are not many LCIDs left for UL and some other Rel-18 WIs also intend to use them. FFS whether there is a need to use the LCIDs or whether we can reuse the legacy LCIDs. Whether we can use the LCIDs (given that multiple WIs may be trying to use them) will be discussed in the main session. How to proceed LCID usage for MUSIM can be discussed in the next meeting based on the main session decision.

Similar to eRedCap, in order to distinguish CCCH and CCCH1 in MUSIM, 2 new LCIDs will be occupied.

Observation 2: R18 NTN/eRedcap/MUSIM intended to introduce new LCIDs for early identification in Msg3.

According to the analysis above, currently we have in total 6 features for RACH partitioning, i.e., RedCap, eRedcap, Slicing, SDT, MSG1-Repetitions for Coverage Enhancements and MSG3-Repetitions for Coverage Enhancements. And there are four features that need to use specific LCID in Msg3 for early identification purpose as illustrated in Figure 1.

Figure 1 possible feature combination

In main session the discussion point is, considering we have to run out of LCID resources if we define all new LCIDs as required, whether to enlarge the LCID space. And the potential/subsequent question is whether to introduce separate LCIDs for different feature combinations, as there could be a LCID collision in Msg3.

There are three cases for discussion:

Case 1: R18 Msg1 repetition + R18 NTN, and there is a RACH partition for R18 Msg1 repetition.

In this case, Msg1 repetition can be identified by network based on specific RACH resource, and NTN PUCCH repetition request can be indicated in Msg3 separately. There are no further issues in this case.

Case 2: R18 eRedcap + R18 NTN, and there is a RACH partition for R18 eRedcap.

In this case, even if there is specific RACH resource for R18 eRedcap early identification, the UE is still supposed to include eRedcap specific LCID in Msg3. And if UE also needs to indicate PUCCH repetition request, there is a collision issue regarding LCID selection. But since eRadcap has been indicated in Msg1, a workaround could be only NTN PUCCH repetition request is indicated in Msg3.

Proposal 1: RAN2 to discuss when there is dedicated Msg1 resource for (e)Redcap, whether to only include LCID associated to other feature in Msg3 in case of a feature combination (e.g., R18 eRedcap + R18 NTN).

Case 3: R18 eRedcap + R18 NTN, and there is NO RACH partition for R18 eRedcap, or, in case of R18 NTN + R18 MUSIM.

Since there is no specific RACH resource for Msg1 transmission, all early identification indications have to be included in Msg3, which means there will be a collision of LCID selection in Msg3 for a feature combination. If there is no specific LCID for a feature combination, RAN2 needs to discuss whether to introduce a priority for LCID selection.

Proposal 2: RAN2 to discuss how to address the collision of LCID selection in Msg3 for a feature combination.

Conclusion

In this paper, we discuss the LCID selection for feature combination with the following observations:

Observation 1: R18 Redcap and R18 coverage enhancements (Msg1 repetition) WIs both agreed to introduce a new feature in feature combination for RACH partitioning.

Observation 2: R18 NTN/eRedcap/MUSIM intended to introduce new LCIDs for early identification in Msg3.

And we propose:

Proposal 1: RAN2 to discuss when there is dedicated Msg1 resource for (e)Redcap, whether to only include LCID associated to other feature in Msg3 in case of a feature combination (e.g., R18 eRedcap + R18 NTN).

Proposal 2: RAN2 to discuss how to address the collision of LCID selection in Msg3 for a feature combination.

Reference

- R2-2309754, Discussion on CCCH LCID extension, CATT