3GPP TSG-RAN WG2 Meeting #123bis		R2-2309720

Xiamen, China, 9th Oct.- 13th Oct. 2023

Source:	vivo

Title:	RRC configuration for LTM

Agenda Item:	7.4.2.1

Document for:	Discussion and Decision

- Introduction

In the RAN2 reflector, the rapporteur has listed the following remaining open issues for LTM.

L1/L2 based inter-cell mobility - RAN2 assumes for RACH-less LTM, the UE determines successful reception of its first UL data based on receiving a PDCCH addressing the UE’s C-RNTI in the target cell scheduling a new transmission after the first UL data, FFS if specified contents should be transmitted with this transmission, e.g. as LTE MAC CE (). - FFS if UE transmits the preamble without the power ramping upon reception of PDCCH order with retransmission indication if preamble transmission encounter the LBT failure. - Security concerns for LTM when using L1/L2 signalling in L1 measurement report or LTM trigger command. - FFS further optimization if configured grant can be used for RACH-less LTM - For RRC reconfiguration with usage of reference configuration - FFS if more than RLC PDCP should be kept and how much of “replacing” need to be specified - FFS how to make sure the procedures work in case the LTM candidate configuration is a complete configuration. - Details of stage-3 RRC issues - How to apply candidate delta configuration on top of reference configuration (new procedure or reuse delta configuration, Need Code meaning updates) - Whether to reuse reconfigurationWithSync - How to describe the sending of RRCReconfigurationComplete - RRC configurations for L1 measurement and reporting, TCI state, and TA management, following RAN1 RRC parameter list - Other RRC configuration details (e.g., early TA acquisition, BWP info, etc.) - Details of MAC CEs for LTM command - MAC CE format - CFRA related info - TCI state for SCells - FFS whether introduce a new filed in LTM cell switch command MAC CE for the case that target cell has same TA value as the source cell - FFS how to indicate UE to keep/release the LTM configurations when L3 handover occurs - FFS how to handle RLC and PDCP entities when LTM occurs for intra/inter-DU LTM

In this paper, we will discuss the remaining RRC configuration issues for LTM that were highlighted above .

- Discussion

  - Reference configuration

The reference configuration is a common template configuration for all candidate cells, and each candidate cell configuration is a delta configuration of the reference. Therefore, the delta configuration of a candidate cell should be able to change or replace the configuration provided in the reference configuration.

Regarding the fields included in the reference configuration, we believe that fields with “need R/S/N” should not be included. For example, if fields with “need R” are provided in the reference configuration, the network will not be able to release these configurations through candidate CG configuration in delta configuration manner because the merged complete configuration will always include these fields. Configurations with “need S” and “need N” have similar problem with “need R”. Once the configurations are provided in the reference configuration, the network cannot make the configurations absent in the complete configuration through candidate CG configuration in delta configuration manner.

Moreover, for “need N”, ToAddModList may be an exception because both the radio bearer configuration and the SCell configuration are provided using the ToAddModList. Including this information in the reference configuration can greatly reduce the signaling overhead. However, on the one hand, it’s not allowed to provide an update to a previously signaled ToAddModList via delta signaling in the current specification. On the other hand, the network will not be able to make the ToAddModList absent in the complete candidate configuration through delta configuration, as we described above. For example, if sCellToAddModList exists in the reference configuration (i.e., CA is configured), the network cannot provide a candidate configuration without SCell. Therefore, whether ToAddModList can be included in the reference configuration and how to handle it should be further discussed.

Proposal 1：For fields included in the reference configuration:

- Configurations with “need R” and “need S” should not be provided in the reference configuration.

- Except for ToAddModList, configurations with “need N” should not be provided in the reference configuration.

- Whether ToAddModList can be provided in the reference configuration and how to handle it should be further discussed.

  - Other configuration

In the last meeting [1], it was agreed that UE should release the radio bearers that are part of the current UE configuration but not part of the target LTM candidate cell configuration.

Upon an LTM cell switch, the UE releases the radio bearer related configuration. Is up to network to provide the radio bearer configuration either within the reference configuration or within the LTM candidate cell configuration. Upon an LTM cell switch, the UE shall release the radio bearer that are part of the current UE configuration but not part of the target LTM candidate cell configuration.

However, for other configurations unrelated to L2 reset, some companies proposed that at least MeasConfig can be kept because the IE does not have to change with LTM execution.

In our view, the complete LTM target cell configuration includes all necessary informations needed to perform the LTM cell switch. Therefore, for other configurations unrelated to L2 reset, we think UE can simply release the current UE configuration and then apply the corresponding fields in the complete LTM target cell configuration, no matter the target LTM configuration is the same as the source configuration or not.

Proposal 2：For other configurations unrelated to L2 reset, UE should release the current UE configuration and then apply the corresponding fields in the complete LTM target cell configuration.

  - L2 handling during LTM cell switch

In RAN2#121, it was agreed that whether to reset L2 or not is based on RRC configuration:

- To determine if to reset L2 or not is based on RRC configuration (e.g. set of cells. FFS if separate for RLC, MAC, PDCP).

For subsequent LTM, a candidate cell may be in the same DU with the serving cell at one time, and the candidate cell may be in a different DU with the serving cell at another time. Therefore, a single candidate configuration should support both intra-DU LTM and inter-DU LTM scenarios.

To support subsequent LTM without RRC reconfiguration, several cell sets, which are configured external to the configurations of serving and candidate cells, can be provided to the UE. Each cell set includes identities of cells that belong to the same DU and UE determines whether to perform L2 reset based on whether the source and target cell belong to the same cell set.

Proposal 3: Several cell sets, each including a list of cell identities, are pre-configured to UE. UE determines whether to perform L2 reset based on whether the source and target cell belong to the same cell set.

Proposal 4: The cell sets are configured externally to the configurations of serving and candidate cells.

The L2 operation for the source cell and target cell in the same cell set or different cell sets during LTM may work as below:

- For MAC: UE performs MAC reset.

- For RLC: If the source PCell and target PCell are in the same cell set, UE performs RLC reestablishment for SRB and does not perform RLC reestablishment for DRB. Otherwise, UE performs RLC reestablishment for SRB and DRB.

- For PDCP

  - For SRB: UE always performs PDCP discard.

For DRB: If the source PCell and target PCell are in the same cell set, UE does not perform PDCP data recovery. Otherwise, whether to perform PDCP data recovery is indicated by the existing IE recoverPDCP.

Proposal 5：When performing LTM, if the source cell and the target cell are in the same cell set:

- For MAC: UE performs MAC reset.

- For RLC: UE performs RLC reestablishment for SRB and does not perform RLC reestablishment for DRB.

- For PDCP

  - For SRB: UE always performs PDCP discard.

  - For DRB: UE does not perform PDCP data recovery.

Proposal 6：When performing LTM, if the source cell and the target cell are in different cell sets:

- For MAC: UE performs MAC reset.

- For RLC: UE performs RLC re-establishment

- For PDCP

  - For SRB: UE performs PDCP discard.

  - For DRB: UE determines whether to perform PDCP data recovery according to the existing IE recoverPDCP for the DRB

  - Maintenance of candidate configurations

Release LTM when inter-CU handover happens

Since R18 LTM only supports intra-CU handover, the LTM candidate cells in the source CU should be released when an inter-CU L3 handover occurs and should not be released when an intra-CU L3 handover occurs.

Observation 1: LTM candidate cell configuration needs to be released when an inter-CU handover occurs.

However,  UE cannot distinguish intra-CU or inter-CU handover. Therefore, we propose introducing an indication in the RRCReconfiguration message to inform the UE whether to keep or release the LTM configuration. When an L3 handover occurs, UE shall release the stored LTM configurations if the indication is not included in the handover command/RRCReconfiguration.

Proposal 7: To indicate UE to keep/release the stored LTM configuration during L3 handover, introduce an indication in the RRCReconfiguraiton message. For RRCReconfiguration that includes ReconfigurationWithSync, if the new indication is present, the UE keeps the stored LTM configurations, otherwise, the UE releases the stored LTM configurations.

Keep LTM after LTM fast recovery

In the last meeting, it was agreed that UE would perform fast recovery similar to CHO upon an LTM cell switch failure or RLF.

Upon an LTM cell switch failure (i.e., supervision timer expiry) or RLF, fast recovery similar to CHO: - a) UE performs cell selection. - b) If selected cell is an LTM candidate cell, UE performs RACH-based LTM cell switch on the selected cell (network-controlled). - c) If selected cell is not an LTM candidate cell, UE transmits RRC re-establishment request.

However, it is unclear whether the UE should release the LTM candidate cells after performing fast recovery. We believe that in case b) above, UE should keep all LTM candidate cell configurations. We explain the reason from the following two aspects.

Firstly, LTM fast recovery is still a type of LTM cell switching and should be considered the same as receiving an LTM cell switch command. If the LTM candidate cell configurations are kept after LTM fast recovery, they can be used for subsequent LTM or LTM fast recovery in the future.

Secondly, if the UE decides to release the LTM candidate cell configurations after performing LTM fast recovery, the network may not know whether the triggered LTM cell switch is LTM fast recovery or not and may still keep the LTM candidate cells in the network, causing misunderstanding between UE and the network. For example, if the UE fails to receive the LTM cell switch command from the source DU and an RLF occurs, UE will perform cell selection. If the selected cell is the same as indicated in the LTM cell switch command, then the network cannot distinguish whether it is a normal LTM cell switch or an LTM fast recovery. Thus, the network cannot determine whether to keep or release the LTM configurations. To avoid such mismatch issues, UE should not release the LTM candidate cell configurations if LTM fast recovery is completed successfully.

Proposal 8: UE shall not release the stored LTM candidate cell configurations if LTM fast recovery is completed successfully.

  - Association between CG occasion and beam

In RAN2#122, it was agreed that the configured grant can be used for RACH-less LTM, and the configured grant occasion is associated with beam(s) similar to CG-SDT. In Rel-17 SDT, the mapping ratio between CG occasion and SSB includes 1/8, 1/4, 1/2, 1, 2, 4, 8, and 16. If the mapping ratio with values of smaller than 1 is also supported in LTM, one CG occasion would be associated with multiple SSBs. In this case, the target SpCell cannot determine the DL beam according to the CG occasion carrying the first UL message. To solve this issue, the beam-related information indicated in the cell switch command, i.e., TCI state index or a pair of TCI state and TCI-UL-State index, needs to be carried in the CG PUSCH to be transmitted to the target SpCell.

Proposal 9: Introduce a mapping rule between SSB within a SSB resource set and CG occasions for LTM to inform the target cell of beam-related information. The SSB resource set is made up of SSB corresponding to QCL source RS index included in TCI states.

-      When the mapping ratio between CG occasion and SSB is smaller than 1, the TCI state index or a pair of TCI state and TCI-UL-State index needs to be carried in the CG PUSCH to inform the network.

- Conclusion

In this contribution, we provide our view on some issues related to RRC. We have the following observations and proposals:

Reference configuraiton

Proposal 1：For fields included in the reference configuration:

- Configurations with “need R” and “need S” should not be provided in the reference configuration.

- Except for ToAddModList, configurations with “need N” should not be provided in the reference configuration.

- Whether ToAddModList can be provided in the reference configuration and how to handle it should be further discussed.

Other configuraiton

Proposal 2：For other configurations unrelated to L2 reset, UE should release the current UE configuration and then apply the corresponding fields in the complete LTM target cell configuration.

L2 handling during LTM cell switch

Proposal 3: Several cell sets, each including a list of cell identities, are pre-configured to UE. UE determines whether to perform L2 reset based on whether the source and target cell belong to the same cell set.

Proposal 4: The cell sets are configured externally to the configurations of serving and candidate cells.

Proposal 5：When performing LTM, if the source cell and the target cell are in the same cell set:

- For MAC: UE performs MAC reset.

- For RLC: UE performs RLC reestablishment for SRB and does not perform RLC reestablishment for DRB.

- For PDCP

  - For SRB: UE always performs PDCP discard.

  - For DRB: UE does not perform PDCP data recovery.

Proposal 6：When performing LTM, if the source cell and the target cell are in different cell sets:

- For MAC: UE performs MAC reset.

- For RLC: UE performs RLC re-establishment

- For PDCP

  - For SRB: UE performs PDCP discard.

  - For DRB: UE determines whether to perform PDCP data recovery according to the existing IE recoverPDCP for the DRB

Maintenance of candidate configurations

Observation 1: LTM candidate cell configuration needs to be released when an inter-CU handover occurs.

Proposal 7: To indicate UE to keep/release the stored LTM configuration during L3 handover, introduce an indication in the RRCReconfiguraiton message. For RRCReconfiguration that includes ReconfigurationWithSync, if the new indication is present, the UE keeps the stored LTM configurations, otherwise, the UE releases the stored LTM configurations.

Proposal 8: UE shall not release the stored LTM candidate cell configurations if LTM fast recovery is completed successfully.

Association between CG occasion and beam

Proposal 9: Introduce a mapping rule between SSB within a SSB resource set and CG occasions for LTM to inform the target cell of beam-related information. The SSB resource set is made up of SSB corresponding to QCL source RS index included in TCI states.

-      When the mapping ratio between CG occasion and SSB is smaller than 1, the TCI state index or a pair of TCI state and TCI-UL-State index needs to be carried in the CG PUSCH to inform the network.

- References

- R2_123 Chair Notes (MediaTek)