3GPP TSG-RAN WG2 Meeting #123bis		R2-2309740

Xiamen, China, 9th Oct. – 13th Oct. 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.306 | CR | 0953 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of support for BWP operation without restriction |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo, Vodafone, ZTE Corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_BWP_wor-Core |  |  |  |  |  | Date: |  |  | 2023-09-29 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | [to be updated] Complete the specification support for BWP operation without restriction (Option C, Option B-1-1, and Option B-1-2) as agreed in work item RP-232677. According to the UE feature list provided from RAN1 in R1-2308521, the corresponding UE capabilities should be added. Besides, RAN4 has agreed that UE shall not indicate support of both B-1-1 and B-1-2 for the same agreed granularity reporting. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To be updated. Introduction of the corresponding UE capability signalling |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification does not support BWP operation without restriction for Option C, Option B-1-1, and Option B-1-2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.7.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR 38.331 CR TBD TS/TR 38.300 CR TBD |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR is based on the latest Release 17 specification version 17.5.0, and it will be implemented in Release 18 specification when available. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of change

4.2.7.2	BandNR parameters

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| [unchanged text omitted] |  |  |  |  |
| bwp-DiffNumerology Indicates whether the UE supports BWP adaptation up to 4 BWPs with the different numerologies, via DCI and timer. Except for SUL, the UE only supports the same numerology for the active UL and DL BWP. For the UE which is a non-RedCap UE capable of this feature, the bandwidth of a UE-specific RRC configured DL BWP includes the bandwidth of the CORESET#0 (if CORESET#0 is present) and SSB for PCell and PSCell (if configured). For the UE which is a RedCap UE capable of this feature, the bandwidth of a UE-specific RRC configured DL BWP may not include the bandwidth of the CORESET#0 (if configured) and SSB for PCell. For SCell(s), the bandwidth of the UE-specific RRC configured DL BWP includes SSB, if there is SSB on SCell(s). | Band | No | N/A | N/A |
| bwpOperationWithoutInterruption-r18 Indicates whether the UE supports RLM/BM/BFD measurements based on CD-SSB without interruptions, where the CD-SSB is outside active DL BWP but is within the bandwidth of the corresponding carrier(s) to be measured. Bandwidth of UE-specific RRC configured BWP may not include bandwidth of the CORESET#0 (if CORESET#0 is present) and CD-SSB for PCell/PSCell (if configured) and bandwidth of the UE-specific RRC configured BWP may not include CD-SSB for SCell. CD-SSB outside active DL BWP but within the bandwidth of the corresponding carrier(s) to be measured can be used as the QCL source for other reference signal. UE performs L3 intra-frequency measurements without gaps based on CD-SSB, where the CD-SSB is outside the active DL BWP but is within the bandwidth of the corresponding carrier(s) to be measured. If a UE is configured with more than one UE-specific DL BWP configurations, the CD-SSB is within the bandwidth of at least one of the UE-specific DL BWP configurations. It only applies if there is no CSI-RS, no NCD- SSB, and no CD-SSB configured for RLM/BM/BFD in the active BWP of the corresponding carrier(s) to be measured. This capability is not applicable for RedCap UEs. A UE indicates at most one of bwpOperationWithoutInterruption-r18 and bwpOperationWithInterruption-r18 per BC. NOTE: The CD-SSB is still within the bandwidth of the carrier configured by SCS-SpecificCarrier of downlinkChannelBW-PerSCS-List in ServingCellConfig | FS | No | N/A | N/A |
| bwpOperationWithInterruption-r18 Indicates whether UE supports RLM/BM/BFD measurements based on CD-SSB outside the active BWP with interruptions, where the CD-SSB is outside active DL BWP but is within the bandwidth of the corresponding carrier(s) to be measured. Bandwidth of UE-specific RRC configured BWP may not include bandwidth of the CORESET#0 (if CORESET#0 is present) and CD-SSB for PCell/PSCell (if configured) and bandwidth of the UE-specific RRC configured BWP may not include CD-SSB for Scell. CD-SSB outside active DL BWP but within the bandwidth of the corresponding carrier(s) to be measured can be used as the QCL source for other reference signal. If a UE is configured with more than one UE-specific DL BWP configurations, the CD-SSB is within the bandwidth of at least one of the UE-specific DL BWP configurations. It only applies if there is no CSI-RS, no NCD- SSB, and no CD-SSB configured for RLM/BM/BFD in the active BWP of the corresponding carrier(s) to be measured. This capability is not applicable for RedCap UEs. A UE indicates at most one of bwpOperationWithoutInterruption-r18 and bwpOperationWithInterruption-r18 per BC. NOTE: with ncd-SSB-BWP-Wor-r18 as prerequisite NOTE: The CD-SSB is still within the bandwidth of the carrier configured by SCS-SpecificCarrier of downlinkChannelBW-PerSCS-List in ServingCellConfig | FS | No | N/A | N/A |
| [unchanged text omitted] |  |  |  |  |
| nack-OnlyFeedbackForSPS-MulticastWithDCI-Enabler-r17 Indicates whether the UE supports DCI-based enabling/disabling NACK-only based HARQ-ACK feedback configured per G-CS-RNTI by RRC signalling via DCI format 4_2. A UE that indicates support of this feature shall indicate support of nack-OnlyFeedbackForSPS-Multicast-r17 and sps-MulticastDCI-Format4-2-r17. | Band | No | N/A | N/A |
| ncd-SSB-BWP-Wor-r18 Indicates whether the UE supports RLM/BM/BFD and gapless L3 intra-frequency measurements based on NCD-SSB, where the NCD-SSB is within the active DL BWP. Bandwidth of UE-specific RRC configured BWP may not include bandwidth of the CORESET#0 (if CORESET#0 is present) and CD-SSB for PCell/PSCell (if configured) and bandwidth of the UE-specific RRC configured BWP may not include CD-SSB for Scell. NCD-SSB within the active DL BWP can be used as the QCL source for other reference signal. UE performs L3 intra-frequency measurements without gaps based on NCD-SSB, where the NCD-SSB is within the active DL BWP. This capability is not applicable for RedCap UEs. | Band | No | N/A | N/A |
| [unchanged text omitted] |  |  |  |  |

End of change