3GPP TSG-RAN WG2 Meeting #123bis	R2-2309744

Xiamen, China, 9th – 13th Oct., 2023

Agenda Item:	7.15.2

Source:	vivo

Title:	Remaining issues on SL LBT failure

Document for:	Discussion and Decision

# Introduction

In this contribution, we will discuss about the remaining issues on SL C-LBT-F and address the RAN4 LS on S-SSB.

# Discussion

## Recovery for connected mode-2 UE

During R2#122 & R2#123, the following agreement is made [1][2]:

R2#122 Agreements on SL C-LBT failure recovery (mode 2, RRC connected UE) 1: RAN2 confirms that SL C-LBT failure indication is reported to the gNB also for mode 2, RRC connected UE. R2#123 Agreements on SL C-LBT failure recovery (RRC connected mode 2) 1: C-LBT failure recovery for RRC idle/inactive mode 2 is applied. Agreements on C-LBT failure cancellation conditions 1: Upon MAC reset. 2: Upon C-LBT count and/or timer reconfiguration. 3: Based on a timer expiry (the timer starts upon C-LBT failure)

For the cancellation conditions agreed above, MAC reset and C-LBT count and/or timer reconfiguration are two general conditions. The timer-based recovery is up to now the only cancellation condition for IDLE/INACTIVE mode 2 UE. Some other cancellation conditions were also mentioned, e.g. upon pool reselection, upon RRC reconfiguration for resource pool, upon mode change, etc. However, with the agreed cancellation conditions for mode 1 UE and IDLE/INACTIVE mode 2 UE, there is no need to define further cancellation conditions. In other words, SL C-LBT-F recovery can be covered by the behaviors of mode 1 and/or IDLE/INACTIVE mode 2 UE.

Proposal 1: With the agreed cancellation conditions for mode 1 UE, i.e. upon SL LBT MAC CE reporting and for IDLE/INACTIVE mode 2 UE, i.e. upon recovery timer expiry, there is no need to define further cancellation conditions.

The only remaining issue for CONNECTED mode-2 UE is when to cancel SL C-LBT-F, since SL LBT MAC CE reporting and timer-based recovery are both supported. According to the contributions so far, many options for connected mode-2 UE to cancel SL C-LBT-F are mentioned, including at least the following major ones:

- Opt. 1: the SL LBT MAC CE is reported;

- Opt. 2: the recovery timer expires;

- Opt. 3: the SL LBT MAC CE is reported, or the recovery timer expires;

- Opt. 4: the SL LBT MAC CE is reported, and the recovery timer expires;

According to R2#122, when SL LBT MAC CE is reported by mode 1 UE, it is up to the gNB how to handle it, i.e. the gNB can reconfigure the resources for the UE, or do nothing [1]:

Agreements on SL C-LBT failure recovery (mode 1) 1: Leave it to gNB implementation after UE reporting SL C-LBT failure indication. No spec change.

Similarly, when SL LBT MAC CE is reported by connected mode 2 UE, it should also be up to the gNB how to handle it. For a connected mode 2 UE, since it is agreed to follow IDLE/INACTIVE mode 2 UE behavior, there is no point to cancel C-LBT-F with only the condition of SL LBT MAC CE reporting, as the UE can autonomously performs resource reselection and pool reselection, while the gNB may not reconfigure the resources. Therefore, Opt. 1 and Opt. 3 are not preferred. Although connected mode 2 UE supports both SL LBT MAC CE reporting and timer-based recovery, it is not certain which event happens earlier, i.e. the SL LBT MAC CE reporting, or the recovery timer expiry. For Opt. 4, if the available resource for reporting SL LBT MAC CE is much later than the time when the recovery timer expires, the UE has to wait for quite some extra time to be able to use the resources again for which SL C-LBT-F has been triggered. Therefore, Opt. 4 is also not preferred. In sum, we propose connected mode 2 UE to follow Opt. 2 for SL C-LBT-F cancellation, which follows the logic of IDLE/INACTIVE mode 2 UE and is aligned with the previous agreement achieved in R2#123.

Proposal 2: For CONNECTED mode 2 UE, the UE cancels SL consistent LBT failure upon the recovery timer expiry, which is the same as IDLE/INACTIVE mode 2 UE.

## SL grant containing RB set that SL C-LBT-F is triggered

According to R2#122, RAN2 has agreed that L1 performs the resource exclusion for the RB set that SL C-LBT failure was detected for an IDLE/INACTIVE mode 2 UE [1]. In other words, the UE shall not generate a selected SL grant containing RB set(s) for which SL C-LBT-F has been triggered, when the recovery timer for the RB set(s) is running. Up to now, RAN2 has never discussed about whether and how to handle the generated selected SL grant containing RB set(s) for which SL C-LBT-F has been triggered. From our perspective, the UE behavior for the selected SL grant containing RB set(s) for which SL C-LBT-F has been triggered should be consistent, regardless whether the selected SL grant has been generated or not.

Take the below Fig.1 as an example: UE already reserved the SL resources via a selected SL grant at T1, but later SL consistent LBT failure is triggered at T2 on a part of the SL RB set(s) spanned by the reserved SL resources, i.e. SL RB set 0 in Case A and SL RB set 1 in Case B.

Theoretically speaking, although the UE is still able to use remaining SL resources on the other SL RB set(s) for subsequent transmissions, it seems questionable whether the remaining resources are still sufficient for the data available for further transmission, and it is also questionable how/whether the UE shall adjust the MCS/TBS to adapt to the buffer status and whether the adjustment will degrade the transmission performance. Also, the adaptation on transmission parameters (e.g. MCS, TX power, etc.) also depend on the remaining SL resources usable for further transmission, leading to potentially differential handling and thus bringing in additional UE implementation complexity. All these aspects may also depend on RAN1’s judgement on whether to keep using the remaining resources are really feasible/preferable or not.

Fig. 1: Example for impact on selected SL grant handling due to SL consistent LBT failure

To this end, the simplest way is to not allow the UE to transmit via the selected SL grant anymore and ask the UE to clear the selected SL grant, in case SL consistent LBT failure is detected on any SL RB set(s) spanned by the SL resources reserved by the grant. Since CONNECTED mode 2 UE is agreed to apply C-LBT failure recovery for IDLE/INACTIVE mode 2 UE, we propose all CONNECTED/IDLE/INACTIVE mode 2 UE to apply the same behavior.

Proposal 3: A mode 2 UE shall clear the selected SL grant(s), in case SL consistent LBT failure is triggered on any SL RB set(s) spanned by the SL resources reserved by the selected SL grant(s).

## Impact to S-SSB/PSFCH transmission

### impact to S-SSB transmission due to SL consistent LBT failure

In R2#122, it was agreed that the UE should count LBT failure indication regardless of whether LBT failure was provided because of S-SSB transmission or data transmission when RB set for S-SSB transmission belongs to the selected TX resource pool [1]. In R2#123, RAN2 agrees not to handle S-SSB that does not belong to the selected SL resource pool unless RAN1 asks [2].

However, after checking RAN1 endorsed CR to TS38.214 [3], we found there is no use case for the agreement achieved in R2#122 for S-SSB. According to the endorsed CR, the slots for legacy S-SSB and the additional S-SSB transmission occasions are not belonging to any resource pool:

The set of slots that may belong to a sidelink resource pool is denoted by (t0SL,t1SL,⋯,tTmax-1SL) where - 0≤tiSL<10240×2μ, 0≤i<Tmax, - the slot index is relative to slot#0 of the radio frame corresponding to SFN 0 of the serving cell or DFN 0, - the set includes all the slots except the following slots, - NS-SSB slots in which S-SS/PSBCH block (S-SSB) or additional transmission occasion for S-SSB is configured,

Observation 1: Slots for legacy S-SSB and additional S-SSB transmission occasion are not belonging to any resource pool.

On one hand, there is no case that the UE counts LBT failure indication for C-LBT-F detection due to S-SSB transmission within the selected TX resource pool. On the other hand, since C-LBT-F can only be detected on the RB set(s) within the selected TX resource pool, there is no case that C-LBT-F can impact the S-SSB transmission from RAN2 perspective. Therefore, we propose RAN2 to confirm S-SSB transmission is not impacted by SL consistent LBT failure from RAN2’s perspective.

Proposal 4: RAN2 to confirm S-SSB transmission is not impacted by SL consistent LBT failure because SL consistent LBT failure can only be detected on the selected TX resource pool while slots for S-SSB transmission occasion do not belong to any resource pool.

### impact to PSFCH transmission due to SL consistent LBT failure

For PSFCH, whose transmission resources are determined by the reception of PSSCH and cannot be arbitrarily selected by the receiving UE as it wishes, forbidding the PSFCH transmission on the associated RB set where SL consistent LBT failure is detected may result in some possible problems such as DTX issue. Therefore, transmission of PSFCH should not be impacted, i.e. the UE can still try to transmit PSFCH on the problematic RB set. The worst case is merely that the UE fails to transmit PSFCH on all the PSFCH occasions, as if the case that regular LBT failure is detected. Hence, we propose that PSFCH transmission within the selected TX resource pool is not impacted by SL consistent LBT failure, since SL consistent LBT failure can only be detected on the selected TX resource pool.

Proposal 5: RAN2 to confirm PSFCH transmission within the selected TX resource pool is not impacted by SL consistent LBT failure, i.e. the UE can still transmit PSFCH on the associated SL RB set(s) where SL consistent LBT failure is detected.

### on addressing RAN4 LS about SLSS transmission

During R4#108, LBT failure impact to S-SSB transmission was discussed in different terms, i.e. CCA failure impact to SLSS transmission, and an LS is sent to RAN2 [4]:

RAN4 has discussed RRM requirements for following sidelink procedure: - Procedure for initiation/cease of SLSS transmissions Based on the discussion, RAN4 has agreed to define measurement delay requirements as function of CCA failures (L1) during the evaluation period (Tevaluate,SLSS_CCA), where - L1 is the number of occasions containing sidelink synchronization reference signal of the SyncRef UE not available at the UE during Tevaluate,SLSS_CCA due to the CCA failures; L1, max is the maximum number of allowed occasions containing sidelink synchronization reference signal of the SyncRef UE not available at the UE during Tevaluate,SLSS_CCA due to the CCA failures where L1 ≤ L1, max. When the extension of evaluation period reaches L1, max, the number of available occasions containing sidelink synchronization reference signals during Tevaluate,SLSS_CCA is not enough to derive an accurate PSBCH-RSRP measurement result, which cannot be used to compared with syncTxThreshO and futher determine whether to initiate/cease SLSS transmission. RAN4 would like to ask RAN2 which of the following options regarding the UE behaviour is reasonable from RAN2 perspective: - The UE measures PSBCH-RSRP and evaluates whether to initiate/cease SLSS transmissions within Tevaluate,SLSS_CAA, and upon exceeding the maximum allowed CCA failures during Tevaluate,SLSS_CCA, the UE shall: - Opton 1: cease all SLSS transmissions, - Option 2: initiate SLSS transmissions - Option 3: UE keeps current SLSS transmission status.

The situation is quite similar to the one discussed for SL C-LBT-F with some difference:

(1) This case is only about SLSS transmission, while SL C-LBT-F is detected with PSSCH/PSCCH/PSFCH transmission;

(2) This case is evaluated in a moving window Tevaluate,SLSS_CAA with fixed-length, while SL C-LBT-F is evaluated in a window with variable-length starting from a slot where the first LBT failure is detected when SL_LBT_COUNTER = 0;

(3) The UE in this case evaluates PSBCH-RSRP of SLSS transmission from other UEs to determine whether it should perform SLSS transmission, while the UE detecting SL C-LBT-F evaluates general RSRP of the channel for transmission of its own PSSCH/PSCCH/PSFCH.

The above options from RAN4 LS are about whether and how the UEs influence the SL synchronization performance of the system.

- Option 1: cease all SLSS transmissions

In this option, since the UEs around may be suffering from severe interference, the SLSS monitoring UE pursues energy saving by not trying to perform SLSS transmission with low success rate. However, the overall synchronization performance of the system can be very bad if option 1 is adopted. Therefore, option 1 is not reasonable.

- Option 2: initiate SLSS transmissions

In this option, the SLSS monitoring UE strive to improve the overall synchronization performance of the system by initiating SLSS transmission once it detects certain number of CCA failures of the UEs around within the evaluation window, with sacrifice of its own energy.

However, there is a case where the SLSS monitoring UE is moving away from the synchronization cluster and thus detects such event. If the SLSS monitoring UE initiates SLSS transmission in this case, it is not improving anything as there may be no peer UEs around.

In sum, Option 2 is also not preferred due to UE energy consumption and use case flaw.

- Option 3: UE keeps current SLSS transmission status.

Option 3 means the UE does not pursue energy saving or improving system synchronization performance when the channel is busy. With option 3, the system synchronization performance is relatively worse compared with that in the licensed band. Considering the defects mentioned above for Option 1 and Option 2, Option 3 seems to the only choice to pursue.

Proposal 6: On replying RAN4 LS, RAN2 to confirm Option 3 is reasonable from RAN2 perspective, i.e. upon exceeding the maximum allowed CCA failures during Tevaluate,SLSS_CCA, the UE shall keep current SLSS transmission status.

# Conclusion

Based on the discussion, we have the following observation and proposals:

Proposal 1: With the agreed cancellation conditions for mode 1 UE, i.e. upon SL LBT MAC CE reporting and for IDLE/INACTIVE mode 2 UE, i.e. upon recovery timer expiry, there is no need to define further cancellation conditions.

Proposal 2: For CONNECTED mode 2 UE, the UE cancels SL consistent LBT failure upon the recovery timer expiry, which is the same as IDLE/INACTIVE mode 2 UE.

Proposal 3: A mode 2 UE shall clear the selected SL grant(s), in case SL consistent LBT failure is triggered on any SL RB set(s) spanned by the SL resources reserved by the selected SL grant(s).

Observation 1: Slots for legacy S-SSB and additional S-SSB transmission occasion are not belonging to any resource pool.

Proposal 4: RAN2 to confirm S-SSB transmission is not impacted by SL consistent LBT failure because SL consistent LBT failure can only be detected on the selected TX resource pool while slots for S-SSB transmission occasion do not belong to any resource pool.

Proposal 5: RAN2 to confirm PSFCH transmission within the selected TX resource pool is not impacted by SL consistent LBT failure, i.e. the UE can still transmit PSFCH on the associated SL RB set(s) where SL consistent LBT failure is detected.

Proposal 6: On replying RAN4 LS, RAN2 to confirm Option 3 is reasonable from RAN2 perspective, i.e. upon exceeding the maximum allowed CCA failures during Tevaluate,SLSS_CCA, the UE shall keep current SLSS transmission status.

# References

- R2#122 meeting minutes

- R2#123 meeting minutes

- R1-2308720 Introduction of specification enhancements for NR sidelink evolution

- R4-2314351 LS on NR SL unlicensed LBT failures UE behavior