3GPP TSG-RAN WG2 Meeting #123bis              	R2-2309792

Xiamen, CN, 9th – 13th Oct, 2023

Agenda Item:	7.17.3

Source: 	vivo

Title:         	Discussion on temporary capability restriction for Rel-18 Multi-SIM

Document for: 	Discussion and Decision

# Introduction

In Rel-18 MUSIM WI, the below objective was agreed in [1].

1. Enhancements for MUSIM procedures to operate in RRC_CONNECTED state simultaneously in NW A and NW B. [RAN2, RAN3, RAN4]. - Specify mechanism to indicate preference on temporary UE capability restriction and removal of restriction (e.g. capability update, release of cells, (de)activation of configured resources) with NW A when UE needs transmission or reception (e.g., start/stop connection to NW B) for MUSIM purpose - RAT Concurrency: Network A is NR SA (with CA) or NR DC. Network B can either be LTE or NR. - Applicable UE architecture: Dual-RX/Dual-TX UE

In the previous meetings, RAN2 has identified several capabilities (i.e., MIMO layer, DC/CA, and measurement gap requirement) that can be changed due to dual active MUSIM. The reporting of these changed capabilities can be used to request the release of the specific configurations, or to avoid receiving an unexpected configuration in the future. Therefore, RAN2 has agreed to introduce “reactive” and “proactive” approaches for the mentioned two cases.

Consider “proactive” approach (wherein the UE can request capability restrictions which can be independent of current RRC configuration if allowed by the NW) to MUSIM capability restrictions in addition to the reactive approach (which has been agreed previously). Such a mechanism shall still be under NW control, i.e. it is up to network whether to allow such signalling. FFS on the details – should aim for a common framework for the reactive and proactive approach. FFS on UE capabilities

And so far, RAN2 has made the below agreements for the impacted UE capabilities due to dual active MUSIM.

1: For Rel-18 MUSIM dual active operation, the maximum MIMO layer may be changed and the change can be indicated to the NW. FFS if this is only for NW A or also NW B. 3: For Rel-18 MUSIM dual active operation, the measurement gap requirement may be changed and the change can be indicated to the NW. FFS if this is only for NW A or also NW B. 4: For Rel-18 MUSIM dual active operation, the measurement gap requirement change is reported for each serving cells, and for target bands or all supported NR bands depending on whether target bands are configured by the NW. FFS on whether the reporting can reuse the current needForGapInfoNR in RRC reconfiguration complete or extend the similar function in UAI. FFS if this is only for NW A or also NW B. 8: The maximum UL power may be changed due to Rel-18 MUSIM dual active operation, but there is no need to introduce any new UE behavior for reporting this change. 6: UE can explicitly request specific serving cells or serving cell group to be released for Rel-18 MUSIM purpose. FFS how/whether this works for the proactive case. 9: RAN2 should avoid duplicating all the capabilities that UE reports via the UECapabilityInformation in the UAI for R18 MUSIM purpose. RAN2 can discuss P2, P5 and P7 from R2-2304397 during RAN2#123. 9: RAN2 assumes no RAN4 impact is expected on maximum UL power change due to R18 MUSIM. Can re-discuss if critical issues are found in RAN2. The UE can indicate that some frequencies (e.g. frequency ranges, bands or BCs) are impacted by NW B so that they are: - 1) forbidden because of collision - 2) having restricted (lower) capabilities (e.g. with lower MIMO layer). 4: The restrictions can apply to CA, DC and/or single CC. 5: The UL/DL MIMO layer and/or the UL/DL supported bandwidth restriction (if supported) shall only work for the restricted frequencies for the proactive case. 1 For dual active MUSIM, the UE can indicate the temporary maximum MIMO layers for specific serving cells for both UL and DL. FFS whether there is a use case for the UE to indicate the temporary supported channel bandwidth for specific serving cells. 3 Maximum MIMO layers/bandwidth restriction is reported per CC ((FFS how we signal this). FFS whether we support indicating temporary capability restrictions on SRS Tx switching capability. FFS whether this could be already indicated by the MIMO layer restrictions.

This paper will continue discussing the remaining open issues, including which UE capabilities can be impacted by temporary UE capability restrictions and in which granularity and the detailed signalling.

# Discussion

Measurement gap requirement change due to MUSIM:

In RAN2#121bis meeting, RAN2 has agreed the below for measurement gap requirement change due to dual active MUSIM.

3: For Rel-18 MUSIM dual active operation, the measurement gap requirement may be changed and the change can be indicated to the NW. FFS if this is only for NW A or also NW B. 4: For Rel-18 MUSIM dual active operation, the measurement gap requirement change is reported for each serving cells, and for target bands or all supported NR bands depending on whether target bands are configured by the NW. FFS on whether the reporting can reuse the current needForGapInfoNR in RRC reconfiguration complete or extend the similar function in UAI. FFS if this is only for NW A or also NW B.

UE’s measurement gap requirement is determined according to UE’s current configuration and UE capability. In current specification which was designed for single USIM case, when receiving RRC reconfiguration, the UE can include measurement gap requirement change in RRC reconfiguration complete, if network requests. Now, the UE may change measurement gap requirement due to capability switching in Rel-18 MUSIM case. From our understanding, only reactive approach is applied to measurement gap requirement change.

Observation 1: Only reactive approach can be applied to measurement gap requirement change for dual active MUSIM operation.

If reusing the current needForGapsInfoNR in RRC reconfiguration complete message for MUSIM purpose, the network needs to send a RRC reconfiguration message to trigger the measurement gap requirement update reporting even if there is no parameter need to be (re)configured. Also, the latency for waiting the RRC reconfiguration message may cause data interruption on some active serving cells in network A, if UE’s measurement gap requirement on these serving cells has changed from ‘no-gap’ to ‘gap’. Thus, it is preferred to extend the similar function in UAI.  If network allows, the UE can directly send UAI to update the measurement gap requirement for each serving frequency, and for each target band included in the requestedTargetBandFilterNR-r16 (RAN2 can discuss whether to introduce a new filter list in OtherConfig) or each supported NR band. Thus, we have the following proposal:

Observation 2: If reusing the current needForGapsInfoNR in RRC reconfiguration complete for MUSIM purpose, the network needs to send RRC reconfiguration to trigger the measurement gap requirement update reporting even if there is no parameter need to be (re)configured.

- For Rel-18 MUSIM dual active operation, extend the similar measurement gap requirement change reporting in RRC reconfiguration complete to UAI.

Impacted band information change due to MUSIM:

In RAN2#123 meeting, the below agreement was made:

The UE can indicate that some frequencies (e.g. frequency ranges, bands or BCs) are impacted by NW B so that they are: - 1) forbidden because of collision - 2) having restricted (lower) capabilities (e.g. with lower MIMO layer).

For the above impacted frequencies, RAN2 has further discussed the details on how the UE reports the frequency information to the network in the RAN2#123 post email discussion. The majority view is the UE can report the forbidden/impacted bands in a BC, and this has been implemented in the corresponding TP. However, from our understanding, for the supported BC = {B1, B2} in network A, both B1 and B2 can be impacted due to MUSIM band conflict in case of dual active MUSIM operation. And it is more efficient for the UE to directly indicate the index of forbidden BC(s) to the network, instead of indicating all the bands in a BC are forbidden. Thus, we have the following proposal:

- For proactive approach, the UE can indicate the forbidden BCs to the network by using BC index which is the corresponding position in the supportedBandCombinationList.

Moreover, in the RAN2#123 post email discussion, most of the companies agree to introduce a network configured band filtering, therefore, if the UE needs to the indicate the forbidden BCs, the UE is only allowed to indicate the forbidden BC containing a band entry which is included in the network configured band filtering. Thus, we have the following proposal:

- For proactive approach, the UE is only allowed to report the forbidden BCs containing a band entry included in the configured band filtering, or for each supported BC if the band filtering is not configured.

Maximum MIMO layer change due to MUSIM:

In RAN2#123 meeting, RAN2 has agreed the below for MIMO layer capability update for serving cell, i.e., reactive approach.

1 For dual active MUSIM, the UE can indicate the temporary maximum MIMO layers for specific serving cells for both UL and DL.

3 Maximum MIMO layers/bandwidth restriction is reported per CC ((FFS how we signal this).

For proactive approach, the same granularity is per FSPC which can provide full flexibility to the UE implementation for dual active MUSIM operation, but has more specification change. Since the network has full UE capabilities, if the UE wants to change MIMO layer for specific BC (can be called impacted BC), the UE can indicate each impacted BC by using BC index concept, which is defined as the position of corresponding BC entry in supportedBandCombinationList.

Figure 1.

For each impacted BC, there is a feature set combination which includes multiple feature set entries for this BC.  And each feature set entry for this BC can be referred by feature set entry index, which is defined as the position in the FeatureSetCombination, which corresponds to one FeatureSetUplink/Downlink for each band entry in the associated BC. For each impacted feature set entry of this BC, the UE can report the maximum MIMO layer per CC per band. This is illustrated by the above Figure 1. We also provide an example below to show how to capture option 2 in specification.

MUSIM-BandCombList-r18 ::= SEQUENCE (SIZE (1..maxBandComb)) OF MUSIM-PerBandComb-r18

MUSIM-PerBandComb-r18 ::= SEQUENCE{

MUSIM-bandCombinationIndex       BandCombinationIndex,

MUSIM-FeatureSetsList  SEQUENCE (SIZE (1..maxFeatureSetsPerBand)) OF FeatureSetEntryIndex,

MUSIM-MIMO-LayerPerFeatureSetEntryList   SEQUENCE (SIZE (1..maxFeatureSetsPerBand)) OF MUSIM-MIMO-LayerPerFeatureSetEntry

}

BandCombinationIndex ::= INTEGER (1.. maxBandComb)

FeatureSetEntryIndex ::= INTEGER (1.. maxFeatureSetsPerBand)

MUSIM-MIMO-LayerPerFeatureSetEntry  SEQUENCE (SIZE (1.. maxSimultaneousBands)) OF MUSIM-MIMO-LayerPerBandPerFeatureSetEntry

MUSIM-MIMO-LayerPerBandPerFeatureSetEntry SEQUENCE{

bandIndex               INTEGER(1..maxSimultaneousBands),

MUSIM-MIMO-LayerPerCC-List  SEQUENCE (SIZE (1.. maxNrofServingCells)) OF MUSIM-MIMO-LayerPerCC

}

MUSIM-MIMO-LayerPerCC ::=     SEQUENCE{

maxNumberMIMO-LayersPDSCH         MIMO-LayersDL            OPTIONAL,

maxNumberMIMO-LayersCB-PUSCH      MIMO-LayersUL            OPTIONAL,

maxNumberMIMO-LayersNonCB-PUSCH   MIMO-LayersUL            OPTIONAL

}

- For proactive approach, the UE can indicate the temporary maximum DL/UL MIMO layer per FSPC.

Bandwidth reduction due to MUSIM:

In RAN2#123 meeting, RAN2 has made the following agreement:

FFS whether there is a use case for the UE to indicate the temporary supported channel bandwidth for specific serving cells.

In the IDC scheme, the network can configure a frequency range list to the UE, and the UE can indicate one affected frequency range overlapping with one frequency range included in the configured frequency range list, where each affected frequency range consist of a center frequency and an affected bandwidth.

1> if transmission of the UEAssistanceInformation message is initiated to provide IDC assistance information according to 5.7.4.2 or 5.3.5.3: <Omit...> 2> if there is at least one affected frequency range overlapping with one frequency range included in candidateServingFreqRangeListNR, and the center frequency of the affected frequency range is within the frequency range included in candidateServingFreqRangeListNR, the UE is experiencing IDC problems that it cannot solve by itself: 3> include the field affectedCarrierFreqRangeList with an entry for each affected frequency range; 3> for each affected frequency range included in the field affectedCarrierFreqRangeList, include centerFreq and affectedBandwidth; 3> for each affected frequency range included in the field affectedCarrierFreqRangeList, include interferenceDirection and optionally victimSystemType, and set it accordingly; 2> if there is at least one supported UL CA or MR-DC combinations comprising of frequency ranges included in candidateServingFreqRangeListNR, and each affected frequency range in the UL CA or MR-DC combination overlapping with one frequency range included in candidateServingFreqRangeListNR, and the center frequency of the affected frequency range is within the frequency range included in candidateServingFreqRangeListNR, the UE is experiencing IDC problems that it cannot solve by itself: 3> include the field affectedCarrierFreqRangeCombList with an entry for each supported UL CA or MR-DC combination comprising of frequency ranges that is affected by IDC problems; 3> for each affected frequency range included in the field affectedCarrierFreqRangeCombList, include centerFreq and affectedBandwidth; 3> for each UL CA or MR-DC combination included in the field affectedCarrierFreqRangeCombList, include interferenceDirection and optionally victimSystemType, and set it accordingly; <Omit...>

Observation 3: In IDC, the UE can indicate one affected frequency range overlapping with one frequency range included in the configured frequency range list, where each affected frequency range consist of a center frequency and an affected bandwidth.

To solve the MUSIM band conflict issue, we think the same granularity can be applied, at least for serving cells.  Currently, the UE will report its supported channel bandwidth on one carrier of a band of a band combination in SupportedBandwidth capability.

SupportedBandwidth ::=      CHOICE {

fr1                         ENUMERATED {mhz5, mhz10, mhz15, mhz20, mhz25, mhz30, mhz40, mhz50, mhz60, mhz80, mhz100},

fr2                         ENUMERATED {mhz50, mhz100, mhz200, mhz400}

}

SupportedBandwidth-v1700 ::= CHOICE {

fr1-r17    ENUMERATED {mhz5, mhz10, mhz15, mhz20, mhz25, mhz30, mhz35, mhz40, mhz45, mhz50, mhz60, mhz70, mhz80, mhz90, mhz100},

fr2-r17    ENUMERATED {mhz50, mhz100, mhz200, mhz400, mhz800, mhz1600, mhz2000}

}

And the UE will be configured the carrier bandwidth of serving cell in the carrierBandwidth per SCS in SCS-SpecificCarrier of each serving cell.

SCS-SpecificCarrier ::=             SEQUENCE {

offsetToCarrier                     INTEGER (0..2199),

subcarrierSpacing                   SubcarrierSpacing,

carrierBandwidth                    INTEGER (1..maxNrofPhysicalResourceBlocks),

...,

[[

txDirectCurrentLocation         INTEGER (0..4095)                                       OPTIONAL            -- Need S

]]

}

Therefore, for each serving cell, when the UE has interference problem on bandwidth level, the UE can indicate its current maximum channel bandwidth on this serving cell to the network.

- To solve MUSIM band conflict issue, the UE can indicate the temporary supported channel bandwidth for each serving cell.

And for non-serving frequency, we can introduce IDC like solution. But considering we have already agreed to introduce the reporting of forbidden bands information, may be this is sufficient to solve MUSIM band conflict issue. Thus, we have the following proposal:

- To solve MUSIM band conflict on non-serving frequency, RAN2 to discuss:

- Option 1: Confirm the reporting of forbidden bands/BCs is sufficient.

- Option 2: Introduce IDC like solution, in which a frequency range consisted of a center frequency with a value ARFCN and a bandwidth is used.

# Conclusion

In this paper, the following observations are given:

Observation 1: Only reactive approach can be applied to measurement gap requirement change for dual active MUSIM operation.

Observation 2: If reusing the current needForGapsInfoNR in RRC reconfiguration complete for MUSIM purpose, the network needs to send RRC reconfiguration to trigger the measurement gap requirement update reporting even if there is no parameter need to be (re)configured.

Observation 3: In IDC, the UE can indicate one affected frequency range overlapping with one frequency range included in the configured frequency range list, where each affected frequency range consist of a center frequency and an affected bandwidth.

And the following proposals are given:

- For Rel-18 MUSIM dual active operation, extend the similar measurement gap requirement change reporting in RRC reconfiguration complete to UAI.

- For proactive approach, the UE can indicate the forbidden BCs to the network by using BC index which is the corresponding position in the supportedBandCombinationList.

- For proactive approach, the UE is only allowed to report the forbidden BCs containing a band entry included in the configured band filtering, or for each supported BC if the band filtering is not configured.

- For proactive approach, the UE can indicate the temporary maximum DL/UL MIMO layer per FSPC.

- To solve MUSIM band conflict issue, the UE can indicate the temporary supported channel bandwidth for each serving cell.

- To solve MUSIM band conflict on non-serving frequency, RAN2 to discuss:

- Option 1: Confirm the reporting of forbidden bands/BCs is sufficient.

- Option 2: Introduce IDC like solution, in which a frequency range consisted of a center frequency with a value ARFCN and a bandwidth is used.

# Reference

[1] RP-223492, Revised WID: Dual Transmission/Reception (Tx/Rx) Multi-SIM for NR, vivo.

# Appendix

## RAN2#123 Agreements

- Use Msg5 for early indication of MUSIM capability restriction for UEs in IDLE. - Using LCIDs would avoid any problems for RRC resume procedure. However, there are not many LCIDs left for UL and some other Rel-18 WIs also intend to use them. - FFS whether there is a need to use the LCIDs or whether we can reuse the legacy LCIDs. - Whether we can use the LCIDs (given that multiple WIs may be trying to use them) will be discussed in the main session. How to proceed LCID usage for MUSIM can be discussed in the next meeting based on the main session decision. - 3: UE sends early indication of MUSIM temporary capability restriction only if the network indicates that it is allowed in SIB1. - Continue discussion in Thursday session with proactive approach on whether UE can indicating frequency that it would prefer to use. - Discussion was not continued due to lack of time. Post-meeting email discussion (long, vivo) on this topic. - If a timer is introduced, RAN2 needs to define UE behaviour when timer expires and network response is not received. RAN2 also needs to define what “network response” means, i.e. is it a RRCReconfiguration message or a particular field or something else? - FFS whether a timer is needed (e.g. to avoid UE from doing something while the network response has not yet arrived) - Companies should provide Stage-3 details for the next meeting on UE behaviour when network does or does not respond to the UE request to restrict the capabilities due to MUSIM. - 1: The UE can indicate that some frequencies (e.g. frequency ranges, bands or BCs) are impacted by NW B so that they are: - 1) forbidden because of collision - 2) having restricted (lower) capabilities (e.g. with lower MIMO layer). - 4: The restrictions can apply to CA, DC and/or single CC. - 5: The UL/DL MIMO layer and/or the UL/DL supported bandwidth restriction (if supported) shall only work for the restricted frequencies for the proactive case. - 1 For dual active MUSIM, the UE can indicate the temporary maximum MIMO layers for specific serving cells for both UL and DL. - FFS whether there is a use case for the UE to indicate the temporary supported channel bandwidth for specific serving cells. - 3 Maximum MIMO layers/bandwidth restriction is reported per CC ((FFS how we signal this). - FFS whether we support indicating temporary capability restrictions on SRS Tx switching capability. FFS whether this could be already indicated by the MIMO layer restrictions. - 1. When requesting periodic MUSIM gap(s), UE indicates priority values (using R17 IE definition) for all or a subset periodic MUSIM gaps. - 2. When receiving priorities for periodic MUSIM gap(s), the UE may receive changed priority values. If network doesn’t retain the relative priorities among MUSIM gaps, UE behaviour is not specified. - Send LS to RAN4 informing them of this agreement. Offline 203 (LGE).

## RAN2#121bis Agreements

Consider “proactive” approach (wherein the UE can request capability restrictions which can be independent of current RRC configuration if allowed by the NW) to MUSIM capability restrictions in addition to the reactive approach (which has been agreed previously). Such a mechanism shall still be under NW control, i.e. it is up to network whether to allow such signalling. FFS on the details – should aim for a common framework for the reactive and proactive approach. FFS on UE capabilities Support “early indication” from UE to network during RRC connection setup/resume procedure. FFS how to indicate this and in which message. The indication will tell network that UE capabilities are temporarily restricted. FFS on details (i.e. when UE can indicate this, what does it indicate, how does it relate to UAI, etc.) No consensus to support UE-initiated SCell deactivation for MUSIM in Rel-18. 1: For Rel-18 MUSIM dual active operation, the maximum MIMO layer may be changed and the change can be indicated to the NW. FFS if this is only for NW A or also NW B. 3: For Rel-18 MUSIM dual active operation, the measurement gap requirement may be changed and the change can be indicated to the NW. FFS if this is only for NW A or also NW B. 4: For Rel-18 MUSIM dual active operation, the measurement gap requirement change is reported for each serving cells, and for target bands or all supported NR bands depending on whether target bands are configured by the NW. FFS on whether the reporting can reuse the current needForGapInfoNR in RRC reconfiguration complete or extend the similar function in UAI. FFS if this is only for NW A or also NW B. 8: The maximum UL power may be changed due to Rel-18 MUSIM dual active operation, but there is no need to introduce any new UE behavior for reporting this change. 6: UE can explicitly request specific serving cells or serving cell group to be released for Rel-18 MUSIM purpose. FFS how/whether this works for the proactive case. 9: RAN2 should avoid duplicating all the capabilities that UE reports via the UECapabilityInformation in the UAI for R18 MUSIM purpose. RAN2 can discuss P2, P5 and P7 from R2-2304397 during RAN2#123. 9: RAN2 assumes no RAN4 impact is expected on maximum UL power change due to R18 MUSIM. Can re-discuss if critical issues are found in RAN2.

## RAN2#121 Agreements

- A2a: When the UE is in Connected mode in two NR networks, it is up to the UE implementation to select which NW to perform signaling for temporary UE capability restrictions.

- A2b: When the UE is in Connected mode in NR NW A and moving from Idle/Inactive to connected mode in NR NW B, the signaling for temporary UE capability restrictions can happen on NW A. FFS how to handle if UE is moving from IDLE/INACTIVE in NW A and is in CONNECTED with NW B.

- A2c: When the UE is in Connected mode in both networks and one is E-UTRAN, the signaling for temporary UE capability restrictions happens on the NR network.

- A3: The UE will request a temporary capability restrictions (e.g. via UAI) only after the NW signals via RRC that this is allowed. FFS whether the UE can indicate if it is already connecting with reduced capabilities during connection set-up/resume.

- A4: RAN2 to discuss whether prohibit timer is needed for the signaling of temporary UE capability restrictions This can wait until after progress is made on the signaling framework.

- A1: UAI can be used for the signaling of temporary UE capability changes for dual-active MUSIM. FFS if we have additional signalling (depends on e.g. SCell/SCG deactivation usability for MUSIM)

- A8: For dual-active MUSIM, at least the following type of UE capabilities can be expected to be impacted:

- •	Transmission and reception capabilities (e.g. MIMO layers)

- •	Measurement capabilities (e.g. gaps)

- •	Supported bandwidth

- •	Supported band-combinations (FFS whether this is CA or DC or both)

- FFS what is the granularity of reported temporary UE capability restrictions (also pending the band conflict discussion).

- FFS whether UE reports some or all of the above or whether we can do something simpler

- A6: For dual-active MUSIM, UE signaling will support the request for release (and reversal) of SCells and SCG. The signaling details (e.g. implicit or explicit request of each SCell or SCG) is FFS. FFS if we support deactivation (based on discussion in which case it can be used). It is up to network how to react to UE request.

- RAN2 does not intend to create new procedures for e.g. SCell/SCG deactivation for MUSIM purposes in Rel-18. Existing procedures can be used based on NW choice.

- B4: RAN2 considers the only RAN3 impact may be to support the UE request of SCG/SCell release via SRB3 (if supported) for MUSIM purpose (e.g. cause value).  If this can be done via inter-node messages, RAN2 expects no RAN3 impacts.

- 1: The UE is only allowed to provide MUSIM assistance information for Rel-17 MUSIM gap preference to NR MN and NR MN configures the UE with Re-17 MUSIM gap(s). This requires no specification impacts.

- Use inter-node messages to convey Rel-17 MUSIM gap configuration from MN to SN in NW A when UE is in NR-DC.

- RAN2 confirms that the band conflict scenarios will be covered by the temporary UE capability restrictions. FFS on signalling details.

## 5.4 RAN2#119bis Agreements

The R18 MUSIM solution should work in DC/CA and RAN sharing scenarios (but need not be optimized for RAN sharing). RAN2 aims to address at least the Scenario 1: the UE in network A in RRC_CONNECTED indicates (i.e. adds/removes) its preference on temporary UE capability due start/stop connection in NW B. This can be e.g. CA/DC capability restriction. 2 The following is assumed when defining the solution: - The two networks are independent (i.e. no inter-network communication); - The Core Network is not aware of the temporary restrictions of the UE capability; 1: RAN2 can discuss NW A MN-SN coordination of Rel-18 MUSIM temporary capability restrictions due to UE being configured with NR-DC in NW A. RAN2 thinks MN-SN coordination for Rel-17 MUSIM gaps requires WI clarification in RAN RAN2 needs to discuss which UE capabilities can be impacted by sharing of resources between the MUSIM links. RAN2 aims to prioritize only few solutions and avoid multiple solutions for the same problem (FFS pending on solution details). A7: The UE can initiate signalling for UE capability restrictions on NW A if NW A allows it. The specification will not capture NW B events which can cause such need. A4: RAN2 to discuss whether the following UE capabilities (not a complete list) are impacted for dual-active MUSIM: MIMO layers, BC capabilities, Measurement capabilities, Bandwidth, srs-TxSwitch, UL tx power, Power Class. For proposals A1-A2, the solution details need more discussion. Other solutions are not precluded (requires company input with details). Will discuss further over email on the solutions (after this meeting) and which capabilities can be affected. For B1-B3, B5, the solution details need more discussion. May prioritize B1, B2 and B5. FFS on signalling details. Other solutions are not precluded (requires company input with details) and none of B1-B5 are agreed as solutions for this WI. Do not consider solution B4 in Rel-18 (since it may have CN impacts which are precluded in this WI) - B1: For UAI based solution, the following steps can be used as a baseline: - The UE is in Connected Mode on NW A. - The UE is configured for UE capability update via UAI. - The UE intends to start or stop connection with NW B or is already in Connected mode in NW B. - The UE requests a change (restriction or removal of restriction) of the UE capabilities at NW A via UAI. - NW A reconfigures the UE, if needed, according to its new capabilities (FFS if NW response is mandatory) - The UE operates in NW A with the updated configuration. - B2: For delta-signaling of UE capability, the following steps can be used as a baseline: - The UE is in Connected Mode in NW A. - The UE is configured for UE capability update. - The UE starts or stops connection with NW B or is already in Connected mode in NW B. - The UE signals the changed UE capabilities to NW A via delta-signaling. - NW A reconfigures, if needed, the UE according to its new capabilities (FFS if NW response is mandatory). - The UE operates in NW A with the updated configuration. - B3: The solution for the repetition of UE capability enquiry, the following steps can be used as a baseline: - The UE is in Connected Mode in NW A. - The UE is configured for UE capability update. - The UE starts or stops connection with NW B or is already in Connected mode in NW B. - The UE requests a UE capabilty update request. - NW A sends UECapabilityEnquiry to the UE - UE sends UECapabilityInformation to the NW A gNB. - NW A reconfigures, if needed, the UE according to its new capabilities (FFS if NW response is mandatory. - The UE operates in NW A with the updated configuration. - B4: The solution based on using UE-profiles for capability restriction, the following steps can be used as a baseline: - The UE signals different temporary UE capability sets during registration (FFS if these profiles can be updated later) - The UE is in Connected Mode in NW A . - The UE starts or stops connection with NW B or is already in Connected mode in NW B. - The UE requests to switch to a different UE capability profile, e.g. by signaling an index of the profile. - NW A reconfigures the UE according to its new capabilities. - The UE operates in NW A with the updated configuration. - B5 (11/15): A baseline procedure for MAC-CE based SCell (de)-activation can be considered as follows: - The UE is in Connected Mode in NW A . - The UE is configured for MAC-CE based SCell (de)-activation operation. - The UE starts or stops connection with NW B or is already in Connected mode in NW B. - The UE sends a request to deactivate SCells via MAC-CE. - NW A deactivates, if needed, the requested SCells (FFS if NW response is mandatory). - The UE operates in NW A with the updated configuration. CX: RAN2 to continue evaluation of any Xn-AP, F1-AP or RAN4 impact due to dual-active MUSIM operation. 1: RAN2 can consider such Band conflict scenarios for MUSIM in CONNECTED to arrive at a graceful specification-based solution intended to mitigate such conflicts. Wait for RAN4 feedback on MUSIM gap priority.