3GPP RAN WG2 Meeting #123-bis	R2-2309849

Xiamen, China, October 09st – 13th, 2023

Agenda item:	7.20.3

Source:	Samsung

Title:	Remaining Issues on Single-DCI based unified TCI extension to multi-TRP operation

Document for:	Discussion & Decision

Introduction

In RAN2#122 and RAN2#123 meetings, following agreements on enhancements of unified TCI extension to mTRP operation were made [1][2].

- RAN2#122 agreements For sDCI based mTRP operation using unified TCI state framework, introduce the new MAC CE, with the following high level design principles: - If the signaling type of the unified TCI state configuration is configured by RRC (i.e. either joint DL/UL TCI state or separate DL/UL TCI state), it applies to both TRP (i.e., as configured by RRC for both TRPs). - The following information can be indicated by the MAC CE (for joint DL/UL TCI mode): - if the unified TCI state is for one of the TRPs (i.e., 1st or 2nd) or for both TRPs, - if the indicated TCI codepoint consists of one TCI state, whether the indicated TCI state(s) is for the first or second TRP(s) FFS for the separate DL/UL TCI mode. - RAN2#123 agreements The following information can be indicated by the MAC CE (for separate DL/UL TCI mode): - if the unified TCI state is for one of the TRPs (i.e., 1st or 2nd) or for both TRPs, - if the indicated TCI codepoint consists of one TCI state, whether the indicated TCI state(s) is for the first or second TRP(s) - if the unified TCI codepoint is for all, or sub-set of {first DL TCI state, first UL TCI state, second DL TCI state, second UL TCI state} - ??Working assumption: separate MAC CEs are introduced for joint TCI State and separate DL/UL TCI States respectively.

From our understanding, above agreements are enough to design MAC CE(s) and corresponding UE operations on 2	multi-TRP operation using unified TCI framework, especially for single DCI based operation.

Multi-TRP operation using unified TCI framework

In Rel-17, unified TCI framework is introduced and is supported for single TRP operation. Given a list of joint/DL TCI states and a list of UL TCI states configured by RRC, unified TCI state activation/deactivation MAC CE is used to active a set of TCI states and DCI indicates one of the activated TCI state to be applied for PDCCH/PDSCH/AP CSI-RS/PUCCH/PUSCH/SRS.

In Rel-18, unified TCI framework is extended to support multi-TRP operation. For multi-DCI multi-TRP, the activated TCI states for each TRP is indicated by a unified TCI state activation/deactivation MAC CE including a CORESET pool index corresponding to the TRP. For single-DCI multi-TRP, a new unified TCI state activation/deactivation MAC CE is introduced to indicate the mapping between the activated TCI states of two TRPs and DCI codepoints. Different from single-TRP operation where only one joint TCI state or one pair of DL/UL TCI states is mapped to a codepoint in DCI, two joint TCI states (each for a TRP) or two pairs of DL/UL TCI states (each pair for a TRP) can be mapped to a codepoint in DCI. Therefore, how to indicate one (either the 1st or the 2nd) or both of the two TCI states to be applied for PDCCH/PDSCH/AP CSI-RS/PUCCH/PUSCH/SRS is an issue. Based on RAN1 agreements (see Appendix), we summarize the solutions of unified TCI state indication for different channel and RS and the relevant RRC parameters, as illustrated in the table below.

- Multi-DCI based multi-TRP operation

Figure 1. TCI selection scheme for mDCI mTRP operation

- Single-DCI based multi-TRP operation

Figure 2. TCI selection scheme for sDCI mTRP operation

Single-DCI based multi-TRP operation

According to the RAN1 agreements in Appendix, the expected sDCI based multi-TRP operation using unified TCI state framework would be depicted as below.

Based on unified TCI framework, scheduling DCI cannot indicate scheduled PDSCH’s TCI state. In Rel-15/16 TCI framework, dynamic switching between single-/multi-TRP based PDSCH scheduling is based on the indicated TCI states. However, TCI states for all channels are simultaneously updated by unified TCI state framework either by MAC CE or DCI after BAT (Beam Application Time). To enable dynamic switching in unified TCI framework, TCI selection field is introduced in Rel-18. Except for the PDSCH reception, other channels use the semi-static TCI selection by RRC configuration (see Figure 2).

One important rule for unified TCI state and considering two TCI states indication using MAC CE (or DCI) was determined by RAN1.

Agreement in RAN1#112b-e (subset/fullset update) On unified TCI framework extension for S-DCI based MTRP operation, support the followings: - For a serving cell configured with joint DL/UL TCI mode, a full-set or any sub-set of {first joint TCI state, second joint TCI state} can be mapped to a TCI codepoint of the existing TCI field in a DCI format 1_1/1_2 by TCI state activation command (MAC-CE) - For a serving cell configured with separate DL/UL TCI mode, a full-set or any sub-set of {first DL TCI state, first UL TCI state, second DL TCI state, second UL TCI state} can be mapped to a TCI codepoint of the existing TCI field in a DCI format 1_1/1_2 by TCI state activation command (MAC-CE) - TCI state activation command (MAC-CE) should indicate that each joint/DL/UL TCI state mapped to a TCI codepoint is the first or second joint/DL/UL TCI state (detail on how to indicate above is up to RAN2 design) - The first/second indicated joint/DL/UL TCI state(s) is updated according to the corresponding first/second joint/DL/UL TCI state(s) mapped to the TCI codepoint received by the UE - If the UE receives a TCI codepoint mapped with a sub-set of {first joint TCI state, second joint TCI state} or {first DL TCI state, first UL TCI state, second DL TCI state, second UL TCI state}, the UE shall update the first/second indicated joint/DL/UL TCI state(s) according to the first/second joint/DL/UL TCI state(s) in the subset and keep other indicated first/second joint/DL/UL TCI state(s) that is not updated by the received TCI codepoint

According to the above RAN1 agreements, at least TCI state activation command (MAC-CE) should indicate whether each TCI state mapped to a TCI codepoint is the first or second. In addition, if the UE receives a TCI codepoint mapped with a sub-set of all possible TCI states, the UE update it as indicated by MAC CE and keep other indicated TCI states as the previous TCI state. We provide the figure based on our understanding.

Figure 3. Rule for unified TCI state and two TCI state indication

For example in Figure 3, at the initial state, UE activates {TCI#1, TCI#2} for each TRP1 and TRP2.

If {TCI#3, none} is indicated by MAC CE, UE applies it after BAT but in this case, 2nd TCI state for TRP2 should be kept (i.e. maintain TCI#2). Later, UE receives the MAC CE with a TCI codepoint {none, TCI#4}, then UE applies it after BAT but in this case, 1st TCI state for TRP1 should be kept (i.e. maintain TCI#3).

Remaining issues

As we mentioned in the introduction, functional feature to design MAC CE(s) for the single-DCI based multi-TRP operation has already made, and there are only limited issues remaining.

For unified TCI state framework, there are two modes i.e. joint (UL/DL) TCI state mode and separate TCI state mode, and the MAC CE design to indicate the activated TCI states for each mode could be different because each mode has different number of components consisting the TCI codepoint. It means that the joint mode includes atmost two TCI states (joint TCI state for TRP1, joint TCI state for TRP2) per TCI codepoint, and the separate mode includes at most four TCI states (DL TCI state for TRP1, UL TCI state for TRP1, DL TCI state for TRP2, UL TCI state for TRP2) per TCI codepoint.

The MAC running CR for Rel-18 MIMO evolution [3] draft these MAC CEs using separate eLCID and each MAC CE format has designed by optimized manner for each mode. We think this approach seems better to combine those two modes in the same section using one eLCID in terms of readability even though this approach uses two eLCID for same functionality.

Proposal 1: RAN2 confirm that separate MAC CEs on the enhanced unified TCI state for Single-DCI based multi-TRP operation are introduced for joint TCI State and separate DL/UL TCI States, respectively.

Rel-15/16 legacy TCI state framework and Rel-17 unified TCI state framework both support the simultaneous update of serving cell set(s) (i.e. MAC CE applies to all the Serving Cells in the RRC configured set) if it is configured by RRC.

- simultaneousTCI-UpdateList1-r16/ simultaneousTCI-UpdateList2-r16

- simultaneousSpatial-UpdatedList1-r16/ simultaneousSpatial-UpdatedList2-r16

- simultaneousU-TCI-UpdateList1-r17/ simultaneousU-TCI-UpdateList2-r17/ simultaneousU-TCI-UpdateList3-r17/ simultaneousU-TCI-UpdateList4-r17

simultaneousSpatial-UpdatedList1, simultaneousSpatial-UpdatedList2 List of serving cells which can be updated simultaneously for spatial relation with a MAC CE. The simultaneousSpatial-UpdatedList1 and simultaneousSpatial-UpdatedList2 shall not contain same serving cells. Network should not configure serving cells that are configured with a BWP with two different values for the coresetPoolIndex in these lists.

simultaneousTCI-UpdateList1, simultaneousTCI-UpdateList2 List of serving cells which can be updated simultaneously for TCI relation with a MAC CE. The simultaneousTCI-UpdateList1 and simultaneousTCI-UpdateList2 shall not contain same serving cells. Network should not configure serving cells that are configured with a BWP with two different values for the coresetPoolIndex in these lists.

simultaneousU-TCI-UpdateList1, simultaneousU-TCI-UpdateList2, simultaneousU-TCI-UpdateList3, simultaneousU-TCI-UpdateList4 List of serving cells for which the Unified TCI States Activation/Deactivation MAC CE applies simultaneously, as specified in TS 38.321 [3] clause 6.1.3.47. The different lists shall not contain same serving cells. Network only configures in these lists serving cells that are configured with unifiedTCI-StateType.

The question would be how to handle the Rel-18 Enhanced Unified TCI States Activation/Deactivation MAC CEs for mTRP operations (i.e. both mDCI and sDCI based schemes).

Based on RAN1 agreements below, it seems both mDCI and sDCI based unfied TCI state enhancements support the simultaneous update of serving cell sets.

Agreement RAN1#112bis

On unified TCI framework extension, support the following cases for CA operation:

- A set of CCs configured for common TCI state ID activation/update can include CC(s) operating in S-DCI based MTRP

- A set of CCs configured for common TCI state ID activation/update can include CC(s) operating in M-DCI based MTRP

We think that the serving cell sets for Rel-17 unified TCI framework could be applied in this case without introducing the new RRC field because RAN1 didn’t provide the new list on this feature. If we support this, the RRC and MAC running CR could be updated if needed.

Proposal 2: The simultaneous TCI state update for the RRC configured serving cell sets are supported in Rel-18 enhanced unified TCI state activation/deactivation MAC CE(s) for multi-TRP operations.

Conclusion

We propose following proposals:

Proposal 1: RAN2 confirm that separate MAC CEs on the enhanced unified TCI state for Single-DCI based multi-TRP operation are introduced for joint TCI State and separate DL/UL TCI States, respectively.

Proposal 2: The simultaneous TCI state update for the RRC configured serving cell sets are supported in Rel-18 enhanced unified TCI state activation/deactivation MAC CE(s) for multi-TRP operations.

References

R2-2306552	Report from NR MIMO evolution session	Session Chair (CATT).

R2-2308972	Report from NR MIMO evolution session	Session Chair (CATT).

R2-2309850	Running CR for TS 38.321 for MIMO Evolution		Samsung.