3GPP TSG RAN2 Meeting #123bis                                                                    R2-2309853

# Xiamen, China, October 9-13, 2023

Agenda item:	7.25.1

Source:	Samsung

Title:	Discussions on Signaling Support for Intra-band Non-Collocated NR-CA, EN-DC

Document for:	Discussion & Decision

Introduction

According to RAN LS (RP-232692: LS on signaling support for intra-band non-collocated NR-CA, EN-DC), the new UE capability to support non-collocated deployment for TDD-TDD intra-band NR-CA and the network signalling to indicate whether Type 1 or Type 2 capability requirements will be applied by a UE.

In this contribution, we consider how to support this feature in Rel-18.

Discussion

In RAN LS (RP-232692) [1], the request of signalling support on UE capability and the network RRC signalling on intra-band non-collocated NR-CA, EN-DC is quite clear. However, we think there are some points to discuss e.g. backward compatible issue, handling of legacy and new parameters, etc.

UE capability on TDD-TDD intra-band NR CA

As mentioned in LS [1], the UE capability “interBandMRDC-WithOverlapDL-Bands-r16” was introduced since Rel-16 for UE to indicate the support of non-collocated deployment for FDD-FDD or TDD-TDD inter-band EN-DC operation with overlapping or partially overlapping bands, while the UE capability to support non-collocated deployment for TDD-TDD intra-band NR-CA is being discussed in Rel-18.

| interBandMRDC-WithOverlapDL-Bands-r16 Indicates the UE supports FDD-FDD or TDD-TDD inter-band (NG)EN-DC/NE-DC operation with overlapping or partially overlapping DL bands with an (NG)EN-DC/NE-DC MRTD according to clause 7.6.2/7.6.5 in 38.133 [5] and inter-band RF requirements (i.e Type 2 UE). If the capability is not reported, the UE supports FDD-FDD or TDD-TDD inter-band operation with overlapping or partially DL bands with (NG)EN-DC/NE-DC MRTD<3us according to clause 7.6.3 in 38.133 [5] and intra-band RF requirements (i.e. Type 1 UE). | BC | No | N/A | FR1 only |
| --- | --- | --- | --- | --- |

In short, UE supporting interBandMRDC-WithOverlapDL-Bands-r16 applies Type 2 capability requirement (i.e. requirement on MRTD/RF), while the UE not supporting interBandMRDC-WithOverlapDL-Bands-r16 applies Type 1 requirement.

In addition, at least one UE capability to indicate supporting TDD-TDD intra-band Non-Collocated NR-CA is required according to the RAN LS [1] and this UE capability description could be similar with interBandMRDC-WithOverlapDL-Bands-r16. In short, UE supporting the new UE capability (e.g. intraBandNR-CA-non-collocated-r18) applies Type 2 capability requirement, while the UE not supporting the new UE capability applies Type 1 requirement. In addition, RAN2/RAN4 is working on changing the field description interBandMRDC-WithOverlapDL-Bands-r16 i.e. the terminologies “Type 1/2 capability requirements” are not used in the specifications (i.e. describe the feature referring the RAN4 specification).

For TDD-TDD intra-band NR-CA During RAN4#107 meeting, RAN4 concluded a separate new capability signalling is needed, which is to indicate the UE supports TDD-TDD intra-band non-collocated NR-CA operation with MTTD/MRTD requirements according to Table 7.5.4.1/Table 7.6.4-2 in 38.133 and UE RF requirements for intra-band non-collocated NR-CA including 7.10A in 38.101-1 (Type 2 capability requirement). If the new UE capability is absent, the UE supports TDD-TDD intra-band NR-CA operation with MRTD according to Table 7.6.4-1 in 38.133 and UE RF requirements for intra-band NR-CA except for 7.10A in 38.101-1 (Type 1 capability requirement). RAN4 also concluded the UE indicating the “New UE capability” can also support Type 1 capability requirement. This new UE optional capability is expected to be per BC, and applicable to only band n77 and n78. RAN would like to request RAN2 to specify “New UE capability” based on the RAN4 agreements.

Proposal 1: The new Per BC UE capability to indicate the supporting TDD-TDD intra-band Non-Collocated NR-CA is introduced.

- UE supporting the new UE capability supports both Type 1 and Type 2 capability requirements, while the UE not supporting the new UE capability supports Type 1 capability requirement only.

- The terminologies, “Type 1/2 capability” requirements are not used in the specifications (i.e. describe the feature referring the RAN4 specification).

New RRC signalling

According to LS [1], RAN agreed to introduce new RRC signaling to indicate whether Type 1 or Type 2 capability requirements will be applied by a UE configured with the value of IE maxMIMO-Layers no more than 2. See the below description on this RRC signaling.

This new BS signalling is applied to the below cases. For TDD-TDD inter-band EN-DC with overlapping or partially overlapping bands This new BS signalling is expected to be applicable for TDD-TDD inter-band EN-DC with overlapping or partially overlapping bands provided the UE has indicated “interBandMRDC-WithOverlapDL-Bands-r16”. For TDD-TDD intra-band NR-CA This new BS signalling is expected to be applicable for TDD-TDD intra-band NR-CA provided the UE has indicated “New UE capability”.

It seems the new RRC signaling could be applied both Rel-16/Rel-18 features, below is our understanding on the new signaling for TDD-TDD inter-band EN-DC with overlapping or partially overlapping bands and capabilities.

-          Rel-16: No new RRC signaling

  - UE supporting interBandMRDC-WithOverlapDL-Bands-r16 applies Type 2 capability requirement

  - UE not supporting interBandMRDC-WithOverlapDL-Bands-r16 applies Type 1 requirement

- Rel-18: With the new RRC signaling (requirement-TDD-TDD-non-collocated-r18)

  - If intraBandNR-CA-non-collocated-r18 (new UE capability) is present,

    - The UE supporting interBandMRDC-WithOverlapDL-Bands-r16 applies Type 2 capability requirement.

  - If intraBandNR-CA-non-collocated-r18 (new UE capability) is not present,

    - UE supporting interBandMRDC-WithOverlapDL-Bands-r16 applies type 1 capability requirement.

The same UE (supporting interBandMRDC-WithOverlapDL-Bands-r16) will have different behavior in Rel-16 and Rel-18.

The gNB cannot distinguish between the following two cases.

- Rel-16 UE supporting interBandMRDC-WithOverlapDL-Bands-r16.

- Rel-18 UE supporting interBandMRDC-WithOverlapDL-Bands-r16.

We think it is good to have a new Rel-18 UE capability to support the new RRC signaling to indicate whether Type 1 or Type 2 capability requirements are applied when UE is configured with the value of IE maxMIMO-Layers no more than 2.

Proposal 2: Introduce the new RRC signaling and a new Rel-18 UE capability to indicate whether Type 1 or Type 2 capability requirements will be applied by a UE configured with the value of IE maxMIMO-Layers no more than 2.

Based on LS [1] and proposals in this contribution, we provide the draft CRs on 38.331 and 38.306.

Proposal 3: Approve the draft CRs for TS 38.331 and 38.306 on the signaling support for Intra-band non-collocated NR-CA, EN-DC. See [2] and [3].

Conclusion

Based on the above, RAN2 is requested to discuss and agree on the following proposals:

Proposal 1: The new Per BC UE capability to indicate the supporting TDD-TDD intra-band Non-Collocated NR-CA is introduced.

- UE supporting the new UE capability supports both Type 1 and Type 2 capability requirements, while the UE not supporting the new UE capability supports Type 1 capability requirement only.

- The terminologies, “Type 1/2 capability” requirements are not used in the specifications (i.e. describe the feature referring the RAN4 specification).

Proposal 2: Introduce the new RRC signaling and a new Rel-18 UE capability to indicate whether Type 1 or Type 2 capability requirements will be applied by a UE configured with the value of IE maxMIMO-Layers no more than 2.

Proposal 3: Approve the draft CRs for TS 38.331 and 38.306 on the signaling support for Intra-band non-collocated NR-CA, EN-DC. See [2] and [3].

References

RP‑232692	LS on signaling support for intra-band non-collocated NR-CA, EN-DC	RAN.

R2-2309854	Signaling support for intra-band non-collocated NR-CA, EN-DC	CR0955	38.306		Samsung.

R2-2309855	Signaling support for intra-band non-collocated NR-CA, EN-DC		CR4315	38.331		Samsung.