3GPP TSG-RAN WG2 Meeting #123bis	R2-2309882

Xiamen, China, October 9th – 13th, 2023

Agenda Item:		7.6.4

Source:				ASUSTeK

Title:		Discussion on TN coverage for discontinuous coverage

Document for: 	Discussion and Decision

- Introduction

Currently a UE could disable its idle mode tasks related to NTN if the UE detects out of coverage based on satellite information in e.g. SIB32. In this contribution, we discuss whether the UE should also consider TN coverage and disable idle mode tasks related to TN.

- Discussion

According to the current TS 36.304 [1], the UE could determine the discontinuous coverage based on available satellite assistance information in SIB32. When the UE determines it is out of coverage of NTN, the UE could disable its idle mode tasks related to NTN for power saving. The related texts are quoted below:

“If SystemInformationBlockType32 has been received and if the UE has determined that it is out of coverage using available satellite assistance information (e.g. ephemeris parameters and coverage parameters in current or previously received SystemInformationBlockType32, SystemInformationBlockType31, t-Service in SystemInformationBlockType3 or other parameters), the AS configuration (e.g. priorities provided by dedicated signalling and logged measurements) is kept, but the UE need not perform any idle mode tasks related to NTN. It is up to UE implementation to handle running timers. The detection of out of coverage using satellite assistance information is up to UE implementation and once in NTN coverage the UE shall perform all idle mode tasks related to NTN.”

Currently, the idle mode tasks related to TN is not impacted by the discontinuous coverage. For example, the UE would keep performing measurement on TN frequency regardless of whether the UE is out of coverage of TN. However, it will be unnecessary and a waste of power consumption to perform idle mode tasks related to TN if there is no TN coverage for the UE. For example, when the UE is in an area where the network service depends on intermittent satellite access only (e.g. on the mountain), it is beneficial for the UE to be able to control whether to disable idle mode tasks related to TN based on availability of TN coverage in addition to disabling idle mode tasks related to NTN based on NTN discontinuous coverage.

On the other hand, in NR NTN, it has been agreed that a UE is not required to perform neighbour cell measurement for TN neighbour cells in area where there is no TN coverage. And TN coverage information for both NR and EUTRA would be provided to the UE by broadcast signalling. The TN coverage information is associated to frequency information, and is provided via a list of areas where each area is defined using center location coordinates + radius. It could be considered to reuse the TN coverage information agreed in NR NTN for IoT NTN. If the UE could detect whether it is out of TN coverage based on the TN coverage information, the UE could disable idle mode tasks related to TN when it determines that the UE is out of TN coverage.

Proposal 1: idle mode tasks related to TN could be disabled if the UE determines that it is out of TN coverage based on TN coverage information provided by network.

- Conclusion

In this contribution, we discuss disabling idle mode tasks related to TN for discontinuous coverage and have the following proposal:

Proposal 1: idle mode tasks related to TN could be disabled if the UE determines that it is out of TN coverage based on TN coverage information provided by network.

- Reference

- TS 36.304 V17.4.0, “UE procedures in idle mode (Release 17).”