3GPP TSG-RAN WG2 #123-bis	                         R2-2309903

Xiamen, P.R. China, October 9th – 13th 2023

Agenda item:	7.16.2.2

Source:	Fujitsu

Title:	Discussions on AIML data collection

Document for:	Discussion/Decision

1. Introduction

In this contribution, we give our views on both UE-side model and NW-side model data collection on the basis of the previous RAN1 and RAN2 discussions [1][2][3] related to AI/ML data collection.

2. Discussion

2.1 Network-side model data collection

Currently, the following sub use cases may involve network-side models:

- CSI Feedback: Two-sided models with decoder part in gNB.

- Beam Management: Spatial or Temporal prediction in gNB.

- Positioning Accuracy Enhancement: LMF-side model (case 2b/3b) and gNB-side model (case 3a)

2.1.1 Model Training

For NW-side model training, according to the discussion in [2], there are sub use cases for both gNB-side and LMF-side models. From the agreed entity mapping in [2], both gNB-centric and OAM-centric data collection can be utilized for gNB-side model of all three use cases. For gNB-centric data collection, the gNB configures UE to complete the data collection and act as the termination point of the collected data, while for OAM-centric data collection, the OAM configures the gNB and subsequently UE to complete the data collection and act as the termination point of the collected data. Combined with the reply given from RAN1 in [1], we have the following table 1 as the potential data c1ollection framework of gNB-side model training.

Table 1 Potential data collection framework for gNB-side model single data sample

| Sub use case | Data collection framework |
| --- | --- |
| CSI Feedback | L1/L3-signaling (UCI/RRC) and immediate MDT |
| Beam Management | L1/L3-signaling (UCI/RRC) and immediate MDT |
| Positioning 3a | L3-signaling (RRC), immediate MDT and logged MDT |

Proposal 1 Use table 1 as the selected data collection frameworks for gNB-side model training.

For CSI and BM, L1-signaling such as UCI can be used for at least the data collection for single data sample since the data size does not exceed the maximum and the latency requirements are relaxed for offline model training. However, due to the scarcity of the L1 uplink resource, it is preferred to enhance L3 signaling to be used as the container to carry collected data for model training.

Proposal 2 Layer 3 signaling is used as the container to carry collected data for gNB-side model training.

As for LMF-side model, the LMF configures the gNB and UE to complete the data collection and the termination point of the collected data is LMF. Combined with the reply given from RAN1 in [1], we have the following proposal for LMF-side model training data collection.

Proposal 3 Use LPP and NRPPa as the selected data collection frameworks for LMF-side model training.

Furthermore, although gNB or OAM is supposed to initiate the configuration of data collection procedure, other entities may need to be involved to trigger/request/stop the data collection: one example is that gNB may play the role of requesting OAM to start or stop the data collection procedure if gNB believes it is necessary for its local model; another example is that the LMF may trigger/stop the data collection for assisted positioning based on gNB-side model, it can send its request along with existing signaling such as UL-SRS configurations since the positioning control function is located at LMF.

Proposal 4 If the data collection procedure is triggered/requested/stopped by entities other than the training entity, signaling among these entities needs to be studied.

2.1.2 Model Monitoring

For NW-side model monitoring, , the network is in charge of the monitoring for its local model so gNB-centric data collection can be used for CSI/BM/Pos 3a and LMF-centric data collection can be used for Pos 2b/3b. Due to the near-real-time latency requirement and relatively smaller size requirements given by RAN1 in [1], L1 signaling is preferred as the baseline for the CSI/BM NW-side model monitoring data collection, the necessity to apply L3 signaling should be studied further. Therefore, we have the following proposal:

Proposal 5 For NW-side CSI and BM, L1 signaling (e.g., UCI) is preferred as the data collection framework for model monitoring. Whether L3 signaling is necessary needs further study.

The reply in [1] does not include the details for positioning model monitoring due to that RAN1 has not concluded yet. From current observations, L3 signaling and LPP are potential data collection frameworks for gNB and LMF-side model respectively.

Proposal 6 L3 signaling and LPP are potential data collection frameworks for gNB and LMF-side positioning model respectively. The details should be studied once RAN1 has solid conclusions.

For both NW-side model training and model monitoring, the collected data may be filtered according to certain rules to save uplink resources, the network can set the rules by multiple factors such as UE capability, model properties, channel conditions and so on, we have the following proposal for filtering of data collection.

Proposal 7 To save uplink resources, the data may need to be filtered before collection by some rules. The details on how the NW set the rules and related signaling to UE should be studied further.

2.1.3 Model Inference

From agreed table in [2], it is observed that the inference entities of almost all use cases except two-sided CSI compression locate in the same entity with model deployment. Therefore, no specific data collection framework needs to be designed for model inference. As RAN1 has confirmed RAN2 assumption for the latency requirement of model inference [1], for each use case, the data can be collected via legacy framework, e.g., L1 report via UCI for CSI/BM and LPP for Positioning.

Observation 1 The main requirement for model inference data collection is latency, which has been confirmed by RAN1 LS reply.

Observation 2 Legacy data collection framework can be used at least as a starting point of model inference, e.g., UCI for CSI/BM and LPP for positioning.

2.2 UE-side model data collection

Currently, the following sub use cases may involve UE-side models:

- CSI Feedback: Two-sided models with encoder part in UE.

- Beam Management: Spatial or Temporal prediction in UE.

- Positioning Accuracy Enhancement: UE-side model (case 1/2a).

For UE-side models, the model training/inference/monitoring may locate at UE or OTT-server, once the data contents are confirmed by RAN1, UE can decide which data it needs to perform specific LCM function per use case. However, the network side may still be in charge of data collection control (e.g., triggering/stopping) if it does not authorize UE to perform autonomous control.

Observation 3 For UE-side models, UE or UE-side OTT-server can decide the data collection details by itself once the data contents have been fixed.

Proposal 8 For UE-side models, if UE does not have autonomous control for data collection, the signaling from network side may be used for triggering/stopping the data collection procedure.

3. Conclusion

Based on the above discussions, we give the following observations:

Observation 1 The main requirement for model inference data collection is latency, which has been confirmed by RAN1 LS reply.

Observation 2 Legacy data collection framework can be used at least as a starting point of model inference, e.g., UCI for CSI/BM and LPP for positioning.

Observation 3 For UE-side models, UE or UE-side OTT-server can decide the data collection details by itself once the data contents have been fixed.

Based on the above discussions, we give the following proposals:

Proposal 1 Use table 1 as the selected data collection frameworks for gNB-side model training.

Proposal 2 Layer 3 signaling is used as the container to carry collected data for gNB-side model training.

Proposal 3 Use LPP and NRPPa as the selected data collection frameworks for LMF-side model training.

Proposal 4 If the data collection procedure is triggered/requested/stopped by entities other than the training entity, signaling among these entities needs to be studied.

Proposal 5 For NW-side CSI and BM, L1 signaling (e.g., UCI) is preferred as the data collection framework for model monitoring. Whether L3 signaling is necessary needs further study.

Proposal 6 L3 signaling and LPP are potential data collection frameworks for gNB and LMF-side positioning model respectively. The details should be studied once RAN1 has solid conclusions.

Proposal 7 To save uplink resources, the data may need to be filtered before collection by some rules. The details on how the NW set the rules and related signaling to UE should be studied further.

Proposal 8 For UE-side models, if UE does not have autonomous control for data collection, the signaling from network side may be used for triggering/stopping the data collection procedure.

4. Reference

[1] R1-23XXXXX RAN2 LS Reply on Data Collection.

[2] RAN2 #123 Chair_Notes_EOM.

[3] RAN2 #122 Chair_Notes_EOM.