3GPP TSG-RAN WG2 Meeting #123bis                                                                            R2-2309916

Xiamen, CN, Oct 09 –13, 2023

Source:	CATT

Title:	Discussion on L1 related issues for LTM

Agenda Item:	7.4.2.1

Document for:	Discussion and Decision

Introduction

In this contribution, we analysis on the L1 related aspects for LTM including the following aspects:

- On L1 measurement configuration;

- On early RACH configuration;

- On TCI configuration;

Discussion

    - On L1 measurement configuration related issues

On configuration of the L1 LTM-CSI-ResourceConfig

In L1 measurement report for ICBM, within the CSI-ResourceConfig, a combination of the SSB sets is configured. For each SSB set, it includes a list of SSB resources to be measured and each SSB can be associated to different PCIs. Then, by including the CSI-ResourceConfigId in the LTM-CSI-ReportConfig, the corresponding resources to be measured is associated with the report resources. That is, based on current spec, it supports to report L1 measurement result for different PCI within one reportConfigId.

As for R18 LTM L1 measurement, the similar mechanism is applied, i.e., by associating the resource configuration with the report configuration, and within the associated resource configuration, a list of RS of all candidates can be included. Besides, RAN2 also agreed in RAN2#122 that the CSI resource configuration is included in the LTM-Config IE and is a separate configuration, e.g. outside of the LTM candidate configuration.

The RS configuration and/or CSI resource configuration for measuring LTM candidate cells is included in the LTM-Config IE and is a separate configuration, e.g. outside of the LTM candidate configuration.

Proposal 1: The LTM-CSI-ResourceConfig is configured outside per LTM candidate cell configuration.

On configuration of the L1 LTM-csi-SSB-ResourceList

Besides, according to RAN1 parameter list, for each SSB resources set, the included SSB can corresponding to different candidate cells. This is similar mechanism like R18 ICBM. Besides, even some companies may doubt the signaling overhead between NW interface when the NW modify the candidate cell or the SSB resources to be measured, but since anyway each candidate DU and the serving DU need to be informed about the updated L1 CSI resource configuration, so there is not much difference on signaling overhead between the approach to make all RS within one resource set corresponding to one candidate cells and the approach as indicated in the RAN1 parameter list. Besides, L1 measurement is indeed RAN1 dominated, unless critical issues is observed, RAN2 should respect RAN1’s conclusion. So it is proposed to follow the RAN1 agreement to configure the LTM-CSI-ResourceConfig.

Proposal 2: Follow what is indicated by RAN1 in the parameter list, i.e., the SSBs within one LTM-csi-SSB-ResourceList includes the SSBs of different candidate cells.

On options of  LTM CSI report configuration

In the RAN1 RRC parameter list, there are two options on CSI reporting for LTM.

- Option 1: reuse the legacy CSI-ReportConfigToAddModList, and adding some LTM related configurations in the new IE LTM-ReportContent within the CSI-ReportConfig;

- Option 2: introduce LTM specific CSI report configuration IE ltm-CSI-ReportConfigToAddModList in the CSI-MeasConfig, and LTM specific CSI report IE ltm-CSI-ReportConfig.

In the following, I summarize the pros and cons for the two options.

| Option | Pros | Cons |
| --- | --- | --- |
| Option 1 | Reuse current IEs in CSI-measConfig | For all the fields that are mandatory present in the current CSI-Report, need to clarify UE shall ignore when LTM-ReportContent is configured |
| Option 2 | More easier for future extension | Need to define IEs in CSI-measConfig |

In general, we think both options can work, and the spec complexity is similar. But since options 2 is already implemented in the running CR, so it is slightly prefer to follow option 2.

Proposal 3: Option 2 in higher-layers parameter list from RAN1 is used to LTM CSI report configuration, i.e., introducing LTM specific CSI report configurations ltm-CSI-ReportConfig.

On how to enable the reporting of the LTM L1 measurement for current serving SpCell

RAN1 also agreed to optional include the current serving SpCell’s measurement results in the L1 measurement report for LTM. This means, the associated LTM CSI resource configuration within the report configuration should also include the SSBs of the serving cells. But according to the current L1 parameter list, the LTM-CandidateId is used to identify which cell the SSB belongs to. From this perspective, enable the current serving cell’s L1 measurement is included within the L1 measurement is not possible.

Observation 1: Since LTM-CandidateId is used to identify which cell the SSB belongs to and the current serving cell may not be configured as one candidate cell, the current serving SpCell’s L1 measurement for LTM cannot be reported even NW requests.

To solve this issue, the following solutions can be considered:

Option 1: NW should ensure that the current serving SpCell should always be configured as one LTM candidate cell;

Option 2: introduce new index which can identify both the LTM-CandidateId and the serving SpCell to identify the cell the SSB belongs to;

Option 3: introduce another CSI-ResourceConfig which associated with the L1 measurement resource of current serving cell in the LTM reporting configuration.

In general, alloptions can work. But for option 1, whether to configure the current serving SpCell as candidate depend on the NW implementation, such restriction is quite limited for NW. Option 2 follows the similar structures as R17 ICBM, and it requires UE to decide which serving SpCell it is and then associate to the approximate SSB resources. For option 3, it is more flexible, but it is unclear whether it will impacts the RAN1 CSI structure for LTM measurement reporting, which already been defined by RAN1. Besides, for option 2/3, since this is slightly different as what is indicated by RAN1, RAN2 may need to send LS to RAN1 to indicate the agreement if concluded. In general, at least we think the option 1 should be excluded.

Proposal 4: RAN2 to further discuss whether the option 2 or option 3 should be taken based on the TP as provided in the annex 1.

- Option 2: introduce new index which can identify both the LTM-CandidateId and the serving SpCell to identify the cell the SSB belongs to;

- Option 3: introduce another CSI-ResourceConfig which associated with the L1 measurement resource of current serving cell in the LTM reporting configuration.

    - On early RACH configuration related issues

Before discussion the detailed configuration, we want to clarify the configuration of early RACH resources for each candidate cell should not be configured per BWP, only one RACH configuration is needed regardless it is in which BWP of the candidate cell. The reason is that the early sync RACH resource is only used to obtain the TA of the cell, and it doesn’t vary between BWPs. Besides, before UE access the target cell, there is no need to differ different BWPs, only one available resource that the UE could determine the frequency resource for the RACH resource is enough.

Proposal 5: Similar like the RACH resources provided to IDLE UE, only one RACH configuration per candidate cell should be provided to UE, i.e., regardless the RACH resources is in which BWP.

The RACH occasion is configured with respective to the start frequency of the BWP where the RACH resource is configured [1]. However, the BWP configuration is within the candidate configuration which will not be used before it as serving cell. The issue is how to indicate the frequency resource of the RACH occasion.

RACH-ConfigGeneric ::= SEQUENCE { prach-ConfigurationIndex INTEGER (0..255), msg1-FDM ENUMERATED {one, two, four, eight}, msg1-FrequencyStart INTEGER (0..maxNrofPhysicalResourceBlocks-1), ---------------------skip unrelated part------------------ } msg1-FrequencyStart Offset of lowest PRACH transmission occasion in frequency domain with respective to PRB 0. The value is configured so that the corresponding RACH resource is entirely within the bandwidth of the UL BWP. (see TS 38.211 [16], clause 6.3.3.2)..

Observation 2: The frequency resource of the RACH occasion is configured with respective to the start frequency location of the BWP.

Based on above analysis, to configure the RACH resources for early sync, the following solutions can be considered:

- Option 1: change the legacy frequency location configuration method for the RACH occasion, i.e., it is configured relative to the PRB indicated by higher layers parameters offsetToCarrier and subcarrierSpacing;

- Option 2: still reusing the legacy RACH configuration method, but need to provide one piece of any of the BWP generic configuration (where the RACH is configured) to UE.

Since option 1 changes the legacy RACH configuration mechanism, both RAN2 and RAN1 spec will be impacted. So among these options, we prefer option 2.

Proposal 6: For the early RACH configuration, except the RACH generic configuration and the association of the SSB with PRACH occasion, the following information should also be provided:

- The generic parameters of any one of the BWPs of the candidate cell, e.g., the initial BWP of the cell;

- The FrequencyInfoUL of the candidate cell, where the absoluteFrequencyPointA is mandatory present.

    - On TCI configuration related issues

On clarification of the BWP index included within the QCL-Info

According to the 38.331 [2], in case CSI-RS is configured as QCL-RS, the BWP index can be configured to indicate which BWP the RS is located in. And if it is configured, it indicates the DL BWP where the RS is located in. Otherwise, it implies that the RS is located in the DL BWP where the TCI state is applied by UE.

bwp-Id The DL BWP which the RS is located in. If the field is absent, the RS is located in the DL BWP in which the TCI-State is applied by the UE.

However, for R18 LTM, before UE access the target cell, there is not any BWP information of the candidate cell. And also we think the TCI states and also the corresponding TRS included in the TCI states should be independent of the BWP. So the field should never be configured for this case, and also the corresponding field description should be modified to clarify the understanding for the case the bwp-Id is not configured.

Proposal 7: Clarification on the bwp-Id within the QCL-Info should be added, i.e., the field is never used for LTM.

On association of the direct QCL RS with SSB

The following agreement is made by RAN1 in last meeting.

Agreement In R18 LTM, on the QCL source of the TCI state before/during the cell switch command, - SSB or TRS can be configured in a TCI state for the candidate cell(s) before/during cell switch command. - Whether the TRS can be used for the candidate cell(s) before/during cell switch command is up to UE capability.

It can be seen that either SSB or TRS can be configured as the direct QCL source RS for the TCI states for candidate cells used before or during the cell switch command. However, according to the RAN1 agreements and the RAN1 parameter list, only the SSB related configuration for candidate cells shall be provided and used for measurement before LTM cell switch is executed. In case TRS is configured as the direct QCL source RS in the TCI states, since UE and the serving cell has no awareness of the QCL-chain between SSB and the TRS, so UE/NW cannot decide which TCI state to be activated in the TCI activation command or indicated in the LTM command.

Observation 3: If UE and NW have no awareness on the QCL-chain between the measured SSB and the TRS included in the TCI state, NW cannot decide which TCI state to be activated or indicated, and UE cannot decide the activated/indicated beam.

To address this issue, the candidate cell also need to provide the QCL-chain association between the SSB and the TRS indicated in the TCI states to the serving cell, so that the serving cell can provide such information to UE correspondingly. As for details on how to provide such information to UE, the corresponding TP is proposed in the annex 2.

Proposal 8: The association on the QCL-chain between the SSB and the TRS indicated in the TCI state should also be pre-configured to UE. The TP can be seen in the Annex 2.

Conclusion

Based on the previous analysis in section 2, our observations and proposals are summarized as follows:

Observation 1: Since LTM-CandidateId is used to identify which cell the SSB belongs to and the current serving cell may not be configured as one candidate cell, the current serving SpCell’s L1 measurement for LTM cannot be reported even NW requests.

Observation 2: The frequency resource of the RACH occasion is configured with respective to the start frequency location of the BWP.

Observation 3: If UE and NW have no awareness on the QCL-chain between the measured SSB and the TRS included in the TCI state, NW cannot decide which TCI state to be activated or indicated, and UE cannot decide the activated/indicated beam.

On L1 measurement configuration related issues

Proposal 1: The LTM-CSI-ResourceConfig is configured outside per LTM candidate cell configuration.

Proposal 2: Follow what is indicated by RAN1 in the parameter list, i.e., the SSBs within one LTM-csi-SSB-ResourceList includes the SSBs of different candidate cells.

Proposal 3: Option 2 in higher-layers parameter list from RAN1 is used to LTM CSI report configuration, i.e., introducing LTM specific CSI report configurations ltm-CSI-ReportConfig.

Proposal 4: RAN2 to further discuss whether the option 2 or option 3 should be taken based on the TP as provided in the annex 1.

- Option 2: introduce new index which can identify both the LTM-CandidateId and the serving SpCell to identify the cell the SSB belongs to;

- Option 3: introduce another CSI-ResourceConfig which associated with the L1 measurement resource of current serving cell in the LTM reporting configuration.

On early RACH configuration related issues

Proposal 5: Similar like the RACH resources provided to IDLE UE, only one RACH configuration per candidate cell should be provided to UE, i.e., regardless the RACH resources is in which BWP.

Proposal 6: For the early RACH configuration, except the RACH generic configuration and the association of the SSB with PRACH occasion, the following information should also be provided:

- The generic parameters of any one of the BWPs of the candidate cell, e.g., the initial BWP of the cell;

- The FrequencyInfoUL of the candidate cell, where the absoluteFrequencyPointA is mandatory present.

On TCI configuration related issues

Proposal 7: Clarification on the bwp-Id within the QCL-Info should be added, i.e., the field is never used for LTM.

Proposal 8: The association on the QCL-chain between the SSB and the TRS indicated in the TCI state should also be pre-configured to UE. The TP can be seen in the Annex 2.

Reference

[1] 3GPP TS 38.331: "NR; Radio Resource Control (RRC) protocol specification"

[2] [Post123][056][feMob] LTM Running CR RRC (Ericsson)

- Annex 1

## Option 2

#### –	LTM-CSI-ResourceConfig

The IE LTM-CSI-ResourceConfig defines a group of one or more CSI resources for an LTM candidate cell configuration.

LTM-CSI-ResourceConfig information element

-- ASN1START

-- TAG-LTM-CSI-RESOURCECONFIG-START

LTM-CSI-ResourceConfig-r18 ::=      SEQUENCE {

ltm-CSI-ResourceConfigId-r18              LTM-CSI-ResourceConfigId-r18,

ltm-CSI-SSB-ResourseSetToAddModList-r18    SEQUENCE (SIZE (1..maxNrofLtmCSI-SSB-ResourcesPerConfig-r18)) OF LTM-CSI-SSB-ResourseSet-r18

OPTIONAL, -- Need N

ltm-CSI-SSB-ResourseSetToReleaseList-r18   SEQUENCE (SIZE (1..maxNrofLtmCSI-SSB-ResourcesPerConfig-r18)) OF LTM-CSI-SSB-ResourseSetId-r18

OPTIONAL, -- Need N    ...

}

LTM-CSI-SSB-ResourceSet-r18 ::=     SEQUENCE {

ltm-CSI-SSB-ResourceSetId-r18             LTM-CSI-SSB-ResourceSetId-r18,

ltm-ServingCandidateCellInfoList-r18		SEQUENCE (SIZE (1..maxNrofLtmCSI-SSB-ResourcesPerSet-r18)) OF LTM-ServingCandidateCellInfoList-r18

ltm-CSI-SSB-ResourceList-r18              SEQUENCE (SIZE (1..maxNrofLtmCSI-SSB-ResourcesPerSet-r18)) OF SSB-Index,

}

LTM-CSI-SSB-ResourceSetId-r18 ::=           INTEGER (0..maxNrofLtmCSI-SSB-ResourceSets-r18-1)

LTM-ServingCandidateCellInfoList-r18::=     INTEGER (0.. maxNrofCellsLTM-r18)

-- TAG-LTM-CSI-RESOURCECONFIG-STOP

-- ASN1STOP

LTM-CSI-ResourceConfig field descriptions

ltm-CSI-ResourceConfigId This field is used to identify an instance of LTM-CSI-ResourceConfig IE.

ltm-CSI-SSB-ResourceSet This field defines one SS/PBCH block resource set from one or more LTM candidate cells.

LTM-CSI-SSB-ResourceSet field descriptions

ltm-ServingCandidateCellInfoList Indicates the cell information of the SSBs in the ltm-CSI-SSB-ResourceList. The first entry of the list indicates the information of the cell for the first entry of csi-SSB-ResourceList, the second entry of this list indicates the information of the cell for the second entry of csi-SSB-ResourceList, and so on. For each entry, the following applies: - If the value is zero, the cell is the current serving cell of UE; - otherwise, the value is LTM-CandidateId-r18 of an LTM-Candidate-r18 configured using the LTM-CandidateToAddModList.

ltm-CSI-SSB-ResourceList This field is used to indicate on SS/PBCH block resources from one or more LTM candidate cells.

ltm-CSI-SSB-ResourceSetId This field is used to idenfity on SS/PBCH block resource set.

## Option 3

#### –	LTM-CSI-ReportConfig

The IE LTM-CSI-ReportConfig is used to configure report on the cell in which the LTM-CSI-ReportConfig is included.

LTM-CSI-ReportConfig information element

-- ASN1START

-- TAG-LTM-CSI-REPORTCONFIG-START

LTM-CSI-ReportConfig-r18 ::=      SEQUENCE {

ltm-CSI-ReportConfigId-r18                     LTM-CSI-ReportConfigId-r18

ltm-ResourcesForChannelMeasurement-r18         LTM-CSI-ResourceConfigId-r18

ltm-ResourcesForChannelMeasurementSpCell-r18   CSI-ResourceConfigId                     OPTIONAL, -- Need R

reportConfigType-r18                           CHOICE {

periodic-r18                                   SEQUENCE {

reportSlotConfig-r18                           CSI-ReportPeriodicityAndOffset,

pucch-CSI-ResourceList-r18                     SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUCCH-CSI-Resource

},

semiPersistentOnPUCCH-r18                      SEQUENCE {

reportSlotConfig-r18                           CSI-ReportPeriodicityAndOffset,

pucch-CSI-ResourceList-r18                     SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUCCH-CSI-Resource

},

},

ltm-ReportContent-r18                          LTM-ReportContent-r18

...

}

LTM-ReportContent-r18 ::= SEQUENCE {

noOfReportedCells-r18                                ENUMERATED {n1,n2,n3,n4}

noOfReportedRS-PerCell-r18                           ENUMERATED {n1,n2,n3,n4}

spCellInclusion-r18                                  ENUMERATED {true}                                          OPTIONAL, -- Need R

}

-- TAG-LTM-CSI-REPORTCONFIG-STOP

-- ASN1STOP

LTM-CSI-ReportConfig field descriptions

ltm-CSI-ReportConfigId This field is used to identify an LTM-CSI-ReportConfig.

ltm-ReportContent This field defines the content of the LTM L1 measurement report.

ltm-ResourcesForChannelMeasurement This field indicates the resources used for LTM L1 measurements.

pucch-CSI-ResourceList Indicates which PUCCH resource to use for reporting on PUCCH.

reportConfigType This field describes the time domain behaviour of how the L1 measurements are reported.

reportSlotConfig Periodicity and slot offset (see TS 38.214 [19], clause 5.2.1.4).

LTM-ReportContent field descriptions

noOfReportedCells This field defines how many cells are reported within a single L1 measurement report instance.

noOfReportedRS-PerCell This field defines how many RSs per cell are reported within a single L1 measurement report instance.

spCellInclusion This field indicates whether the UE shall include a L1 measurement report associated to the current SpCell.

- Annex 2

#### –	LTM-CandidateToAddModList

The IE LTM-CandidateToAddModList concerns a list of LTM candidate cell configurations to add or modify.

LTM-CandidateToAddModList information element

-- ASN1START

-- TAG-LTM-CANDIDATETOADDMODLIST-START

LTM-CandidateToAddModList-r18 ::= SEQUENCE (SIZE (1..maxNrofCellsLTM-r18)) OF LTM-Candidate-r18

LTM-Candidate-r18 ::=     SEQUENCE {

ltm-CandidateId-r18                            LTM-CandidateId-r18,

ltm-CandidatePCI-r18                           PhysCellId,

ltm-SSB-Config-r18,                            LTM-SSB-Config-r18                                    OPTIONAL,    -- Need M

ltm-CandidateConfig-r18                        OCTET STRING (CONTAINING RRCReconfiguration),         OPTIONAL,    -- Need M

ltm-ConfigComplete-r18                         ENUMERATED {true}                                     OPTIONAL,    -- Need R

ltm-EarlyUL-SyncConfig-r18                     SetupRelease { EarlyUL-SyncConfig-r18 }               OPTIONAL,    -- Need M

ltm-NoResetID-r18                              INTEGER (1..maxNrofCellsLTM-r18-1)                    OPTIONAL,    -- Need M

ltm-dl-OrJointTCI-StateToAddModList-r18        SEQUENCE (SIZE (1..maxNrofLtmTCI-States-r18)) OF CandidateTCI-State-r18

OPTIONAL,    -- Need N

ltm-dl-OrJointTCI-StateToReleaseList-r18       SEQUENCE (SIZE (1..maxNrofLtmTCI-States-r18)) OF CandidateTCI-StateId-r18

OPTIONAL,    -- Need N

ltm-ul-TCI-ToAddModList-r18                    SEQUENCE (SIZE (1..maxUL-LtmTCI-r18)) OF CandidateTCI-UL-State-r18       OPTIONAL,    -- Need N

ltm-ul-TCI-ToReleaseList-r18                   SEQUENCE (SIZE (1..maxUL-LtmTCI-r18)) OF CandidateTCI-UL-StateId-r18     OPTIONAL,    -- Need N   ltm-ReleaseSCG-r18                             ENUMERATED {true}                                     OPTIONAL,    -- Need SCG-Release

ltm-CSI-RS-SSB-QCLRelationList-r18             SEQUENCE (SIZE (1..maxNrofLtmQCLAssociationInfo-r18)) OF LTM-CSI-RS-SSB-QCLRelation-r18

OPTIONAL     -- Need R

...

}

LTM-SSB-Config-r18 ::= SEQUENCE {

ssbFrequency-r18                               ARFCN-ValueNR,

subCarrierSpacing-r18                          SubCarrierSpacing,

ssb-Periodicity-r18                             ENUMERATED {ms5, ms10, ms20, ms40, ms80, ms160, spare2, spare1}      OPTIONAL,   -- Need R

ssb-PositionsInBurst-r18                        CHOICE {

shortBitmap                                    BIT STRING (SIZE (4)),

mediumBitmap                                   BIT STRING (SIZE (8)),

longBitmap                                     BIT STRING (SIZE (64))

}                                                                                                                OPTIONAL,   -- Need R

ss-PBCH-BlockPower-r18                         INTEGER (-60..50)                                                     OPTIONAL,   -- Need R

...

}

-- TAG-LTM-CANDIDATETOADDMODLIST-STOP

-- ASN1STOP

LTM-CandidateToAddModList field descriptions

ltm-CandidateId This field indicates an LTM candidate cell configuration.

ltm-CandidateConfig This field includes an RRCReconfiguration message used to configure an LTM candidate cell.

ltm-CandidatePCI This field indicates the PCI of the LTM candidate cell.

ltm-ConfigComplete This field indicates whether the LTM candidate cell configuration within ltm-CandidateConfig is a complete configuration.

ltm-dl-OrJointTCI-StateToAddModList A list of TCI states to add and/or modify.

ltm-dl-OrJointTCI-StateToReleaseList A list of TCI states to remove.

ltm-EarlyUL-SyncConfig A configuration used to perform the early UL synchronization procedure.

ltm-ul-TCI-ToAddModList A list of uplink TCI states to add and/or modify.

ltm-ul-TCI-ToReleaseList A list of uplink TCI states to remove.

ltm-NoResetID This field indicates whether the UE should perform no L2 reset when an LTM cell switch procedure is executed towards an LTM candidate. If the value of ltm-NoResetID in the LTM candidate cell is the same as the value of ltm-ServingCellNoResetID in the serving cell of a cell group, then the UE shall not perform any L2 reset during an LTM cell switch procedure.

ltm-SSB-Config This field indicates an SSB configuration associated with an LTM candidate cell.

LTM-SSB-Config field descriptions

ssb-PositionsInBurst Indicates the time domain positions of the transmitted SS-blocks in a half frame with SS/PBCH blocks as defined in TS 38.213 . The first/leftmost bit corresponds to SS/PBCH block index 0, the second bit corresponds to SS/PBCH block index 1, and so on. Value 0 in the bitmap indicates that the corresponding SS/PBCH block is not transmitted while value 1 indicates that the corresponding SS/PBCH block is transmitted.

ssbFrequency Indicates the frequency of the SS/PBCH block associated with the LTM candidate cell.

ssb-Periodicity Indicates the periodicity of the SS/PBCH block.

ss-PBCH-BlockPower Indicates the average EPRE of the resources elements that carry secondary synchronization signals in dBm that the network uses for SSB transmission on the LTM candidate cell.

subCarrierSpacing Indicates the subcarrier spacing of the SSB.

| Conditional Presence | Explanation |
| --- | --- |
| SCG-Release | This fiels is mandatory present if the LTM-CandidateConfig is associated with an MCG. Otherwise, it is absent, Need N. |

#### –	LTM-CSI-RS-SSB-QCLRelation

The IE LTM-CSI-RS-SSB-QCLRelation is used to configure the association relationship between TRS and SSB of each LTM candidate cell.

LTM-CSI-RS-SSB-QCLRelation information element

-- ASN1START

-- TAG-LTM-CSI-RS-SSB-QCLRelation-START

LTM-CSI-RS-SSB-QCLRelationList-r18 ::=      SEQUENCE {

ltm-CSI-RS-SSB-QCLRelationId-r18        LTM-CSI-RS-SSB-QCLRelationId-r18,

qcl-InfoAssiciation-r18 				SEQUENCE {

nzp-CSI-RS-ResourceId-r18               NZP-CSI-RS-ResourceId,

associatedqcl-infoRS-r18                SSB-index,

},

...

}

LTM-CSI-RS-SSB-QCLRelationId-r18 ::= INTEGER (0..maxNrofRSassocationsLTM-r18))

-- TAG-LTM-CSI-RS-SSB-QCLRelationList-STOP

-- ASN1STOP