3GPP TSG-WG2 Meeting #123bis	R2-2309920

# Xiamen, China, 9th October - 13th October, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.355 | CR | 0471 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of reference SCS for dl-prs-ResourceSetPeriodicityReq-r17 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh-Core |  |  |  |  |  | Date: |  |  | 2023-09-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The field dl-prs-ResourceSetPeriodicityReq-r17 indicates the requested periodicity of DL-PRS in unit of slots. Thus, the requested periodicity can be interpreted differently depending on the referred μ (SCS value). Regarding the referred μ, the field description of dl-prs-ResourceSetPeriodicityReq-r17 specifies that the μ refers to the target device’s current primary cell (i.e., Pcell) as below. dl-prs-ResourceSetPeriodicityReq This field specifies the requested periodicity of the DL-PRS Resource Set in slots. The periodicity depends on the subcarrier spacing (SCS) and takes values 2μ4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 160, 320, 640, 1280, 2560, 5120, 10240 slots, where μ=0, 1, 2, 3 for SCS of 15, 30, 60 and 120 kHz respectively. μ refers to the target devices current primary cell.. However, there is currently no way for LMF/(non-serving)gNBs/TRPs to know the SCS (μ) of target UE’s Pcell and thus they can not correctly interpret the periodicity requested by UE (i.e., dl-prs-ResourceSetPeriodicityReq-r17). Moreover, there is no reason why the SCS of target UE’s Pcell is used as the reference SCS for the requested periodicity. If there are multiple BWPs using different numerology (SCS) in the target UE’s Pcell, the ambiguity on the reference SCS will still remain. Thus, it seems reasonable to allow the UE to use any reference SCS for the requrested DL PRS periodicity regardless of the numerology of its Pcell. |
| Summary of change: |  | - Introduce a new field (dl-prs-ReferenceSCS-v17xy) to allow the UE to indicate the reference SCS for dl-prs-ResourceSetPeriodicityReq-r17. - Correct the field description of dl-prs-ResourceSetPeriodicityReq-r17 to specify that the μ refers to the new field dl-prs-ReferenceSCS-v17xy. Impact analysis Impacted 5G architecture options: NR SA, (NG)EN-DC, NE-DC,NR-DC Impacted functionality: - On-demand PRS Inter-operability: 1. if the Network supports the change and the UE does not: No interoperability problems are foreseen. 2. if the UE supports the change and the network does not: When the UE sets the newly introduced field, the NW can not understand and ignore it. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The LMF/(non-serving)gNBs/TRPs can not correctly interpret the periodicity requested by UE (i.e., dl-prs-ResourceSetPeriodicityReq-r17). |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start changes

–	NR-On-Demand-DL-PRS-Information

The IE NR-On-Demand-DL-PRS-Information defines the requested on-demand DL-PRS.

-- ASN1START

NR-On-Demand-DL-PRS-Information-r17 ::= SEQUENCE (SIZE (1..nrMaxFreqLayers-r16)) OF

NR-On-Demand-DL-PRS-PerFreqLayer-r17

NR-On-Demand-DL-PRS-PerFreqLayer-r17 ::= SEQUENCE {

dl-prs-FrequencyRangeReq-r17			ENUMERATED { fr1, fr2, ...},

dl-prs-ResourceSetPeriodicityReq-r17	ENUMERATED { p4, p5, p8, p10, p16, p20, p32, p40,

p64, p80, p160, p320, p640, p1280, p2560,

p5120, p10240, p20480, p40960, p81920, ...,

p128-v1760, p256-v1760, p512-v1760}

OPTIONAL,

dl-prs-ResourceBandwidthReq-r17			INTEGER (1..63)							OPTIONAL,

dl-prs-ResourceRepetitionFactorReq-r17	ENUMERATED {n2, n4, n6, n8, n16, n32, ...}

OPTIONAL,

dl-prs-NumSymbolsReq-r17				ENUMERATED {n2, n4, n6, n12, ...}		OPTIONAL,

dl-prs-CombSizeN-Req-r17				ENUMERATED {n2, n4, n6, n12, ...}		OPTIONAL,

dl-prs-QCL-InformationReqTRPlist-r17	DL-PRS-QCL-InformationReqTRPlist-r17	OPTIONAL,

...

[[

dl-prs-ReferenceSCS-v17xy				ENUMERATED {kHz15, kHz30, kHz60, kHz120, ...}	OPTIONAL

]]

}

DL-PRS-QCL-InformationReqTRPlist-r17 ::= SEQUENCE (SIZE (1..nrMaxTRPsPerFreq-r16)) OF

DL-PRS-QCL-InformationReqPerTRP-r17

DL-PRS-QCL-InformationReqPerTRP-r17 ::= SEQUENCE {

dl-PRS-ID-r17						INTEGER (0..255),

nr-PhysCellID-r17					NR-PhysCellID-r16							OPTIONAL,

nr-CellGlobalID-r17					NCGI-r15									OPTIONAL,

nr-ARFCN-r17						ARFCN-ValueNR-r15							OPTIONAL,

dl-prs-QCL-InformationReqSet-r17	SEQUENCE (SIZE (1..nrMaxSetsPerTrpPerFreqLayer-r16)) OF

DL-PRS-QCL-InfoReq-r17,

...

}

DL-PRS-QCL-InfoReq-r17 ::= SEQUENCE {

nr-DL-PRS-ResourceSetID-r17			NR-DL-PRS-ResourceSetID-r16,

dl-prs-QCL-InformationReq-r17		CHOICE {

dl-prs-QCL-InfoRecPerResourceSet-r17	DL-PRS-QCL-Info-r16,

dl-prs-QCL-Info-requested-r17			NULL

},

...,

[[

dl-prs-QCL-InfoRecPerResource-r17	SEQUENCE  (SIZE (1..nrMaxResourcesPerSet-r16)) OF

DL-PRS-QCL-Info-r16			OPTIONAL

]]

}

-- ASN1STOP

NR-On-Demand-DL-PRS-Information field descriptions

dl-prs-FrequencyRangeReq This field specifies the frequency range for which the on-demand DL-PRS is requested.

dl-prs-ResourceSetPeriodicityReq This field specifies the requested periodicity of the DL-PRS Resource Set in slots. The periodicity depends on the subcarrier spacing (SCS) and takes values 2μ4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 160, 320, 640, 1280, 2560, 5120, 10240 slots, where μ=0, 1, 2, 3 for SCS of 15, 30, 60 and 120 kHz respectively. μ refers to dl-prs-ReferenceSCS-v17xy.

dl-prs-ResourceBandwidthReq This field specifies the requested number of PRBs allocated for the DL-PRS Resource (allocated DL-PRS bandwidth) in multiples of 4 PRBs. Integer value 1 corresponds to 24 PRBs, value 2 corresponds to 28 PRBs, value 3 corresponds to 32 PRBs and so on.

dl-prs-ResourceRepetitionFactorReq This field specifies the requested DL-PRS Resource repetition. Enumerated values n2, n4, n6, n8, n16, n32 correspond to 2, 4, 6, 8, 16, 32 resource repetitions, respectively.

dl-prs-NumSymbolsReq This field specifies the requested number of symbols per DL-PRS Resource within a slot.

dl-prs-CombSizeN-Req This field specifies the requested Resource Element spacing in each symbol of the DL-PRS Resource.

dl-prs-QCL-InformationReqTRPlist This field specifies the recommended or requested QCL indication with other DL reference signals. - dl-PRS-ID indicates the DL-PRS ID of the TRP for which the QCL information is recommended. - nr-PhysCellID indicates the physical Cell-ID of the TRP for which the QCL information is recommended, as defined in TS 38.331 [35]. - nr-CellGlobalID indicates the NCGI, the globally unique identity of a cell in NR, of the TRP for which the QCL information is recommended, as defined in TS 38.331 [35]. - nr-ARFCN indicates the NR-ARFCN of the TRP's CD-SSB (as defined in TS 38.300 [47]) corresponding to nr-PhysCellID. - dl-prs-QCL-InformationReqSet indicates the recommended QCL information per DL-PRS Resource Set. - nr-DL-PRS-ResourceSetID indicates the DL-PRS Resource Set ID for which the QCL information is recommended. - dl-prs-QCL-InformationReq - dl-prs-QCL-InfoRecPerResourceSet indicates a single recommended QCL source for the DL-PRS Resource Set. - dl-prs-QCL-Info-requested indicates that the UE requests to provide the QCL information in the assistance data. - dl-prs-QCL-InfoRecPerResource indicates a list of recommended QCL sources for the DL-PRS Resource Set. If this field is present, the dl-prs-QCL-InformationReg shall be ignored by the receiver.

dl-prs-ReferenceSCS This field specifies the reference SCS for dl-prs-ResourceSetPeriodicityReq-r17.

End of change