3GPP TSG-RAN WG2 Meeting #123bis	R2-2309929

Xiamen, China, October 9th – 13th, 2023

Agenda item:		7.9.4

Source:		Lenovo

Title:		Discussion on direct path addition

Document for:		Discussion and Decision

- Introduction

RAN2 achieved some agreement for multi-path in the previous meeting. In this contribution, we further discuss the issues for direct path addition in multi-path [1].

- Discussion

Figure 1: Procedure for direct path addition on top of indirect path

Once the timer for direct path addition e.g T304 expires, that means UE fails to add the direct path. In this case, the UE is not expected to perform re-establishment procedure since the remote UE has still another available indirect path. UE should report the direct path addition failure to the serving gNB via the available indirect path upon expiry of the corresponding timer for the direct path addition.

Proposal 1: Once the timer T304 for the direct path addition expires, the remote UE reports the failure information to the network via the indirect path if available.

When the timer for the direct path addition expires, it is possible that the indirect path could be suspended e.g PC5 RLF or reception of notification/PC5 release. Therefore, once the timer for the direct path addition expires, the remote UE initiates re-establishment procedure if the indirect path is suspended/unavailable.

Proposal 2: Once the timer T304 for the direct path addition expires, the remote UE initiates re-establishment procedure if the indirect path is suspended.

Regarding the case of the direct path addition after UE has the indirect path already, the failure may happen in the PC5 link or Uu interface of the indirect path. Specifically, the remote UE may receive the notification message from relay UE due to e.g Uu RLF in the indirect path when UE is performing the direct path addition procedure. Or the remote UE may detect RLF on PC5 link in the first path when UE is performing the second direct path addition procedure.

Proposal 3: If the remote UE detects PC5 RLF or receives the notification message/PC5 unicast release message from relay UE on the indirect path when UE is performing the direct path addition procedure, UE reports the failure information once the direct path is successfully added.

- Conclusion

In this contribution, we have the following proposals:

Proposal 1: Once the timer T304 for the direct path addition expires, the remote UE reports the failure information to the network via the indirect path if available.

Proposal 2: Once the timer T304 for the direct path addition expires, the remote UE initiates re-establishment procedure if the indirect path is suspended.

Proposal 3: If the remote UE detects PC5 RLF or receives the notification message/PC5 unicast release message from relay UE on the indirect path when UE is performing the direct path addition procedure, UE reports the failure information once the direct path is successfully added.

References

RAN2#123 Chairman nodes.