3GPP TSG-RAN WG2 Meeting #123bis	R2-2309946

Xiamen, China, 9th – 13th October 2023

Agenda item:	 7.11.2.1

Source:	 Lenovo

Title:		Control plane aspects of multicast reception in RRC_INAVTICE

Document for:		Discussion and Decision

- Introduction

This contribution further discusses the remaining CP related issues of multicast reception in RRC_INACTIVE state on RRC resume cause and access category, configured radio threshold and multicast neighbour cell list.

- Discussion

  - RRC resume cause and access category

In RAN2#123, it was agreed that:

Unless issues are identified with using one of existing resume causes, no new resume causes are introduced for UEs receiving MC in INACTIVE when they resume due to bad quality or lack of SIBx/PTM configuration

As specified in the TS 24.501, the mapping for access identities/access categories and RRC establishment cause is given in the Table 4.5.6.1:

| Rule # | Access identities | Access categories | RRC establishment cause is set to |
| --- | --- | --- | --- |
| 1 | 1 | Any category | mps-PriorityAccess |
| 2 | 2 | Any category | mcs-PriorityAccess |
| 3 | 11, 15 | Any category | highPriorityAccess |
| 4 | 12,13,14, | Any category | highPriorityAccess |
| 5 | 0 | 0 (= MT_acc) | mt-Access |
|  |  | 1 (= delay tolerant) | Not applicable (NOTE 1) |
|  |  | 2 (= emergency) | emergency |
|  |  | 3 (= MO_sig) | mo-Signalling |
|  |  | 4 (= MO MMTel voice) | mo-VoiceCall |
|  |  | 5 (= MO MMTel video) | mo-VideoCall |
|  |  | 6 (= MO SMS and SMSoIP) | mo-SMS |
|  |  | 7 (= MO_data) | mo-Data |
|  |  | 9 (= MO IMS registration related signalling) | mo-Data |

As shown in the table, the mapping for access identities/access categories and RRC establishment cause is clearly specified. For example, in the table it is clearly specified that ‘mo-data’ cause is set for access category 7 and 9.

For the Access category setting, it is also clearly defined in the in the Table 4.5.2.2: Mapping table for access categories. Taking mo-data as an example:

| 9 | UE NAS initiated 5GMM connection management procedure or 5GMM NAS transport procedure | Access attempt is for MO data | 7 (= MO_data) |
| --- | --- | --- | --- |
| 10 | An uplink user data packet is to be sent for a PDU session with suspended user-plane resources | No further requirement is to be met | 7 (= MO_data) |

As shown in the table, the events for setting access categories is clearly defined in TS 24.501. 1, e.g., Access category 7 (=MO_Data) includes two cases: UE NAS initiated 5GMM connection management procedure or 5GMM NAS transport procedure and An uplink user data packet is to be sent for a PDU session with suspended user-plane resources.

For the multicast reception in RRC_INACTIVE, since the RRC Resume Request is caused by the the PTM configuration of the multicast session is not available in the cell, the existing RRC resume causes specified in TS 24.501 are not applicable.

Observation 1:  the existing RRC resume causes specified in TS 24.501 are not applicable for RRC Resume Request triggered for multicast reception in RRC_INACTIVE.

In the TS 38.331, there is on RRC resume cause ‘RNA-update’ that is defined by AS layer and the corresponding access category is 8 (for non-emergency case). According to the cause ‘RNA-update’, the network may decide to perform RAN update procedure without entering RRC_CONNECTED state. Then the cause ‘RNA-update’ is also not applicable for RRC Resume Request triggered by multicast reception in RRC_INACTIVE.

Observation 2:  the cause ‘RNA-update’ is also not applicable for RRC Resume Request triggered by multicast reception in RRC_INACTIVE.

Introducing a new RRC Resume cause can also give the NW flexibility to perform access control for different causes.

Define a new RRC Resume cause to support that RRC Resumption for MC reception in RRC_INACTIVE due to bad quality or lack of SIBx/PTM configuration.

- In RAN2#121bis, it was agreed that:

When a UE reselects to a cell for which PTM configuration is not available in multicast MCCH, the UE initiates RRC resumption procedure for an active multicast session it is interested to receive or continue receiving.

- When UE moves to a cell of which the UEs has no PTM configuration of a MRB, the UE should perform RRC Resume procedure to acquiring the PTM configuration. Since we already agreed that the PTM configuration can be provided by RRCRelease message, it is not necessary for the UE entering RRC_CONNECTED state to get the PTM configuration for sake of signalling overhead. For example, the UE indicates PTM configuration request indication to the network in the RRCResumeRequest message. The PTM configuration request indication is used to indicate the RRCResumeRequest is only for acquiring the PTM configuration which implies there is no need to resume the UE into RRC_CONNECTED state. In response to the PTM configuration request indication in the RRCResumeRequest message, the network can directly provide the updated PTM configuration in the RRCRelease message without entering RRC_CONECTED state.

If PTM configuration provision by RRCRelease without entering RRC_CONNECTED state is supported, an additional RRC Resume cause should be introduced.

- Currently, the access category 2 or 8 are used for RNA update case. If an emergency service is on-going, the UE shall select ‘2’ as the Access Category, otherwise, the UE shall select ‘8’ as the Access Category.

- For RRC resumption due to MC reception in RRC_INACTIVE, we think the same rule with RNA update can be used.

Access category 2 and 8 are used for RRC Resumption for MC reception in RRC_INACTIVE due to bad quality or lack of SIBx/PTM configuration.

  - Configured radio threshold for triggering RRC connection resumption

- In RAN2#123 meeting, it was agreed that:

For a UE receiving multicast in RRC_INACTIVE, the UE resumes the RRC connection when the measured RSRP or RSRQ based on the existing measurement requirements (whichever is configured by the NW) of the serving cell becomes lower than the threshold configured by network. FFS whether/how we need to address ping-pong issue

- As specified in TS 38.304, the Srxlev and Squal is used for cell selection criterion S and for triggering measurements for intra-frequency or inter frequency with equal or lower priority for cell reselection, where the Srxlev and Squal are defined as:

- Srxlev = Qrxlevmeas – (Qrxlevmin + Qrxlevminoffset )– Pcompensation - Qoffsettemp

- Squal = Qqualmeas – (Qqualmin + Qqualminoffset) - Qoffsettemp

- Following the same principle, the Srxlev and Squal should also be used for trigging RRC Resumption.

For cell reselection, Treselection is used to avoid ping-pong:

TreselectionRAT This specifies the cell reselection timer value. For each target NR frequency and for each RAT other than NR, a specific value for the cell reselection timer is defined, which is applicable when evaluating reselection within NR or towards other RAT (i.e. TreselectionRAT for NR is TreselectionNR, for E-UTRAN TreselectionEUTRA).

For RRC_CONNECTED state, a time to trigger is also used for measurement report to avoid unnecessary measurement report.

Similarly, in order to avoid ping-pong, a time value that the threshold condition needs to be fulfilled for UE retuning RRC_CONNECTED for multicast reception should be also provided.

Srxlev and Squal are used for representing the measured and RSRP and RSRQ.

- A configured time value for which the threshold condition needs to be fulfilled is introduced for avoiding ping-pong.

Furthermore, a prohibited time for UE triggering to transit RRC_CONNECTED for multicast reception can also be provides to avoid the UE returning back RRC_CONNECTED unnecessary. For example, the UE Upon receiving the RRCRelease message, the UE enters RRC_INACTIVE state, the UE starts the prohibited timer. When prohibited timer is running, the UE shall not trigger state transition from RRC_INACTIVE to RRC_CONNECTED procedure, i.e., the UE only performs state transition from RRC_INACTIVE to RRC_CONNECTED when the timer expires.

- A prohibited time for UE returning RRC_CONNECTED for multicast reception is configured to UE.

  - MBS Neighbor Cell List

In RAN2#121bis meeting, it was agreed that

The neighbour cell list mechanism for multicast reception in RRC_INACTIVE may be configured e.g. it can be used by UE to resume RRC connection if service is not available in the re-selected cell by NCL, without reading MCCH in the re-selected cell, in some aspects similar to Rel-17 NCL mechanism in MBS broadcast.

In Rel-17, the MBS-NeighbourCellList IE is introduced for indicating a list of neighbour cells where ongoing MBS sessions provided via broadcast MRB in the current cell are also provided. This allows the UE, e.g. to request unicast reception of the service before moving to a cell not providing the MBS broadcast service(s) using PTM transmission. Different with broadcast, the multicast transmission status in a neighbour cell may include:

-	multicast session is not provided by the neighbor cell either by PTM transmission or PTP transmission, e.g., the neighbour cell is out of multicast service area;

-	the multicast session is supported by the neighbour cell but PTM transmission has not been started e.g. it is the first UE moves to the cell for the multicast reception or only PTP transmission is used for a small group of UEs.

For the first case, same with broadcast, the UE may need to request unicast reception of the multicast service before moving to the neighbour cell. For the second case, since the PTM configuration is not provided by MCCH, the UE may need prepare to resume to RRC_CONNECTED state when cell reselection to the neighbor cell.

Neighbour cell list is enhanced to include:

-	Whether the multicast session is supported by the neighbour cell or not e.g., the neighbour cell is out of multicast service area;

-	In case that the multicast session is supported, whether PTM transmission is provided in the neighbour cell.

3	Conclusion

This contribution further discusses the remaining CP related issues of multicast reception in RRC_INACTIVE state on RRC resume cause and access category, configured radio threshold and multicast neighbour cell list. And we propose:

Observation 1:  the existing RRC resume causes specified in TS 24.501 are not applicable for RRC Resume Request triggered for multicast reception in RRC_INACTIVE.

Observation 2:  the cause ‘RNA-update’ is also not applicable for RRC Resume Request triggered by multicast reception in RRC_INACTIVE.

- Define a new RRC Resume cause to support that RRC Resumption for MC reception in RRC_INACTIVE due to bad quality or lack of SIBx/PTM configuration.

- If PTM configuration provision by RRCRelease without entering RRC_CONNECTED state is supported, an additional RRC Resume cause should be introduced.

- Access category 2 and 8 are used for RRC Resumption for MC reception in RRC_INACTIVE due to bad quality or lack of SIBx/PTM configuration.

- Srxlev and Squal are used for representing the measured and RSRP and RSRQ.

- A configured time value for which the threshold condition needs to be fulfilled is introduced for avoiding ping-pong.

- A prohibited time for UE returning RRC_CONNECTED for multicast reception is configured to UE.

- Neighbor cell list is enhanced to include:

-	Whether the multicast session is supported by the neighbor cell or not e.g., the neighbor cell is out of multicast service area;

-	In case that the multicast session is supported, whether PTM transmission is provided in the neighbor cell.

Reference