3GPP TSG-RAN WG2 Meeting #123bis	R2-2309958

Xiamen, China, October 9th – 13th, 2023

Agenda item:		7.6.3.1

Source:		Lenovo

Title:		Views on some remaining issues for mobility in IoT NTN

Document for:		Discussion

- Introduction

The latest RAN2#123 meeting made the following agreements regarding mobility in IoT NTN:

Agreements: 1. For eMTC, the new SIB (SIBxx) is not an essential SIB. UE does not need to consider the cell barred if it is unable to acquire the SIB when scheduled. 2. RAN2 will not consider to include cell stop time of neighbor cell in the new SIB (SIBxx) in this release. 3. for NB-IoT NTN, location-based measurement initiation can also be optionally used in RRC_IDLE for cell re-selection purposes (like in NR-NTN), with the assumption that it is up to the UE to update GNSS location. 4. validity duration is optional, and if this field is absent, the UE uses validity duration from the serving cell 5. For re-acquisition of SIBXX the UE may rely on T317/T318 in connected mode 6. For CHO in NTN (both NR NTN and eMTC NTN, time and location-based trigger conditions may be configured independently (i.e., without a jointly configured measurement condition). We add a description/note saying in which scenarios this is reasonable, e.g. at least hard-switch case where gap is assumed to be zero/negligible

In this contribution we further discuss some remaining issues in mobility based on previous discussions and agreements.

- Discussion

## Start serving time of neighbour cell

During the discussion on the t-ServiceStartNeigh in SIBxx, an issue was raised that whether it shall be per cell or per frequency/satellite. The option of per cell is straight-forward and has been supported by majority while other prefers the option of per frequency/satellite due to that the time is used to indicate UE to start measure on specific frequencies. In our understanding both options are workable and thus a down-selection can be made. Meanwhile for each option, the definition of t-ServiceStartNeigh could be different and shall be specified.

Proposal 1a: t-ServiceStartNeigh in SIBxx is per neighbour cell, and it is defined as the starting time of the corresponding neighbour cell.

Proposal 1b: t-ServiceStartNeigh in SIBxx is per frequency/satellite, and it can be defined as the earliest starting time among all neighbor cells associated to the corresponding frequency/satellite.

## Reference location of the serving earth-moving cell

For location/distance-based neighbor cell measurement triggering, it has been agreed to use a reference location as well for the earth-moving cells and the remaining detail is whether to use an IE different from the referenceLocation introduced in Rel-17 for quasi-earth-fixed cells. In our understanding although using the same IE is straight-forward, a potential issue is that how UE can differentiate between a quasi-earth-fixed cell and an earth-moving cell (in Rel-17 referenceLocation is configured only for quasi-earth-fixed cells), and as a result a cell type indication is needed causing more specification impact. On the contrary, introducing a different IE can achieve both functionalities of indicating cell type and the moving reference location.

Proposal 2: A separate reference location IE is used for moving cells and no cell type indication is needed.

## Stop serving time of the serving earth-moving cell

Similar issue was also discussed for time-based neighbor cell measurement triggering, but the situation is a bit different. The reference location is the same concept for both quasi-earth-fixed cell and earth-moving cells, only that the reference location for earth-moving cells is mobile, and UE needs to derive that actual reference location based on ephemeris. The t-Service refers to the cell-specific stop serving time due to service link switch or feeder link switch in quasi-earth-fixed cells and the cell-specific stop serving time due to feeder link switch in earth-moving cells, respectively, and the UE behaviors in both cases are the same (i.e., trigger neighbour cell measurement before t-Service). Therefore, there is no need to introduce a separate cell stop serving time IE for earth-moving cells apart from the t-Service.

Proposal 3: No need to introduce a separate cell stop serving time IE apart from the t-Service for earth-moving cells.

## Handling when specific fields are absent

For the new SIBxx introduced for neighbour cells, some further details about UE handling also remain to be specified, e.g., when some specific fields including the satellite ID, the list of frequencies or cells are absent. Considering the definition and functionality of the new SIBxx, it is reasonable to let UE reuse the same fields in SIB31 (of the serving cell) by default if absent in the new SIB, i.e., assuming that the corresponding fields are absent in case they have the same value for the serving and neighbour cells.

Proposal 4: When satellite ID is absent, UE considers a neighbour cell as belonging to the same satellite as the serving cell (i.e., intra-satellite mobility).

Proposal 5: When the list of frequencies in new SIB is absent, UE considers a neighbour cell as using the same frequency as the serving cell (i.e., intra-frequency mobility).

Proposal 6: When the list of cells in new SIB is absent, UE considers a neighbour cell as using the same PCI as the serving cell (i.e., PCI unchanged case).

- Conclusion

In this contribution we discuss some remaining issues in mobility based on previous discussions and agreements. It is proposed that:

Proposal 1a: t-ServiceStartNeigh in SIBxx is per neighbour cell, and it is defined as the starting time of the corresponding neighbour cell.

Proposal 1b: t-ServiceStartNeigh in SIBxx is per frequency/satellite, and it can be defined as the earliest starting time among all neighbor cells associated to the corresponding frequency/satellite.

Proposal 2: A separate reference location IE is used for moving cells and no cell type indication is needed.

Proposal 3: No need to introduce a separate cell stop serving time IE apart from the t-Service for earth-moving cells.

Proposal 4: When satellite ID is absent, UE considers a neighbour cell as belonging to the same satellite as the serving cell (i.e., intra-satellite mobility).

Proposal 5: When the list of frequencies in new SIB is absent, UE considers a neighbour cell as using the same frequency as the serving cell (i.e., intra-frequency mobility).

Proposal 6: When the list of cells in new SIB is absent, UE considers a neighbour cell as using the same PCI as the serving cell (i.e., PCI unchanged case).