3GPP TSG-RAN WG2 Meeting #123bis	R2-2309988

Xiamen, China, October 9-14, 2023

Agenda item:	6.1.3.3

Source:	Qualcomm Incorporated

Title:	Discussion on autonomous BWP switch for non-RedCap

WID/SID:	NR_redcap-Core

Document for:	Discussion and Decision

# Introduction

In RAN2#123 meeting, the following agreement was made for RedCap on UE autonomous switching to BWP outside the configured UE channel bandwidth.

If a RedCap UE needs to autonomously switch to its initial BWP to perform RACH (as in Iegacy) but its current UE channel BW does not cover the initial BWP, the UE autonomously changes its UE channel BW to cover the initial BWP.

It is FFS if the same behaviour applies to non-RedCap [1]. In this paper, we discuss the applicable non-RedCap UE scenario that RAN2 discussed previously and try to close the open issue.

# Discussion

In RAN2#115-e, the following agreement was made for non-RedCap.

R2-2107980 Allowed bandwidth in BWP configuration Ericsson discussion [016] noted [016] R2 Confirms the following understanding: - When configuring a UE with a dedicated BWP that is not within the channel bandwidth that the UE applied when acquiring SIB1, the network configures the downlinkChannelBW-PerSCS-List and/or uplinkChannelBW-PerSCS-List and firstActiveBWPID so that the channel bandwidth covers at least the active BWP. UE behaviour is not specified when channel bandwidth doesn't contain active BWP size. - The network avoids DCI- and timer-based BWP switching to BWPs that are not within current channel bandwidth.

The corresponding scenario is captured in the contribution above and reproduced below. In this example, the network channel bandwidth is 30MHz. After UE access using the BWP0, the network configures the UE with the UE specific channel bandwidth of 20MHz to cover BWP1, which becomes the active BWP.

Figure 1 – BWP0 outside UE specific channel bandwidth

It is our understanding that the intention of the RAN2 agreement above was that it is network’s responsibility to avoid any possible cases where the UE needs to switch to BWP0 that is outside the UE specific channel bandwidth, even though RAN2 unfortunately failed to discuss and capture the random access scenario explicitly.

Proposal:	For non-RedCap, RAN2 confirm that RACH resources are always configured in an active BWP when the UE specific channel bandwidth does not cover UE’s initial BWP.

# Conclusion

Based on the above discussion, we’d recommend RAN2 to discuss and adopt the following proposal:

Proposal:	For non-RedCap, RAN2 confirm that RACH resources are always configured in an active BWP when the UE specific channel bandwidth does not cover UE’s initial BWP.

# Reference

[1]	R2-2309215	Summary of Offline #020 Autonomous BWP switching beyond UE’s configured channel bandwidth	Qualcomm Incorporated.