3GPP TSG-RAN WG2 Meeting #123bis	R2-2309989

Xiamen, China, October 9th – 13th, 2023

Agenda item:		7.7.3

Source:		Lenovo

Title:		Views on some remaining issues for network verified UE location

Document for:		Discussion

- Introduction

The latest RAN2#123 meeting made the following agreements regarding network verified UE location:

Agreements: 1. A Rel-18 UE capability is needed for indicating whether UE supports the feature of network verified UE location in NR NTN network (FFS whether this is an additional capability on top of FG 44-3) 2. RAN2 assumption is that how the network handles the access to NR NTN cells for R18 UEs that do not support the new Rel-18 NR NTN “network verified UE location” capability is up to NW implementation, with no need for specs impact (RAN2 can still introduce needed changes to RAN2 specs for this, if requested by other groups)

Meanwhile, RAN1#114 also made the following progress:

Agreement The legacy R17 definition of UE Rx-Tx time difference is adopted for NTN with an offset that is determined based on the following: - UE reports the actual index difference between subframe j and subframe i - The uplink subframe j is closest in time to the DL subframe #i received from the TP - The DL timing drift due to Doppler over the service link associated with the UE RX-TX time difference measurement period is reported. Agreement Confirm the working assumption with the additional note below: In NTN, gNB receive-transmit time difference calculated at uplink time synchronization reference point is reported to the LMF. Note: This does not imply that the actual gNB receive-transmit time difference measurement is necessarily made at the uplink time synchronization reference point. Conclusion No need to support common TA information report from UE to LMF with consideration that common TA information report from gNB has been agreed supported. Conclusion To resolve the mirror positions ambiguity for multi-RTT positioning, the following methods can be used without RAN1 specification impact from RAN1 perspective: - by gNB or LMF implementation - existing ECID method - UL-AoA Agreement Ephemeris information for UE location verification, including accurate satellite position and velocity at the time of measurement, should be available at LMF.

In this contribution we further discuss some remaining issues in network verified UE location based on previous discussions and agreements.

- Discussion

## UE report for Rx-Tx time difference measurement offset

RAN1 have agreed to adopt the legacy R17 definition of UE Rx-Tx time difference for NTN with an offset that is determined based on the actual index difference and DL timing drift associated to UE Rx-Tx time difference measurement. It is expected that RAN2 shall define the signaling from UE to LMF entity for the corresponding parameters.

Proposal 1: RAN2 to define the signaling support for UE reporting of actual index difference and DL timing drift to LMF for location verification using multi-RTT positioning.

## UE reporting of ephemeris used for measurement

RAN1 have agreed that the accurate satellite position and velocity at the time of measurement should be available at LMF. Considering that some of the positioning measurements (e.g., multi-RTT) are performed at UE, and UE may not use the up-to-date ephemeris broadcast by gNB (UE only requires SIB19 when stored ephemeris expires), it is necessary for the UE to report the accurate satellite position and velocity at the time of measurement performed at UE to help LMF in deriving more accurate positioning results.

Proposal 2: RAN2 to define the signaling support for UE reporting of the accurate satellite position and velocity at the time of positioning measurement performed at UE to LMF for location verification.

## gNB reporting of ephemeris used for measurement

Similarly, some of the positioning measurements (e.g., multi-RTT) are performed at gNB, and it is necessary for the gNB to report the accurate satellite position and velocity at the time of measurement performed at gNB to help LMF in deriving more accurate positioning results. However, this work should be up to RAN3.

Proposal 3: It is up to RAN3 to define the signaling support for gNB reporting of the accurate satellite position and velocity at the time of positioning measurement performed at gNB to LMF for location verification.

## Verification procedure impacted by cell change

In legacy TN, a TRP is assumed to be fixed on earth and always available for use, e.g., antenna unit belonging to a ground gNB. In NTN, the serving satellite providing NR access for the UE becomes the TRP which travels on orbit around the Earth, making its position vary over time with limited available duration (i.e. call change is expected at cell stop serving time t-Service). For example, a LEO satellite moves at a maximum speed of 7.9km/s and its serving duration for a given location could be only several seconds. The duration of performing some positioning operations may beyond the t-Service and thus the positioning will fail, and things could be worse when the positioning operation is performed by a single TRP at different times instead of multiple TRPs at one time.

Therefore, the stop serving time of the corresponding TRP used for UE location verification should be available at LMF. Such information could be reported by UE or gNB within RAN2 or RAN3 scope.

Proposal 4a: RAN2 to define the signaling support for UE reporting of the stop serving time of the TRP (NTN cell) to LMF for location verification.

Proposal 4b: It is up to RAN3 to define the signaling support for gNB reporting of the stop serving time of the TRP (NTN cell) to LMF for location verification.

- Conclusion

In this contribution we discuss some remaining issues in network verified UE location based on previous discussions and agreements. It is proposed that:

Proposal 1: RAN2 to define the signaling support for UE reporting of actual index difference and DL timing drift to LMF for location verification using multi-RTT positioning.

Proposal 2: RAN2 to define the signaling support for UE reporting of the accurate satellite position and velocity at the time of positioning measurement performed at UE to LMF for location verification.

Proposal 3: It is up to RAN3 to define the signaling support for gNB reporting of the accurate satellite position and velocity at the time of positioning measurement performed at gNB to LMF for location verification.

Proposal 4a: RAN2 to define the signaling support for UE reporting of the stop serving time of the TRP (NTN cell) to LMF for location verification.

Proposal 4b: It is up to RAN3 to define the signaling support for gNB reporting of the stop serving time of the TRP (NTN cell) to LMF for location verification.