3GPP TSG RAN WG2 Meeting #123bis                                                 R2-2310033

# Xiamen, China, October 09-13, 2023

Agenda item:	7.4.1

Source:	Intel Corporation

Title:	Discussion on L2/3 UE capabilities for NR further mobility enhancements

Document for:	Discussion and decision

# Introduction

This document discusses the possible UE capability signalling required for feMob WI based on the agreements made so far.  Some of the discussion points are still FFS and will need to take into consideration additional agreements when they are made.

# Discussion

The sections below cover the three different objectives of the feMob WI and the possible capability signalling that could be defined for each with aim to reach agreement on what capability needs to be signalled and the indication from this signalling.  Apart from the UE implementation aspects, the discussion also considers the possible network deployment aspects in terms of opportunity for interoperability testing and whether an IOT/capability bit could be useful for indication of successful IOT.

## LTM:

| Feature/ scenario | Capability signalling | Discussion | Other aspects |
| --- | --- | --- | --- |
| LTM | Optional feature | Any dependency with other feMOB features? | Per UE, no FRx/xDD differentiation |
| Scenarios supported | - MCG LTM - SCG LTM - Inter-DU (different SK-counter handling) | Need for separate IOT bits for each scenario? | Per UE, no FRx/xDD differentiation |
| Reference configuration | Optional | Already agreed | Per UE, no FRx/xDD differentiation |
| RACHless LTM | Optional | Implies UE supports: Dynamic grant, Early RACH | Per UE, no FRx/xDD differentiation |
| RACHless LTM with CG | Optional | Separate bit needed over RACHless LTM? | Per UE, no FRx/xDD differentiation |

## Subsequent CPAC in NR-DC

| Feature/ scenario | Capability signalling | Discussion | Other aspects |
| --- | --- | --- | --- |
| SCPAC | Optional feature | Any dependency with other feMOB features? | Per UE, no FRx/xDD differentiation |
| Scenarios supported | - Intra SN (within a group ID, no change/list of SK-counter) - SN initiated inter-SN based on SN measurement configuration - MN initiated inter-SN based on MN measurement configuration | Is there a need for separate IOT bits for different scenario? If so, how to split it (e.g., MN initiated/SN initiated, intra-SN/Inter-SN)? | Per UE, no FRx/xDD differentiation |
| Reference configuration | Optional | Relationship with Reference configuration for LTM- Single capability bit for reference configuration for both LTM and SCPAC? | Per UE, no FRx/xDD differentiation |

## CHO including target MCG and candidate SCGs for CPC CPA in NR-DC

| Feature/ scenario | Capability signalling | Discussion | Other aspects |
| --- | --- | --- | --- |
| CHO+CPA/CPC | Separate capabilities for CHO+CPC and CHO+CPA? | Features are quite similar. Is there a possibility for different IOT opportunity and hence a need for IOT bits? | Per UE, no FRx/xDD differentiation |
| Max # of condCells | Capability signalling required? | UE capability or hard coded in specs? | Per UE, no FRx/xDD differentiation |

# Summary and proposals

This document provided an initial discussion point for potential UE capability signalling for feMob.  These will need to be further updated based on additional discussions/agreements.  In the meantime, it is proposed:

Proposal: Discuss the above listed features/capabilities to see conclusions can be reached