3GPP TSG-RAN WG2 Meeting #123bis	R2-2310058

Xiamen, China, October 9th – 13th, 2023

Agenda Item:    7.11.2.2

Source:         Xiaomi

Title:         	Discussion on the data loss during the PDCP count synchronization

Document for:	Discussion and Decision

Introduction

In this contribution, we would like to have further discussion about the PDCP operation issue to support the multicast reception in RRC_INACTIVE state, and provide our considerations and the draft TP accordingly.

Discussion

In last meeting, we achieved the following agreement to support the PDCP count synchronization for the multicast reception in RRC_INACTIVE during the inactive mobility. [1]

- For “non-synchronised“ cell (in terms of PDCP COUNT), upon cell reselection, UE sets the initial PDCP count of the MRB for the multicast reception in RRC_INACTIVE state based on the same mechanism as R17 MBS broadcast.

Upon cell reselection, to ensure PDCP count synchronization in the target cell during INACTIVE mobility, the UE needs to re-initialize the PDCP count of the MRB which is indicated as non-synchronized by the upper layer. However, we can note that once the PDCP count is re-initialized, the PDCP SDUs in the reception buffer that have not been delivered to the upper layer will be discarded, which results in the data loss. To avoid such data loss during the PDCP count synchronization, it is better to deliver the PDCP SDUs in the reception buffer to the upper layer before re-initializing the PDCP count.

Observation 1: PDCP count re-initialisation may cause data loss as the PDCP SDUs in the reception buffer will be discarded.

Proposal 1: The PDCP SDUs in the reception buffer needs to be delivered to the upper layer before the PDCP count re-initialisation for the synchronisation.

To specify UE behaviour for the data lossless during the PDCP count synchronisation, one way is to define a new PDCP entity handling procedure in the TS 38.323, which includes the delivery of the stored PDCP SDUs to the upper layer and the PDCP count re-initialisation. As an alternative, since the existing PDCP entity re-establishment procedure includes the PDCP SDU delivery before the PDCP count re-initialisation as quoted below, to reduce the spec impact, UE can perform the PDCP count re-initialisation for the multicast MRB which is indicated as non-synchronized by the upper layer via PDCP entity re-establishment procedure.

- 5.1.2	PDCP entity re-establishment

…

When upper layers request a PDCP entity re-establishment, the receiving PDCP entity shall:

-	process the PDCP Data PDUs that are received from lower layers due to the re-establishment of the lower layers, as specified in the clause 5.2.2.1;

-	for SRBs, discard all stored PDCP SDUs and PDCP PDUs;

-	for SRBs, UM DRBs and UM MRBs, if t-Reordering is running:

-	stop and reset t-Reordering;

-	for UM DRBs and UM MRBs, deliver all stored PDCP SDUs to the upper layers in ascending order of associated COUNT values after performing header decompression;

-	for AM DRBs and AM MRBs for Uu interface, perform header decompression using ROHC for all stored PDCP SDUs if drb-ContinueROHC is not configured in TS 38.331 [3];

…

Observation 2: In the existing PDCP entity re-establishment for the multicast MRB(s), the stored PDCP SDUs are delivered to the upper layer before the PDCP count re-initialisation.

Proposal 2: UE performs the PDCP entity re-establishment for the multicast MRB(s) which is indicated as non-synchronized by upper layer.

Proposal 3: RAN2 agrees on the draft TP in the Annex.

Conclusions

According to the analysis given above, we have the following observations and proposals:

Observation 1: PDCP count re-initialisation may cause data loss as the PDCP SDUs in the reception buffer will be discarded.

Observation 2: In the existing PDCP entity re-establishment for the multicast MRB(s), the stored PDCP SDUs are delivered to the upper layer before the PDCP count re-initialisation.

Proposal 1: The PDCP SDUs in the reception buffer needs to be delivered to the upper layer before the PDCP count re-initialisation for the synchronisation.

Proposal 2: UE performs the PDCP entity re-establishment for the multicast MRB(s) which is indicated as non-synchronized by upper layer.

Proposal 3: RAN2 agrees on the draft TP in the Annex.

References

- R2-23xxxxx MBS session notes (Dawid) 2023-08-25 1030

Annex: Draft TP 38.323

### 5.1.2	PDCP entity re-establishment

When upper layers request a PDCP entity re-establishment, the UE shall additionally perform once the procedures described in this clause for Uu or PC5 interface. After performing the procedures in this clause, the UE shall follow the procedures in clause 5.2.

When upper layers request a PDCP entity re-establishment, the transmitting PDCP entity shall:

-	for UM DRBs and AM DRBs, reset the ROHC protocol for uplink and start with an IR state in U-mode (as defined in RFC 3095 [8] and RFC 4815 [9]) if drb-ContinueROHC is not configured in TS 38.331 [3];

-	for UM DRBs and AM DRBs, reset the EHC protocol for uplink if drb-ContinueEHC-UL is not configured in TS 38.331 [3];

-	for AM DRBs, reset the UDC compression buffer to all zeros and prefill the dictionary if drb-ContinueUDC is not configured in TS 38.331 [3];

-	for SRBs and UM DRBs, set TX_NEXT to the initial value;

-	for SRBs, discard all stored PDCP SDUs and PDCP PDUs;

-	apply the ciphering algorithm and key provided by upper layers during the PDCP entity re-establishment procedure;

-	apply the integrity protection algorithm and key provided by upper layers during the PDCP entity re-establishment procedure;

-	for UM DRBs, for each PDCP SDU already associated with a PDCP SN but for which a corresponding PDU has not previously been submitted to lower layers, and;

-	for AM DRBs for Uu interface whose PDCP entities were suspended, from the first PDCP SDU for which the successful delivery of the corresponding PDCP Data PDU has not been confirmed by lower layers, for each PDCP SDU already associated with a PDCP SN:

-	consider the PDCP SDUs as received from upper layer;

-	perform transmission of the PDCP SDUs in ascending order of the COUNT value associated to the PDCP SDU prior to the PDCP re-establishment without restarting the discardTimer, as specified in clause 5.2.1;

-	for AM DRBs whose PDCP entities were not suspended, from the first PDCP SDU for which the successful delivery of the corresponding PDCP Data PDU has not been confirmed by lower layers, perform retransmission or transmission of all the PDCP SDUs already associated with PDCP SNs in ascending order of the COUNT values associated to the PDCP SDU prior to the PDCP entity re-establishment as specified below:

-	perform header compression of the PDCP SDU using ROHC as specified in the clause 5.7.4 and/or using EHC as specified in the clause 5.12.4;

-	If drb-ContinueUDC is configured and if the PDCP SDU has been compressed before:

-	submit the PDCP SDU previously compressed to integrity protection and ciphering function;

-	else:

-	perform uplink data compression of the PDCP SDU as specified in clause 5.14.4, and submit the PDCP SDU to integrity protection and ciphering function;

-	perform integrity protection and ciphering of the PDCP SDU using the COUNT value associated with this PDCP SDU as specified in the clause 5.9 and 5.8;

-	submit the resulting PDCP Data PDU to lower layer, as specified in clause 5.2.1.

When upper layers request a PDCP entity re-establishment, the receiving PDCP entity shall:

-	process the PDCP Data PDUs that are received from lower layers due to the re-establishment of the lower layers, as specified in the clause 5.2.2.1;

-	for SRBs, discard all stored PDCP SDUs and PDCP PDUs;

-	for SRBs, UM DRBs and UM MRBs, if t-Reordering is running:

-	stop and reset t-Reordering;

-	for UM DRBs and UM MRBs, deliver all stored PDCP SDUs to the upper layers in ascending order of associated COUNT values after performing header decompression;

-	for AM DRBs and AM MRBs for Uu interface, perform header decompression using ROHC for all stored PDCP SDUs if drb-ContinueROHC is not configured in TS 38.331 [3];

-	for AM DRBs for PC5 interface, perform header decompression using ROHC for all stored PDCP IP SDUs;

-	for AM DRBs and AM MRBs for Uu interface, perform header decompression using EHC for all stored PDCP SDUs if drb-ContinueEHC-DL is not configured in TS 38.331 [3];

-	for UM DRBs, AM DRBs, UM MRBs and AM MRBs, reset the ROHC protocol for downlink and start with NC state in U-mode (as defined in RFC 3095 [8] and RFC 4815 [9]) if drb-ContinueROHC is not configured in TS 38.331 [3];

-	for UM DRBs, AM DRBs, UM MRBs and AM MRBs, reset the EHC protocol for downlink if drb-ContinueEHC-DL is not configured in TS 38.331 [3];

-	for SRBs and UM DRBs, set RX_NEXT and RX_DELIV to the initial value;

-	for UM MRBs and AM MRBs, set RX_NEXT and RX_DELIV to the initial value if initialRX-DELIV is configured in TS 38.331 [3];

-	for multicast MRBs indicated as non-synchronized by upper layer, set RX_NEXT and RX_DELIV to the initial value.

-	apply the ciphering algorithm and key provided by upper layers during the PDCP entity re-establishment procedure;

-	apply the integrity protection algorithm and key provided by upper layers during the PDCP entity re-establishment procedure.

NOTE 1:	After PDCP re-establishment on a sidelink ‎SRB/DRB, UE determines when to transmit and receive with the new key and discard the old key as specified in TS ‎‎33.536 [14].‎

NOTE 2:	At PDCP re-establishment, the MRB type (i.e. UM MRB or AM MRB) is determined by the target configuration.‎